Software validation specification (SVS)

Version 1.6

Solar Orbiter Mission

RPW INVESTIGATION

MEB (Main Electronic Box) Instrument

LFR (Low Frequency Receiver) Sub-Instrument

| Prepared by: | Function: | Signature: | Date (Dd/mm/yyyy) |
| --- | --- | --- | --- |
| V. BOUZID(LPP) B. KATRA (LPP) | V&V Engineer |  | 04/05/2017 |
| Verified by: | Function: | Signature: | Date |
| Approved by: | Function: | Signature: | Date |
| W.RECART (NEXEYA SERVICES) | Software Quality Engineer |  |  |
| For application: | Function: | Signature: | Date |

| Iss | Rev | Date | Author | Notes |
| --- | --- | --- | --- | --- |
| 0 | 0 | 15-Feb-13 | G.SAULE | First Issue according to the ECSS‐E‐ST‐40C_6March2009 document (Annex L (normative) Software validation specification (SVS) - DRD \ L.2 Expected response \ L.2.1 Scope and content) |
| 0 | 1 | 11-Jun-13 | G.SAULE | Taking into account: - The RPW-SYS-SSS-00013-LES_Issue1_rev2 document. - The relecture_SVS.xlsx sheet. |
| 0 | 2 | 30-Jul-13 | G.SAULE | Minor evolutions |
| 1 | 0 | 15-Nov-13 | G.SAULE | Evolutions for TRR. |
| 1 | 1 | 10-Apr-14 | G.SAULE | Evolutions for V2. |
| 1 | 2 | 25-jun-14 | B.KATRA V.BOUZID | Evolutions for TRR2 |
| 1 | 3 | 15-aug-14 | B.KATRA V.BOUZID | Update/corrections from JIRA issues for CDR/TRB R2 : RPWSWR-433 |
| 1 | 4 | 20-oct-15 | B.KATRA V.BOUZID | paragraphs 5.3, 5.6,8.1.3 and 9 updated RD07 added RD08 added All TestCases updated SVS-0004 SVS-0015 SVS-0016 SVS-0017 SVS-0023 SVS-0026 SVS-0037 SVS-0060 SVS-0061 SVS-0072 deleted SVS-0068 SVS-0082 SVS-0083 SVS-0084 SVS-0086 SVS-0087 SVS-0088 SVS-0089 SVS-0091 SVS-0092 SVS-0093 SVS-0094 SVS-0096 SVS-0097 added |
| 1 | 5 | 16-Feb-16 | B.KATRA V.BOUZID | Minor corrections Shift bug in SVS numbering fixed Updated SW (3.0.0.10 to 3.0.0.22) SVS-0003 updated (TC with length > MAX_LEN) SVS-0005 SVS-0007 SVS-0008 SVS-0011 SVS-0012 SVS-0019 SVS-0020 SVS-0027 SVS-0029 SVS-0031 SVS-0032 SVS-0034 SVS-0055 SVS-0056 SVS-0057 SVS-0058 SVS-0059 SVS-0067 SVS-0068 SVS-0069 SVS-0070 SVS-0073 SVS-0074 SVS-0076 SVS-0080 SVS-0083 SVS-0089 SVS-0095 updated SVS-0088 updated to comply with ICD >=3.1 SVS-0026 SVS-0037 SVS-0060 SVS-0061 added SVS-0006 SVS-0085 SVS-0096 SVS-0097 deleted |
| 1 | 6 | 02-Apr-17 | B.KATRA V.BOUZID | Major updated and tests added for R3++ compliancy Updated SW (3.0.0.22 to 3.2.0.24) and HW (1.1.89 to 1.1.91) Merged requirements Updated ( New TC and improvements) SVS-0003 SVS-0005 SVS-0007 SVS-0008 SVS-0012 SVS-0018 SVS-0019 SVS-0020 SVS-0021 SVS-0022 SVS-0026 SVS-0027 SVS-0028 SVS-0029 SVS-0030 SVS-0031 SVS-0032 SVS-0034 SVS-0035 SVS-0037 SVS-0040 SVS-0041 SVS-0042 SVS-0043 SVS-0044 SVS-0045 SVS-0053 SVS-0054 SVS-0055 SVS-0056 SVS-0057 SVS-0059 SVS-0060 SVS-0062 SVS-0064 SVS-0069 SVS-0077 SVS-0078 SVS-0080 updated SVS-0024 SVS-0025 SVS-0039 SVS-0046 SVS-0047 SVS-0047 SVS-0048 SVS-0049 SVS-0050 SVS-0051 SVS-0052 SVS-0058 SVS-0068 SVS -0075 SVS-0084 SVS-0087 SVS-0092 SVS-0093 SVS-0094 deleted SVS-0096 SVS-1100 SVS-1110 SVS-1102 SVS-1103 SVS-1109 SVS-1201 SVS-1202 SVS-1203 SVS-1204 Added |
| 1 | 7 | 7-nov-18 | VBO | Minor Updated tests due to improve GCOV analyze Updated SW (3.2.0.24 to 3.2.0.24) and HW ( 1.1.91 to 1.1.91) updated SVS-0003 SVS-0007 SVS-0013 SVS-0018 SVS-0019 SVS-0026 SVS-0027 SVS-0034 SVS-0037 SVS-0041 SVS-0042 SVS60043 SVS-0044 SVS-0045 SVS-0053 SVS-0057 SVS-0061 SVS-0065 SVS-0069 SVS-0077 SVS-0079 SVS-0081 SVS-0058 Added SVS-0068 SVS-0080 Deleted |

Table of Contents

1	Introduction	6

2	Applicable and reference documents	6

2.1	Applicable documents	6

2.2	Reference documents	6

3	Terms, definitions and abbreviated terms	7

4	Software overview	8

5	Software validation specification task identification	8

5.1	Task and criteria	8

5.2	Features to be tested	8

5.3	Features not to be tested	8

5.4	Test pass - fail criteria	8

5.5	Items that cannot be validated by test	8

5.6	Manually and automatically generated code	9

6	Software validation testing specification design	9

6.1	General	9

6.2	Organization of each identified test design	9

7	Software validation test case specification	9

7.1	General	9

7.2	Organization of each identified test case	9

8	Software validation test procedures	11

8.1	General	11

8.2	Organization of test procedures	13

Validation test platform requirements	196

9	Software validation specification additional information	203

List of Figures

This section has been left blank intentionally.

Aucune entrée de table d'illustration n'a été trouvée.

List of Table

This section has been left blank intentionally.

Aucune entrée de table d'illustration n'a été trouvée.

# Introduction

This document provides the validation tests procedures to be run according to the validation test strategy describe in AD01.

# Applicable and reference documents

## Applicable documents

|  | Reference | Ver. | Title |
| --- | --- | --- | --- |
|  | RPW-MEB-LFR-SDP-00040-1-1_LFR_Software_Development_Plan | 2.0 | Software Validation and Verification Plan (included in referenced Software Development Plan) |
|  | RPW-MEB-LFR-PLN-00033 |  | SPAP |
|  | RPW-SYS-SSS-00013-LES | Issue4 rev3 | RPW Instrument - Software System Specification |
|  | RPW-MEB-LFR-PLN-00035-LPP-0-0_SValP | 1.1 | SOFTWARE VALIDATION PLAN (SVALP) - Version 0.0 - Solar Orbiter Mission - RPW INVESTIGATION - MEB (Main Electronic Box) Instrument - LFR (Low Frequency Receiver) Sub-Instrument |
|  | SOL.S.ASTR.TN.00079_03 | 3 | Solar Orbiter TM-TC and Packet Structure ICD |
|  | SOL-EST-RCD-0050 | i3 | Solar Orbiter Experiment Interface Document – Part A (EID-A) |
|  | RPW-MEB-LFR-SRS-00020 | 2.1 | Software Requirements Specification (SRS) |
|  | RPW FDIR |  | RPW-SYS-MEB-###-FMC-000207-LES |
|  | ECSS-E-ST-50-53C | C | Space engineering / SpaceWire – CCSDS packet transfer protocol |
|  | Solar Orbiter Operations Requirements Document | I1r6 | SO-ESC-RS-05001 |
|  | RPW-SYS---ICD-00067-LES | Issue4 rev3 | RPW IDB PARAMETER DEFINITION |
|  | RPW-MEB-LFR-SDD-00039 | 1.3 | Software Design Document |

## Reference documents

|  | Reference | Ver. | Title |
| --- | --- | --- | --- |
|  | ECSS-Q-ST-80 | C | Space product assurance Software product assurance |
|  | ECSS-E-ST-40 | C | Space engineering - Software |
|  | ECSS‐E‐ST‐50‐12 | C | Space engineering - SpaceWire – Links, nodes, routers and networks |
|  | RPW-MEB-LFR-DRP-00028 |  | Design report |
|  |  | Version 1.1.0 - B4104, November 2010 | GRLIB IP Core User’s Manual - |
|  | RPW-SYS-MEB-LFR-ICD-00097 | Issue4_Rev3 | RPW_ICD_LFR_TC_TM |
|  | RPW-MEB-LFR-NTT-00124 | 1.1 | SGSE Software User Manual |
|  | RPW-MEB-LFR-NTT-00125 | 1.0 | LFR GSE : system clock simulator |

# Terms, definitions and abbreviated terms

| Abbreviation | Mean |
| --- | --- |
| AD | Applicable document |
| APID | Application Process Identifier |
| ASM | averaged spectral matrices |
| BIAS | Biasing Unit |
| bit | Binary unit |
| CCSDS | Consultative Committee for Space Data Systems |
| CRC | Cyclic Redundancy Code |
| CTR | Central Time Reference |
| CWF | continuous waveforms |
| DAS | DPU Application Software |
| DMS | Data Management System |
| dop | degree of polarization |
| DPS | DPU Flight Software |
| DPU | Data Processing Unit |
| DR | Reference documents |
| DRD | Document Requirements Definition |
| ECSS | European Cooperation on Space Standardization |
| EID-A | Experiment Interface Document part A |
| EM | Engineering Model |
| FDIR | Failure Detection Isolation and Recovery |
| fn | Function |
| FSW | Flight Software |
| HK | House Keeping |
| HMI | Human machine interface |
| HW | Hardware |
| i.e. | Id est |
| ICD | Interface Control Document |
| ID | Identifier |
| IDB | Instrument DataBase |
| LA | logical address |
| LED |  |
| LF | Low Frequency |
| LFR | Low Frequency Receiver |
| LSB | Least Significant Bit |
| max | Maximum |
| MEB | Main Electronics Box |
| ms | millisecond |
| MSB | Most Significant Bit |
| NA | Not Applicable |
| NOK | Not OK |
| NT | Not tested |
| PB | PlayBack |
| PE | spectral power of E field |
| PLN | Plan |
| POK | Partially Ok |
| PUS | Packet Utilization Service |
| RB | requirements baseline |
| RD | Reference Document |
| REQ | requirement |
| Rev | Revision |
| RMAP | remote memory access protocol |
| RPW | Radio and Plasma Wave analyzer |
| s | second |
| SBM | Selected Burst Mode |
| SCM | Search Coil Magnetometer |
| SGICD | Space to Ground Interface Control Document |
| SPAP | Software Product Assurance Plan |
| SpW | SpaceWire |
| SRS | Software Requirement(s) Specification |
| SSS | System Software Specification |
| SVALP | Software Validation Plan |
| SVS | Software Verification Specification |
| SW | Software |
| SWF | snapshot waveform |
| tab |  |
| TBC | To Be Confirmed |
| TC | TeleCommand |
| TDS | Time Domain Sampler |
| THR | TNR-HFR |
| TM | Telemetry |
| Topcased | Toolkit in Open-Source for Criticical Application & Systems Development |
| TS | Time Series |
| Ver | Version |
| Vol | volume |

# Software overview

The purpose of the LFR FSW is to drive the appropriate LFR scientific modes upon TC received from the RPW DPU.

The LFR FSW main functions are the following:

- Communication with the RPW DPU: telecommands interpretation, telemetry products packaging and transmission to the DPU.

- Configuration of the LFR sampling strategy and pre-processing abilities following the reception of telecommands and of appropriate parameter sets coming from the RPW DPU.

- Time keeping.

- Computation of a given set of scientific products: the Basic Parameters.

- Housekeeping data collection and housekeeping reporting to the DPU.

The LFR FSW is made of one only product that implements all of the requirements related to the LFR instrument scientific modes as described in the AD03.

# Software validation specification task identification

## Task and criteria

Each SRS requirement tagged as “TEST” is related to a test case in this document (see §8). Each test case can be divided into several steps(§8.1.3) which represent a sub-case intended to validate a part of the requirement.

## Features to be tested

Each SRS requirement (derived from SSS requirements) that are tagged “TEST” should be covered by a SVS test case e.g. related to one or more tests validate it.

## Features not to be tested

SRS requirement (derived from SSS requirements) that are tagged “DESIGN” are not tested and not described in this document (see §6). Also, some requirements tagged “TEST” that have test case associated could not be tested and are tagged “NA” e.g. “Not Applicable” because they may be related to yet not implemented functions.

## Test pass - fail criteria

A test is considered as “PASSED” or “OK” if : all steps of test case or “OK” e.g. tests results are matching expected results.

A test is considered as “FAILED” or “NOK” e.g. “Not OK” if : all steps of test case are are “NOK” e.g. tests results do not match expected results.

A test is considered as “POK” e.g. “Partially OK” if : 1 or more steps are “NOK” or “POK”.

## Items that cannot be validated by test

SRS requirement (derived from SSS requirements) that are tagged “DESIGN” are not tested and not described in this document. All requirements that cannot be tested by test are at least tested by design. Some test case hardly linked to SpaceWire communication protocol are also noted “NT” e.g. “Not Tested” because of a lack of advanced interfaces and tools to interact with SpaceWire registers and input/output.

## Manually and automatically generated code

We internally developed our own framework (SocExplorer) to interact with LFR and be able to write and execute python scripts. Those script are used to implement test case described in this document (1 script = 1 step of a test case). Around 70% of tests used are ran and automatically analyzed by scripts to determine pass – fail criteria. Also, some tests have been implemented as rules that are always and automatically checked for all tests even if it’s not the purpose of the test (for example: periodicity of HK is always checked).

# Software validation testing specification design

Everything that is relative to test by “DESIGN” is detailed in AD12.

## General

This section has been left blank intentionally.

## Organization of each identified test design

This section has been left blank intentionally.

### General

This section has been left blank intentionally.

### Features to be tested

This section has been left blank intentionally.

### Approach refinements

This section has been left blank intentionally.

# Software validation test case specification

## General

Most of the elements of this chapter are described in §5 and §8 of this document.

## Organization of each identified test case

### Test case identifier

See §8.1.1.

### Inputs specification

This section has been left blank intentionally.

### Outputs specification

This section has been left blank intentionally.

### Test pass - fail criteria

See §5.4.

### Environmental needs

This section has been left blank intentionally.

### Special procedural constraints (ECSS-Q-ST-80 clause 6.3.5.25)

This section has been left blank intentionally.

### Interfaces dependencies

This section has been left blank intentionally.

# Software validation test procedures

## General

The organization of each identified test procedure is done according the template below:

Note some fields use a dedicated style to be treated by TOPCASED/TRAMWAY tool: those styles are: Assessment_Topcased, Date_Topcased, TestOperator_Topcased, TestProcedureIdentifier_Topcased, Step_Topcased, Comments_Topcased, ExpectedResult_Topcased, Status_Topcased (the field <XXX> is in <XXX>_Topcased style).

### Test procedure identifier

It’s an identifier “SVS-<X>_Ed<i>” with the “TestProcedureIdentifier_Topcased” style.

<X> is an integer.

<i> is the release of the SRS req. The goal of this index is to trace the changes of substance.

### Purpose

This section has been left blank intentionally.

### Procedure steps

The steps are cut. They preferentially are status results oriented. So, the steps -by in anticipation of eventual fail- correspond to the verifiable independent characteristics. Example when one action is sufficient to check x items: x steps will get have all the combination of verdicts. The potential faulty steps will isolate parts to fail. Some steps may be tagged not applicable (NA) if the expected result is not relevant compared to other steps. The reason should be explained in the “Comments” column (see for example SVS-0008/Step 1).

### Test script

If possible, tests are automated. This is the case of repetitive or long tests. Python 2.7 is used.

The program sends integer sequences to simulate TCs. It stores the TCs and the integers received (TMs).

At the end of progress (date=xxx=yyyy_mm_dd-hh_mm_ss, for example 2013_10_15-23_46_55):

-Raw data files are generated: for example, xxx-Tc.csv and xxx-Tm.csv.

-All the TCs/TMs are automatically analyzed. All fields of all TCs/TMs are then extracted. This is the file xxx-Detail.txt. Note a synthetic file is too generated: xxx-Synth.txt.

All displays (logs, info, warnings, errors) obtained during the execution of the script are recorded in xxx-Nb.txt.

For some tests, using xxx-Tc.csv, and xxx-Tm.csv:

-if it is more ergonomically to have the relevant fields to reach a verdict: an xxx-Extract.txt is generated.

-if analysis is automated, it generates an xxx-Analyze.txt file.

This section has been left blank intentionally.

## Organization of test procedures

| TEST CASE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  | Date |  | Assessment |  |  |  |
| SVS-0001_Ed3 |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |  |
| TS Tested requirements |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  | REQ-LFR-SRS-5200_Ed1 |  |  |  |  |  |
| Component: |  |  | Version: V1 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |
| HW: 1.1.91 |  | StarDundee |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm | Test Duration: |  |  |  |  |  |
| Purpose |  | Check the LFR FSW processes the command packets transmitted by the DPU. |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |
| Requirement Title |  | Command management. |  |  |  |  |  |  |
| dependencies |  | SSS-CP-FS-020 |  |  |  |  |  |  |
| restrictions |  | -DBS –used only in SAFE mode- does not communicate with LFR. -There’s no Check memory feature for the LFR. -There’s no self tests feature for the LFR (no req): see §5.5.1 (Requirements\Operational requirements\LFR mode management) of SRS. -There’s no diagnostic feature for the LFR. |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |
| test script |  | grspw_registers.py commonIntensiveLoop.py, actionsTcLfr*.py, |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |
| Notes |  | This requirement is a very high level: its complete verification includes many TEST CASES. Socexplorer can be used to check GRPSPW registers. |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |
| Step | Actions |  |  | Expected Results |  |  | Comments | Status |
|  | Spw board is powered on. The FSW is not launched. Read in the board register at 0x80000500+APB address offset: the destination key for the RMAP protocol the Node address. - See in RD05 document, at the paragraph §"GRSPW - SpaceWire codec with AHB host Interface and GRSPW target"/"Registers" Currently, Destination key is 0x10. - The RPWSSS-202 Jira issue of the RPW-SSS project is "RMAP key values". Currently, "The analyzer FPGA and S/W shall be configured with the following RMAP key: LFR RMAP key = 2". - SY_LFR_DPU_LA is defined in IBD. Currently SY_LFR_DPU_LA=0xFE. The grspw_registers() script realize this verification, and produce a log file *Synth.txt |  |  | Verify Node address =SY_LFR_DPU_LA, and Verify Destination key = LFR RMAP key. Verify the test result is success (“VERDICT = PASS”). See snapshots below. |  |  | Use Also Socexplorer |  |
|  | FSW LFR is loaded, and is launched. Launch the /lfrverif/LFR_SVS/SVS-0001 /Step03/actionsTcLfr*.py scripts: send successively each TC with nominal values (PACKET_LENGTH, CRC correct …). For TC_LFR_LOAD_NORMAL_PAR, enter in STANDBY mode, else pass in NORMAL mode. For the free parameters of each TC: If possible, take the default values, otherwise set all allowable ranges for each discrete parameter. Notice: -Sends comply with timing constraints. -The TC-chaining is not representative of use. -The SEQUENCE_CNT, CUC are not affected in a relevant way. |  |  | Verify: - the FSW stays healthy. - the TM_LFR_TC_EXE_SUCCESS transmission. |  |  |  |  |
|  | FSW LFR is loaded, and is launched. Successively, for each TC: repeatedly send it with nominal values (PACKET_LENGTH, CRC correct …). -For TC_LFR_LOAD_NORMAL_PAR, enter in STANDBY mode, else enter in NORMAL mode. -Sends comply with timing constraints. -The TC-chaining is not representative of use. -The SEQUENCE_CNT, CUC are not affected in a relevant way. |  |  | Verify the FSW stays healthy. |  |  |  |  |
|  | End |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0002_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5233_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: 1.1.91 |  |  | StarDundee |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 12 mn |  |  |  |
| Purpose: |  | Check the LFR FSW is able to receive, to process and to execute the command packets without affecting its other running independent processes. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Command management. |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-FS-031, EIDA R-192 |  |  |  |  |  |  |  |  |
| restrictions |  |  | The verification of the complete RPW system is not made at this level |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | Tc_during_matrix_transmission_asm4s.py and tc_during_matrix_transmission.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | For the LFR validation, LFR is isolated; so it’s impossible to ensure that the on-board reception, processing and execution of telecommand do not affect other instruments processes. For the LFR, the sending of TM_LFR_SCIENCE_* is the task independent. During this task, a TC is sent. |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | LFR FSW is in NORMAL mode. It receipts erroneous TC during matrix transmission: Send TC_LFR_DUMP_PAR with erroneous CRC, during TM reception: TM_LFR_SCIENCE_NORMAL_SWF_F0 TM_LFR_SCIENCE_NORMAL_ASM_F0. |  |  |  | Verify: -matrix transmission continues normally until its expected end (not stopped immediately by the action). -Acceptance failure is sent. -HK transmission is not affected by the action. |  |  |  |  |  |  |
|  | LFR FSW is in NORMAL mode. Activate burst mode during matrix transmission (TM_LFR_SCIENCE_NORMAL_SWF_F0). |  |  |  | Verify: -matrix transmission continues normally until its expected end (not stopped immediately by the action). -Acceptance is sent. -HK transmission is not affected by the action. |  |  |  |  |  |  |
|  | LFR FSW is in SBM1 mode. It receipts nominal TC during matrix transmission: Send TC_LFR_DUMP_PAR, during TM_LFR_SCIENCE_SBM1_CWF_F1 reception. |  |  |  | Verify: -matrix transmission continues normally until its expected end (not stopped immediately by the action). -Acceptance success is sent. -HK transmission is not affected by the action. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date | Assessment |  |  |
| SVS-0003_Ed3 |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5234_Ed1 REQ-LFR-SRS-5236_Ed1 REQ-LFR-SRS-5237_Ed1 REQ-LFR-SRS-5206_Ed1 REQ-LFR-SRS-5220_Ed1 REQ-LFR-SRS-5564_Ed1 REQ-LFR-SRS-5407_Ed1 REQ-LFR-SRS-5209_Ed1 REQ-LFR-SRS-5546_Ed1 |  |  |  |
| Component: |  |  |  |  | Version: V2 V1 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |
| HW:1.1.91 |  |  | StarDundee |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 3mn |  |  |  |
| Purpose: |  |  | Check the execution of each TC is verifiable through a resulting change in the value of a TM parameter. The LFR FSW shall compute the CRC on the CCSDS Telecommand Source Packet part of the received packet |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |
| Requirement Title |  | Command management. And DPU / RPW Analyzer SW communication protocol |  |  |  |  |  |  |
| dependencies |  | SSS-CP-FS-032 and SSS-CP-FS-055 and SSS-CP-FS-376 and SSS-CP-FS-075 and SSS-CP-EQS-354 and SSS-IF-DPS-EQ-160 and SSS-IF-DPS-EQ-180 SSS-CP-EQS-170 RPW-SYS-MEB-LFR-ICD-00097, SSS-CP-FS-032, EIDA R-402 |  |  |  |  |  |  |
| restrictions |  | -Verification is only on the “Telecommand Verification Packets”. Functional results (by TM transmission -Housekeeping Reporting, Science, LFR Configuration-) are not tested. -Exhaustive coverage of all cases is infeasible; the steps below are a subjective compromise to perform a suitable verification. |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |
| test script |  | loop_tm_lfr_tc_exe(): Nominal TC+TC with error detected during the acceptance stage verifications (wrong CRC) |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |
| notes |  | For this TEST CASE, in the "ack" field in each telecommand packet: ACK_ACCEPTANCE=”report required” (1). ACK_EXECUTION_COMPLETION=”report required” (1). TC_LFR_RESET is only sent in with error in acceptance stage |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |
| Step | Actions |  |  | Expected Results |  |  | Comments | Status |
|  | Nominal TC_LFR_ENTER_MODE. For mode=0..4; loop all the steps below. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS in case of change LFR mode (TM_LFR_TC_EXE_NOT_EXECUTABLE if LFR is already in the target mode). -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | Nominal TC_LFR_RESET. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS, with consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  | TC_LFR_RESET performs exit() |  |
|  | TC_LFR_RESET with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_COMMON_PAR. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_COMMON_PAR with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_NORMAL_PAR. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS (except in NORMAL, SBM1 and SBM2: NOT_EXECUTABLE). -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_NORMAL_PAR with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_BURST_PAR. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS(except in BURST: NOT_EXECUTABLE). -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_BURST_PAR with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_SBM1_PAR. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS (except in SBM1: NOT_EXECUTABLE). -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_SBM1_PAR with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_SBM2_PAR. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS (except in SBM2: NOT_EXECUTABLE). -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_SBM2_PAR with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_DUMP_PAR. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_DUMP_PAR with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_ENABLE_CALIBRATION. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_ENABLE_CALIBRATION with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_TC_LFR_DISABLE_CALIBRATION. |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_TC_LFR_DISABLE_CALIBRATION with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_UPDATE_TIME. (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW: -increases HK_LFR_DPU_SPW_PKT_RCV_CNT and HK_LFR_UPDATE_TIME_TC_CNT. -starts the execution of the commands: verify the TIME field in the TM_LFR_HK (No Timecode sent) |  |  | REQ-LFR-SRS-5237_Ed1 |  |
|  | TC_LFR_UPDATE_TIME with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW: -sends TM_LFR_HK with HK_LFR_DPU_SPW_PKT_RCV_CNT increased. |  |  | REQ-LFR-SRS-5237_Ed1 |  |
|  | Nominal TC_LFR_UPDATE_INFO. (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW sends TM_LFR_HK with HK_LFR_DPU_SPW_PKT_RCV_CNT and HK_LFR_UPDATE_INFO_TC_CNT increased. |  |  |  |  |
|  | TC_LFR_UPDATE_INFO with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW sends TM_LFR_HK with HK_LFR_DPU_SPW_PKT_RCV_CNT increased. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_KCOEFFICIENTS. |  |  | Verify the LFR FSW. -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_KCOEFFICIENTS with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_DUMP _KCOEFFICIENTS. |  |  | Verify the LFR FSW - sends 2 TM_LFR_KCOEFFICIENTS_DUMP. -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_DUMP_KCOEFFICIENTS with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Nominal TC_LFR_LOAD_FBINS_MASK |  |  | Verify the LFR FSW. -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_FBINS_MASK with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | Send a TC with length > SY_LFR_TC_MAX_LEN (228 bytes) |  |  | Spacewire rejects the TC too long In TM_LFR_HK; only these Two fields are updated: HK_LFR_DPU_SPW_RX_TOO_BIG is incremented HK_LFR_ME_CNT is incremented |  |  |  |  |
|  | Send a TC with length > SY_LFR_TC_MAX_LEN (228 bytes) with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | WARNING: NO TM_LFR_TC_EXE_CORRUPTED because the spacewire rejects the TC too long first. In TM_LFR_HK; only these Two fields are updated: HK_LFR_DPU_SPW_RX_TOO_BIG is incremented HK_LFR_ME_CNT is incremented |  |  |  |  |
|  | Nominal TC_LFR_LOAD_ FILTER_PAR |  |  | Verify the LFR FSW. -sends TM_LFR_TC_EXE_SUCCESS. -transmits consistent PA_RPW_TELECOMMAND_PKT_ID and PA_RPW_PKT_SEQ_CONTROL parameters. |  |  |  |  |
|  | TC_LFR_LOAD_FILTER_PAR with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | For mode=0..4; loop the steps 34-35 Send a TC with length < CCSDS_TC_PKT_MIN_SIZE (12 bytes) (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR_FSW don’t acknowledge this TC In TM_LFR_HK, only HK_LFR_DPU_SPW_PKT_RCV_CNT field is incremented. |  |  |  |  |
|  | Send a TC with length < CCSDS_TC_PKT_MIN_SIZE (12 bytes) with error detected during the acceptance stage verifications (wrong CRC). (Wait 1.1*SY_RPW_ACK_RESPONSE_TIME) |  |  | Verify the LFR_FSW don’t acknowledge this TC In TM_LFR_HK, only HK_LFR_DPU_SPW_PKT_RCV_CNT field is incremented. |  |  |  |  |
|  | For mode=0..4 TC_LFR_ENTER_MODE with error detected during the acceptance stage verifications (wrong CRC). |  |  | Verify the LFR FSW: -sends TM_LFR_TC_EXE_CORRUPTED. -transmits consistent PA_RPW_COMPUTED_CRC parameter. |  |  |  |  |
|  | End |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  | Date | Assessment |  |  |  |
| SVS-0005_Ed2 |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |  |
| TS Tested requirements |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  | REQ-LFR-SRS-5203_Ed2 REQ-LFR-SRS-5206_Ed1 |  |  |  |  |
| Component: |  |  | Version: V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |
| HW:1.1.91 |  | Stardundee |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm | Test Duration: 30sec |  |  |  |  |
| Purpose: |  | Check the LFR FSW validates the received commands prior to their execution (acceptance stage). |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |
| Requirement Title |  | Command management. |  |  |  |  |  |
| dependencies |  | SSS-CP-FS-040, AD06: EIDA R-187/EIDA-2809, AD10:TCV-1 and SSS-CP-FS-075 |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |
| test script |  | acceptation_execution_failures.py |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results | Comments | Status |
|  | For each TC in TC_LFR_LOAD_COMMON_PAR, TC_LFR_LOAD_NORMAL_PAR, TC_LFR_LOAD_BURST_PAR, TC_LFR_LOAD_SBM1_PAR, TC_LFR_LOAD_SBM2_PAR, TC_LFR_DUMP_PAR, TC_LFR_ENABLE_CALIBRATION, TC_LFR_DISABLE_CALIBRATION,TC_LFR_LOAD_KCOEFFICIENTS,TC_LFR_KCOEFFICIENTS_DUMP,TC_LFR_LOAD_FBINS_MASK,TC_LFR_LOAD_FILTER_PAR: -Send that nominal TC; ensure the LFR returns TM_LFR_TC_EXE_SUCCESS -Send that TC with forbidden source ID (and CRC updated). |  |  |  | Check the LFR FSW validates the acceptance stage: TM_LFR_TC_EXE_CORRUPTED. |  |  |
|  | For each <mode> in NORMAL, BURST, SBM1, SBM2: -Enter in that mode. -Send nominal TC_LFR_LOAD_<mode>_LOAD; ensure the LFR returns TM_LFR_TC_EXE_NOT_EXECUTABLE. -Send that TC_LFR_LOAD with forbidden source ID (and CRC updated). |  |  |  | Check the LFR FSW validates the acceptance stage: TM_LFR_TC_EXE_CORRUPTED. |  |  |
|  | For each <mode> in STANDBY, NORMAL, BURST, SBM1, SBM2: -Enter in that mode. Ensure the expected mode is reached. -Send again that nominal TC_LFR_ENTER_MODE; ensure the LFR returns TM_LFR_TC_EXE_NOT_EXECUTABLE. -Send that TC_LFR_ENTER_MODE with forbidden source ID (and CRC updated). |  |  |  | Check the LFR FSW validates the acceptance stage: TM_LFR_TC_EXE_CORRUPTED. |  |  |
|  | End |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  | Assessment |  |  |
| SVS-0007_Ed2 |  |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5205_Ed2 REQ-LFR-SRS-5204_Ed1 REQ-LFR-SRS-5212_Ed1 REQ-LFR-SRS-5206_Ed1 |  |  |  |  |
| Component: |  |  |  |  |  | Version: V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW:1.1.91 |  |  |  |  | Stardundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Duration :3mn40sec |  |  |  |  |
| Purpose: |  | Check if the acceptance of the command fails, the LFR FSW systematically generates a Telecommand Execution Completed Failure Report (TM_LFR_TC_EXE_CORRUPTED or TM_LFR_TC_EXE_NOT_EXECUTABLE) conforming to the PUS telecommand verification service, whatever the value of the acknowledgment flag in the command packet header. This requirement does not apply to the following packets: TC_LFR_UPDATE_INFO, TC_LFR_UPDATE_TIME. Criterias are defined by REQ-LFR-SRS-5204_Ed1 Upon the reception of any command packet, the LFR FSW shall verify if the packet can be accepted by: - Checking the following items in this specific order : - APID (PID, CAT) - Length of the received packet should match packet_length field contained in packet header - service type and service subtype - source ID - Length_packet field value is relevant with expected length considering definition of the sub-type. - packet error control (CRC). The algorithm to compute the packet error control is specified in the [AD10], appendix 6. If at least one of this criteria fails, TM_LFR_EXE_CORRUPTED packet is emitted with failure code 42005 - Checking that the command is allowed in the current mode (see tab 5.2) and/or state of the instrument. No check of the packet sequence counter shall be made |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Command acceptance stage / Content of the verification report |  |  |  |  |  |  |
| dependencies |  |  |  | AD05, SSS-CP-FS-065 SSS-CP-FS-050 SSS-CP-FS-120 SSS-CP-FS-075 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  | Acceptance of the command fails. |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | loop_tm_lfr_tc_exe_no_ack.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | -criterias of acceptance failed are listed in REQ-LFR-SRS-5204 -CRC test : each byte will be tested TC too long or too short don’t reach this acceptance stage. |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  | Comments | Status |
|  | For each combinaison of ack_execution and ack_acceptance send a TC with Failed CRC first byte Failed CRC second byte Invalid PID for LFR (PID=9) Valid LFR PID but not the right one for the TC Invalid LFR CAT (CAT different to 1) Invalid LFR Type (Type=13) Valid LFR Type but not the right one for the TC Invalid LFR SUBTYPE Bad packet-length (65535) Invalid source_id (255) Send a TC_LFR_DISABLE_CALIBRATION with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_DUMP_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_ENABLE_CALIBRATION with error detected during the acceptance stage verifications verifications |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_ENTER_MODE with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_LOAD_BURST_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_LOAD_COMMON_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_LOAD_NORMAL_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_LOAD_SBM1_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_LOAD_SBM2_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | TC_LFR_LOAD_KCOEFFICIENTS with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED |  |  |  |
|  | TC_LFR_ DUMP_KCOEFFICIENTS with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED |  |  |  |
|  | TC_LFR_LOAD_FBINS_MASK with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED |  |  |  |
|  | TC_LFR_LOAD_FILTER_PAR with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED |  |  |  |
|  | TC_LFR_RESET with error detected during the acceptance stage verifications. |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED. |  |  |  |
|  | In STANDBY, send all TC_LFR_xx, excepted TC_LFR_UPDATE_xxx and TC_LFR_RESET. |  |  |  |  |  | Verify if the acceptance of the command fails, the LFR FSW systematically generates a Telecommand Execution Completed Failure Report FSW replies. Only TM_LFR_TC_EXE_NOT_EXCUTABLE will be seen. |  |  |  |
|  | In NORMAL, send all TC_LFR_xx, excepted TC_LFR_UPDATE_xxx and TC_LFR_RESET. |  |  |  |  |  | Verify if the acceptance of the command fails, the LFR FSW systematically generates a Telecommand Execution Completed Failure Report FSW replies. |  |  |  |
|  | In BURST, send all TC_LFR_xx, excepted TC_LFR_UPDATE_xxx and TC_LFR_RESET. |  |  |  |  |  | Verify if the acceptance of the command fails, the LFR FSW systematically generates a Telecommand Execution Completed Failure Report FSW replies. |  |  |  |
|  | In SBM1, send all TC_LFR_xx, excepted TC_LFR_UPDATE_xxx and TC_LFR_RESET. |  |  |  |  |  | Verify if the acceptance of the command fails, the LFR FSW systematically generates a Telecommand Execution Completed Failure Report FSW replies. |  |  |  |
|  | In SBM2, send all TC_LFR_xx, excepted TC_LFR_UPDATE_xxx and TC_LFR_RESET. |  |  |  |  |  | Verify if the acceptance of the command fails, the LFR FSW systematically generates a Telecommand Execution Completed Failure Report FSW replies. |  |  |  |
|  | For each combinaison of ack_execution and ack_acceptance send all TC with a wrong packet-length vs Type/subtype |  |  |  |  |  | Ensure FSW replies TM_LFR_TC_EXE_CORRUPTED except for TC_LFR_UPDATE_xxx. |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  | Assessment |  |
| SVS-0008_Ed2 |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5207_Ed2 REQ-LFR-SRS-5212_Ed1 REQ-LFR-SRS-5206_Ed1 REQ-LFR-SRS-5202_Ed1 REQ-LFR-SRS-5207_Ed2 |  |  |  |
| Component: |  |  |  |  | Version: V2 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |
| HW:1.1.91 |  |  |  | Stardundee |  |  |  |  |
| Start time: |  |  |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |
| End Time: |  |  |  | (DD/MM/YYYY) hh:mm | Test Duration:22sec |  |  |  |
| Purpose: |  | Check, if the execution of the command fails, the LFR FSW generates a Telecommand Execution Completed Failure Report conforming to the PUS telecommand verification service, even if this has not been requested in the execution acknowledgement flag in the telecommand packet header. This requirement does not apply to the following packets: TC_LFR_UPDATE_INFO, TC_LFR_UPDATE_TIME. |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Command execution stage |  |  |  |  |  |
| dependencies |  |  | SSS-CP-FS-080, “Solar Orbiter Operations Requirements Document”-TCV-5 B and SSS-CP-FS-120 and SSS-CP-FS-075 and SSS-CP-FS-100 and SSS-CP-EQS-200 |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |
| prerequisite |  |  | Acceptance of the command does not fail. |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |
| test script |  |  | tc_execution_failure_report.py |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |
| notes |  |  | ACK_EXECUTION_COMPLETION=0 («no report required»). Note case bit=1 is seen in REQ-LFR-SRS-5234 (SVS-0003). TC_LFR_DISABLE_CALIBRATION TC_LFR_ENABLE_CALIBRATION TC_LFR_DUMP_PAR TC_LFR_DUMP_KCOEFFICIENTS TC_LFR_LOAD_FBINS_MASK TC_LFR_LOAD_COMMON_PAR never generated a failure report. |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results | Comments | Status |
|  | TC_LFR_ENTER_MODE with wrong or inconsistent data field. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT with PA_RPW_BYTE_POSITION=11 |  |  |
|  | TC_LFR_ENTER_MODE when it cannot be executed at this time either when enter_mode = current_mode |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_NOT_EXECUTABLE. | REQ-LFR-SRS-5202_Ed1 |  |
|  | TC_LFR_LOAD_BURST_PAR with wrong / inconsistent data field. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT. | See SVS-0065 all parameters are tested |  |
|  | TC_LFR_LOAD_BURST_PAR when it cannot be executed at this time either in BURST mode. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_NOT_EXECUTABLE. |  |  |
|  | TC_LFR_LOAD_NORMAL_PAR with wrong /inconsistent data field. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT. | See SVS-0065 all parameters are tested |  |
|  | TC_LFR_LOAD_NORMAL_PAR when it cannot be executed at this time either in NORMAL, SBM1, SBM2 modes. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_NOT_EXECUTABLE. |  |  |
|  | TC_LFR_LOAD_SBM1_PAR with wrong or inconsistent data field. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT. | See SVS-0065 all parameters are tested |  |
|  | TC_LFR_LOAD_SBM1_PAR when it cannot be executed at this time either in SBM1 mode. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_NOT_EXECUTABLE. |  |  |
|  | TC_LFR_LOAD_SBM2_PAR with wrong or inconsistent data field. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT. | See SVS-0065 all parameters are tested |  |
|  | TC_LFR_LOAD_SBM2_PAR when it cannot be executed at this time. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_NOT_EXECUTABLE. |  |  |
|  | TC_LFR_LOAD_KCOEFFICIENTS with wrong / inconsistent data field. |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT with PA_RPW_BYTE_POSITION=10 Only one inconsistent parameter. |  |  |
|  | TC_LFR_LOAD_FILTER_PAR with wrong/inconsistent data field |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_INCONSISTENT | See SVS-0065 all parameters are tested |  |
|  | End |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  | Assessment |  |  |
| SVS-0009_Ed2 |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5209_Ed2 REQ-LFR-SRS-5206_Ed1 REQ-LFR-SRS-5202_Ed1 REQ-LFR-SRS-5210_Ed1 |  |  |  |  |
| Component: |  |  |  |  | Version: V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: 1.1.91 |  |  |  | Stardundee |  |  |  |  |  |
| Start time: |  |  |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |
| End Time: |  |  |  | (DD/MM/YYYY) hh:mm | Test Duration: |  | 6mn 30sec |  |  |
| Purpose: |  | Check, when a command has been properly executed, the LFR FSW systematically generates a report of successful completion for the execution stage, whatever the value of the acknowledgement flag in the command packet header. This requirement does not apply to the following packets: TC_LFR_UPDATE_TIME and TC_LFR_UPDATE_INFO. |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Command execution stage |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-FS-090, AD10: TCV-4 + TCV-6 and SSS-CP-FS-075 and SSS-CP-FS-100 and SSS-CP-FS-095 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | A command has been properly executed |  |  |  |  |  |  |
| used test programs |  |  | completionExecution.py |  |  |  |  |  |  |
| test script |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |
| notes |  |  | Python script surveys the acknowledges; if a TC with expected ack would have no report after the SY_RPW_ACK_RESPONSE_TIME delay: Timeout will be displayed, and noticed in NB*.txt file. |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  | Comments | Status |
|  | For each combination of ACK_EXECUTION_COMPLETION, and ACK_ACCEPTANCE, reach each mode. For each mode, send all TC (except the TC_LFR_UPDATE_*), |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_SUCCESS when TC are ok (see tab 5.2 in SRS). |  |  |  |
|  | For each combination of ACK_EXECUTION_COMPLETION, and ACK_ACCEPTANCE, make all possible transitions (except when expected mode=current mode ) |  |  |  |  | Verify the LFR FSW sends TM_LFR_TC_EXE_SUCCESS . |  |  |  |
|  | End |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date | Assessment |  |  |  |
| SVS-0010_Ed2 |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5211_Ed1 |  |  |  |  |
| Component: |  |  |  |  |  | Version: V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW:1.1.91 |  |  |  | Stardundee |  |  |  |  |  |  |
| Start time: |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |
| End Time: |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration:50s |  |  |  |  |
| Purpose: |  |  | Check the TC acceptance/execution success report contains: -copy of the TC Packet ID field. -copy of the TC Packet Sequence Control field. |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  | Content of the verification reports |  |  |  |  |  |  |  |  |
| dependencies |  | SSS-CP-FS-110, AD10: TCV-7 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  | successful_content_verification_reports.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  | No feedback for TC_LFR_UPDATE_* |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  | Comments | Status |
|  | For each mode, send all TC. |  |  |  | Verify the TM for TC acceptance/execution success report contains: -a copy of the TC Packet ID field. -a copy of the TC Packet Sequence Control field. |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0011_Ed2 |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5218_Ed1 |  |  |  |
| Component: |  |  |  |  |  | Version: V2 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW:1.1.91 |  |  | StarDundee |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration:38s |  |  |  |
| Purpose: |  | Check after initialization, the LFR FSW starts its local time set the most significant bit of its local time to 1 and all other bits to 0. |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Time management |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-FS-360,AD06 R-214; EIDA-2931 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | The CTR SpaceWire time codes are properly transmitted (SY_RPW_CTR_FREQUENCY) TC_LFR_RESET can’t be used because it performs exit(0) |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |
| test script |  |  | starting_time.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  | Comments | Status |
|  | Restart the FSW by an HW reset. Launch the Python script. No automatic timecode sent |  |  |  | TIME field (CUC format): verify the LFR FSW sets the most significant bit of its local time to 1. LFR status shown in TM_LFR_HK is: TIME=0x8000000248ba, HK_LFR_MODE: STANDBY = 0, HK_LFR_DPU_SPW_ENABLED: ENABLED = 1, HK_LFR_DPU_SPW_LINK_STATE: RUN = 5HK_LFR_SC_POTENTIEL_FLAG: ON = 1 SY_LFR_WATCHDOG_ENABLED: ENABLED = 1, HK_LFR_CALIB_ENABLED: DISABLED = 0, HK_LFR_RESET_CAUSE: POWER_ON = 1, One error HK_LFR_LE_CNT=1 HK_LFR_LAST_ER_RID: LE_LFR_TIMEC = 42129, HK_LFR_LAST_ER_CODE: MISSING = 21, HK_LFR_LAST_ER_TIME=0x800000022f12 HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=1, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 |  |  |  |  |
|  | Synchronize LFR Send a TC_LFR_UPDATE_TIME with CP_RPW_TIME = 1 and a valid timecode (1) wait 5s without timecode |  |  |  | TIME field = 0x000000004c7c on the TM_LFR_HK received status LFR in TM_LFR_HK shows the synchronization HK_LFR_DPU_SPW_TICK_OUT_CNT=1, HK_LFR_DPU_SPW_LAST_TIMC=1 No timing errors detected After TM_LFR_HK shows the timecode absence in the suitable error counter only one time. HK_LFR_TIMECODE_MISSING=2. and HK_LFR_LE_CNT=2 |  |  |  |  |
|  | Reload LFR software and wait the boot sequence. |  |  |  | verify the LFR FSW sets the most significant bit of its local time to 1 and all bits to 0 Time is 0x8000000231ba . Boot sequence around 2sec All suitable fields in TM_HK_LFR are set to 0. HK_LFR_UPDATE_TIME_TC_CNT=0, HK_LFR_LE_CNT=0, HK_LFR_ME_CNT=0, HK_LFR_LAST_ER_RID: NO_ERROR = 0, HK_LFR_LAST_ER_CODE: NO_ERROR = 0, HK_LFR_LAST_ER_TIME=0x000000000000, HK_LFR_DPU_SPW_TICK_OUT_CNT=0, HK_LFR_DPU_SPW_LAST_TIMC=0, HK_LFR_DPU_SPW_PARITY=0, HK_LFR_DPU_SPW_DISCONNECT=0, HK_LFR_DPU_SPW_ESCAPE=0, HK_LFR_DPU_SPW_CREDIT=0, HK_LFR_DPU_SPW_WRITE_SYNC=0, HK_LFR_DPU_SPW_RX_AHB=0, HK_LFR_DPU_SPW_TX_AHB=0, HK_LFR_DPU_SPW_EARLY_EOP=0, HK_LFR_DPU_SPW_INVALID_ADDR=0, HK_LFR_DPU_SPW_EEP=0, , HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=0, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 This operation is not equivalent to HW reset. |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  |  |  | Date |  | Assessment |  |  |
| SVS-0012_Ed2 |  |  |  |  |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  | REQ-LFR-SRS-5219_Ed1 |  |  |  |  |
| Component: |  |  |  |  |  |  |  |  | Version: V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HW:1.1.91 |  |  |  | StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm |  |  |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm |  |  |  |  | Test Duration: | 25mn |  |  |  |  |  |
| Purpose: |  |  | Check the LFR FSW synchronizes its local time with the CTR distributed as a SpW command packet coupled to a SpW time code: - The CTR SpaceWire time code is transmitted at SY_RPW_CTR_FREQUENCY. - The CTR SpaceWire command packet containing the CTR is not necessarily sent. - When it is generated, the CTR SpaceWire command packet is transmitted > SY_RPW_CTR_MIN_DELAY prior to the time code itself. - The CTR SpaceWire command packet is distributed thanks to the “Accept Time Update” command. - The SpaceWire time code contains the least significant bits of the CTR coarse time part. |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  |  |  | Time management |  |  |  |  |  |  |  |
| dependencies |  |  |  |  |  | §3.2.64 Time-Code of DR03. SSS-CP-FS-370, AD06OBTM-1 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  |  |  | ccsds_time_code_format.py + ccsds_time_code_format_xxx.py (burst,normal,sbm1,sbm2) each mode performs step from 1 to 5 send_timecode_not_nominal.py step 6 |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  | AD03: 1/SY_RPW_CTR_FREQUENCY=1s, SY_RPW_CTR_MIN_DELAY=300ms |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  |  |  |  | Expected Results |  | Comments | Status |
|  | Restart the FSW by an HW reset and waitt the end of boot sequence. No automatic timecode is sent. LFR mode is standby and no synchronized. For each mode, send the nominal sequence At t0: TC_LFR_UPDATE_TIME with any CP_RPW_TIME. At t0+SY_RPW_CTR_MIN_DELAY: valid time code A timecode is valid: - For spacewire, timecode = previous + 1 - For CTR timecode = 6 bits of the least coarse time In this case, the spacewire constraint is mandatory |  |  |  |  |  |  |  |  | Verify the LFR FSW synchronizes its local time with the CTR. TM are consistent with the CP_RPW_TIME MSB=0 TM_LFR_HK: Field HK_LFR_DPU_SPW_TICK_OUT incremented of 1 HK_LFR_DPU_SPW_LAST_TMEC = time code |  |  |  |
|  | For each mode, send the time-code at SY_RPW_CTR_FREQUENCY during 1mn: |  |  |  |  |  |  |  |  | LFR FSW local time is incremented. TM_LFR_HK: Field HK_LFR_DPU_SPW_TICK_OUT incremented of 1 HK_LFR_DPU_SPW_LAST_TMEC = time code HK_LFR_TIME_TIMECODE_CTR incremented by 1 and traced into the last error fields. last error detected is: HK_LFR_LAST_ER_RID: LE_LFR_TIME = 42119, HK_LFR_LAST_ER_CODE: TIMECODE_CTR = 26, HK_LFR_LAST_ER_TIME error counter HK_LFR_LE_CNT is incremented. |  |  |  |
|  | For each mode, no time code of SY_RPW_CTR_FREQUENCY during 1mn. |  |  |  |  |  |  |  |  | .during the one minute delay: LFR stays synchronize MSB = 0 TM_LFR_HK: Field HK_LFR_DPU_SPW_TICK_OUT not updated HK_LFR_DPU_SPW_LAST_TMEC not updated HK_LFR_TIME_TIMECODE_CTR not updated HK_LFR_TIMECODE_MISSING is incremented and traced into the last error fields. last error detected: HK_LFR_LAST_ER_RID: LE_LFR_TIMEC = 42129, HK_LFR_LAST_ER_CODE: MISSING = 21, HK_LFR_LAST_ER_TIME error counter HK_LFR_LE_CNT is incremented After 1 mn LFR lost the synchronization. MSB = 1 LFR is desynchronized for the first time HK_LFR_TIME_NOT_SYNCHRO=1 and traced into the last error fields HK_LFR_LAST_ER_RID: LE_LFR_TIME = 42119, HK_LFR_LAST_ER_CODE: NOT_SYNCHRO = 25, HK_LFR_LAST_ER_TIME HK_LE_CNT is incremented. |  |  |  |
|  | For each mode, send the time-code at SY_RPW_CTR_FREQUENCY during 1mn |  |  |  |  |  |  |  |  | LFR increments its local time but stays desynchronized MSB = 1 TM_LFR_HK: Field HK_LFR_DPU_SPW_TICK_OUT incremented of 1 HK_LFR_DPU_SPW_LAST_TMEC = time code HK_LFR_TIME_TIMECODE_IT is incremented. 2 possible errors: HK_LFR_TIME_TIMECODE_CTR incremented by 1 and traced into the last error fields. HK_LFR_LAST_ER_RID: LE_LFR_TIME = 42119, HK_LFR_LAST_ER_CODE: TIMECODE_CTR = 26, HK_LFR_LAST_ER_TIME=0x1xxxx OR HK_LFR_TIME_TIMECODE_IT is incremented by 1 and traced into the last error fields: HK_LFR_LAST_ER_RID: LE_LFR_TIME = 42119, HK_LFR_LAST_ER_CODE: TIMECODE_IT = 24 HK_LE_CNT is incremented. After 1mn, LFR always is desynchronized LFR TIME has MSB = 1 LFR needs a CTR couple to be synchronized |  |  |  |
|  | For each mode: At t0: TC_LFR_UPDATE_TIME with any CP_RPW_TIME. At t0+SY_RPW_CTR_MIN_DELAY: Send time code at SY_RPW_CTR_FREQUENCY |  |  |  |  |  |  |  |  | Verify the LFR FSW synchronizes its local time with the CTR. MSB=0 TM_LFR_HK: No more error detected Field HK_LFR_DPU_SPW_TICK_OUT incremented of 1 HK_LFR_DPU_SPW_LAST_TMEC = time code HK_LFR_TIME_TIMECODE_CTR not updated |  |  |  |
|  | This step uses another the script send_timecode_not_nominal.py For each mode send a not nominal sequence (timecode is sent at t0+2s) At t0: TC_LFR_UPDATE_TIME with any CP_RPW_TIME. At t0+2s: valid time code Wait 3s (no timecode is sent) For each mode send a not nominal sequence (invalid timecode) At t0: TC_LFR_UPDATE_TIME with any CP_RPW_TIME. At t0+0.3s invalid time code |  |  |  |  |  |  |  |  | Verify the LFR FSW synchronizes its local time with the CTR. TM are consistent with the CP_RPW_TIME. HK_LFR_LFR_TIMECODE_MISSING is incremented of only 1. HK_LFR_LE_CNT is incremented of 1 HK_LFR_LAST_ER_CODE: MISSING = 21 Verify the LFR FSW doesn’t synchronize its local time with the CTR. TM_LFR_HK: Field HK_LFR_DPU_SPW_TICK_OUT and HK_LFR_DPU_SPW_LAST_TMEC are not updated (=5) HK_LFR_LE_CNT not updated No error counters updated (HK_LFR_TIMECODE_MISSING=6) and the others =0 |  | LFR is tolerant with SY_RPW_CTR_MIN_DELAY delay --------------- |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  | Assessment |  |  |  |
| SVS-0013_Ed2 |  |  |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5220_Ed2 |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration:20mn |  |  |  |  |  |
| Purpose: |  |  |  | Check , upon reception of the CTR SpW time code: -If a CTR command packet coupled has been received prior to the time code: LFR updates the coarse time part of the local time with the coarse time value previously transmitted in the CTR command packet. -If a CTR command packet has not been received prior to the time code: LFR doesn’t ignores the time code. -LFR sets the fine time part of its local time to 0. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  |  |  | Time management |  |  |  |  |  |  |
| dependencies |  |  |  |  |  | SSS-CP-FS-380 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  |  |  | 4 scripts nominal_tc_lfr_update_time_Step0i.py (i=1,2,3,4,5,6) Reset before each step |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  | SY_RPW_CTR_FREQUENCY=1s, SY_RPW_CTR_MIN_DELAY=300ms |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  |  |  | Expected Results |  | Comments |  | Status |
|  |  | Run the LFR FSW. For each mode: At t0, send TC_LFR_UPDATE_TIME At t1=t0+1s, send an other TC_LFR_UPDATE_TIME. At t2=t1+SY_RPW_CTR_MIN_DELAY, send a time code. |  |  |  |  |  | Verify the LFR FSW, upon reception of the time code: -updates the coarse time part of the local time with the coarse time value previously transmitted in the t2 emitted TC_LFR_UPDATE_TIME. -sets the fine time part of its local time to 0. |  | Only the last TC_LFR_UPDATE_TIME is used. |  |  |
|  |  | Run the LFR FSW. For each mode, for several delays: At t0, send TC_LFR_UPDATE_TIME At t1=t0+delay, send a time code. |  |  |  |  |  | -At each time code, the FSW LFR sets the fine time part of its local time to 0. Reception time code > SY_RPW_CTR_FREQUENCY Coarse time incremented of 1. |  | LFR is tolerant with SY_RPW_CTR_MIN_DELAY delay |  |  |
|  |  | Run the LFR FSW. For each mode, for several periods T: At ti, send TC_LFR_UPDATE_TIME. At t0=ti+ SY_RPW_CTR_MIN_DELAY, send a time code. At tn=t0+ n*T, send a time code during 20s (n is a non null integer). |  |  |  |  |  | Verify the LFR FSW, at each reception of time code: -At t0 only, LFR updates the coarse time part of the local time with the coarse time value previously transmitted in the TC_LFR_UPDATE_TIME. -LFR sets the fine time part of its local time to 0. |  |  |  |  |
|  |  | Run the LFR FSW. For each mode, for several periods T: At t0=n*T send TC_LFR_UPDATE_TIME At t1=t0+ SY_RPW_CTR_MIN_DELAY, send a time code. At tn=t0+ n*T, send a time code during 1mn (n is an non null integer). |  |  |  |  |  | Verify the LFR FSW, at each reception of time code: -LFR updates the coarse time part of the local time with the coarse time value previously transmitted in the TC_LFR_UPDATE_TIME. -LFR sets the fine time part of its local time to 0. |  |  |  |  |
|  |  | Run the LFR FSW. |  |  |  |  |  |  |  |  |  |  |
|  |  | Run the LFR FSW. automatic timecode is launched For each mode send four TC_UPDATE_TIME |  |  |  |  |  |  |  |  |  |  |
|  |  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  |  | Assessment |  |  |
| SVS-0014_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5238_Ed1 |  |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: V2 |  |  |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration:1mn14s |  |  |  |  |  |  |  |
| Purpose: |  |  | Check, on successful time synchronization, the LFR FSW sets the most significant bit of its local time to 0. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Time management |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-405, AD06 R-788 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | time_msb.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  |  | For each mode Send nominal TC_LFR_UPDATE_TIME Packet (CP_RPW_TIME with MSB=0 + send time code |  |  |  | Verify the LFR FSW sets the most significant bit of its local time to 0. |  |  |  |  |  |  |
|  |  | Send TC_LFR_UPDATE_TIME Packet with CP_RPW_TIME= 0x876543210000 + send time code Wait at least 3s. |  |  |  | Verify the LFR FSW sets the most significant bit of its local time to 0. |  |  |  |  |  |  |
|  |  | Send TC_LFR_UPDATE_TIME Packet with CP_RPW_TIME=0x7FFFFFFF0000 + send time code Wait at least 3s. |  |  |  | Verify the LFR FSW sets the most significant bit of its local time to 0. |  |  |  |  |  |  |
|  |  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0018_Ed1 |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5239_Ed2 |  |  |  |  |  |
| Component: |  |  |  |  | Version: | V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 15Mn + 6h15 |  |  |
| Purpose: |  |  | Check the LFR FSW sets the Destination ID field by complying with the rules when building a TM packet. |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Telemetry management |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-580 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | source_id_loop_step1.py ( step1 to step 3) and source_id_loop_step2_xxx.py (step 4 to 7) one script by mode (normal,burst,sbm1,sbm2) ASM=4s |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | SOURCE_ID values defined are the enum of RD06 No ack for TC_LFR_UPDATE_INFO/TC_LFR_UPDATE_TIME |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  |  | Expected Results |  | Comments | Status |
|  |  | LFR FSW builds TM packet out exception: TM packet having the packet category <> 2 (HK essential), <> 3 (Table), <> 4 (HK routine), <> 8 (diagnostic) and <> 9 (dump). Send TC_LFR_LOAD_COMMON_PAR, TC_LFR_LOAD_NORMAL_PAR, TC_LFR_LOAD_BURST_PAR, TC_LFR_LOAD_SBM1_PAR, TC_LFR_LOAD_SBM2_PAR, TC_LFR_ENABLE_CALIBRATION, TC_LFR_DISABLE_CALIBRATION, TC_LFR_LOAD_KCOEFFICIENTS, TC_LFR_FBINS_MASK, TC_LFR_LOAD_FILTER_PAR, TC_LFR_ENTER_MODE (with mode transition). For each TC, send all the SOURCE_ID values allowed (0, …, 255). |  |  |  |  | Verify for telemetry generated as an answer to a command, the Destination ID is: -the copy of the command Source ID field for defined cases (using RD06) -0 for undefined Id. |  |  |  |
|  |  | LFR FSW builds a TM packet having the packet category = 4. Wait a TM_LFR_HK. |  |  |  |  | Verify the Destination ID always is set to zero. |  |  |  |
|  |  | LFR FSW builds a TM packet having the packet category = 6 Send TC_LFR_DUMP_PAR, TC_LFR_DUMP_KCOEFF, For each TC, send all the SOURCE_ID values allowed (0, …, 255). |  |  |  |  | Verify for telemetry generated as an answer to a command, the Destination ID is: -the copy of the command Source ID field for defined cases (using RD06) -0 for undefined Id. |  |  |  |
|  |  | For each TC, send all the SOURCE_ID values defined+ one undefined value: -Enter in NORMAL mode. Wait the all TM_LFR_SCIENCE_NORMAL_*. ( CWF_F3 and CWF_LONG_F3) |  |  |  |  | Verify the Destination ID always is set to zero. |  |  |  |
|  |  | For each TC, send all the SOURCE_ID values defined+ one undefined value: -Enter in BURST mode. Wait all TM_LFR_SCIENCE_BURST_*. |  |  |  |  | Verify the Destination ID always is set to zero. |  |  |  |
|  |  | For each TC, send all the SOURCE_ID values defined+ one undefined value: -Enter in SBM1 mode. Wait all TM_LFR_SCIENCE_SBM1_*. ( NORMAL_CWF_F3 and NORMAL_CWF_LONG_F3) |  |  |  |  | Verify the Destination ID always is set to zero. |  |  |  |
|  |  | For each TC, send all the SOURCE_ID values defined+ one undefined value: -Enter in SBM2 mode. Wait all TM_LFR_SCIENCE_SBM2_*. ( NORMAL_CWF_F3 and NORMAL_CWF_LONG_F3) |  |  |  |  | Verify the Destination ID always is set to zero. |  |  |  |
|  |  | End |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0019_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5240_Ed2 REQ-LFR-SRS-5241_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 9h |  |  |  |  |  |  |
| Purpose: |  |  | Check the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented when a packet is released. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Telemetry management |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-590 and SSS-CP-FS-201 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | tm_sequence_counter_loop.py (step 1 -8) tm_sequence_counter_hk.py (step 9) tm_sequence_counter_dump_loop.py (step 10) tm_sequence_science_loop.py (step 11) tm_sequence_counter_loop_acceptance_failed.py (step12) |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  | Expected Results |  |  |  | Comments | Status |
|  |  | For the TM_LFR_TC_EXE_*: For each SOURCE_ID defined, send (to receive a TM_LFR_TC_EXE_SUCCESS): TC_LFR_LOAD_COMMON_PAR, TC_LFR_LOAD_NORMAL_PAR, TC_LFR_LOAD_BURST_PAR, TC_LFR_LOAD_SBM1_PAR, TC_LFR_LOAD_SBM2_PAR, TC_LFR_ENTER_MODE, TC_LFR_RESET, TC_LFR_ENABLE_CALIBRATION, TC_LFR_DISABLE_CALIBRATION, TC_LFR_LOAD_KCOEFFICIENTS, TC_LFR_DUMP_KCOEFICIENTS, TC_LFR_FBINS_MASK, TC_LFR_DUMP_PAR, TC_LFR_LOAD_FILTER_PAR Alter each TC to cause -if possible- the answer TM_LFR_TC_EXE_SUCCESS, TM_LFR_TC_EXE_INCONSISTENT, TM_LFR_TC_EXE_NOT_EXECUTABLE, TM_LFR_TC_EXE_CORRUPTED. Ensure the verification packets: PID=76, CAT=1. For TC_LFR_DUMP_KCOEFICIENTS and TC_LFR_DUMP_PAR we wait TM_LFR_PARAMETER_DUMP. TM_LFR_KCOEFFICIENTS_DUMP Ensure the verification packets: PID=76, CAT=6. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0 |  |  |  |  |  |
|  |  | For each SOURCE_ID defined, send: TC_LFR_RESET case CORRUPTED, TC_LFR_LOAD_COMMON_PAR case CORRUPTED, TC_LFR_LOAD_NORMAL_PAR cases CORRUPTED, NOT_EXECUTABLE, TC_LFR_LOAD_BURST_PAR cases CORRUPTED, NOT_EXECUTABLE, TC_LFR_LOAD_SBM1_PAR cases CORRUPTED, NOT_EXECUTABLE, TC_LFR_LOAD_SBM2_PAR cases NOT_EXECUTABLE, TC_LFR_ENTER_MODE cases CORRUPTED, INCONSISTENT and NOT_EXECUTABLE, TC_LFR_ENABLE_CALIBRATION case CORRUPTED, TC_LFR_DISABLE_CALIBRATION case CORRUPTED. TC_LFR_LOAD_KCOEFFICIENTS case CORRUPTED TC_LFR_DUMP_KCOEFICIENTS case CORRUPTED TC_LFR_FBINS_MASK case CORRUPTED TC_LFR_DUMP_PAR case CORRUPTED TC_LFR_LOAD_FILTER_PAR case CORRUPTED Ensure the verification packets: PID=76, CAT=1. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0. |  |  |  |  |  |
|  |  | In TM_LFR_HK, ensure the verification packets: PID=76, CAT=4. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0. |  |  |  |  |  |
|  |  | SWF_P =22s Enter in NORMAL mode. Wait TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2 (note durations are in TC_LFR_LOAD_NORMAL_PAR). Ensure the verification packets: PID=76, CAT=12. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0. |  |  |  |  |  |
|  |  | Enter in BURST mode. Wait TM_LFR_SCIENCE_BURST_CWF_F2, TM_LFR_SCIENCE_BURST_BP1_F0, TM_LFR_SCIENCE_BURST_BP2_F0, TM_LFR_SCIENCE_BURST_BP1_F1, TM_LFR_SCIENCE_BURST_BP2_F1 (note durations are in TC_LFR_LOAD_BURST_PAR). Ensure the verification packets: PID=76, CAT=12. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0. |  |  |  |  |  |
|  |  | Enter in SBM1 mode. Wait TM_LFR_SCIENCE_SBM1_CWF_F1, TM_LFR_SCIENCE_SBM1_BP1_F0, TM_LFR_SCIENCE_SBM1_BP2_F0 (note durations are in TC_LFR_LOAD_SBM1_PAR). Ensure the verification packets: PID=79, CAT=12. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0. |  |  |  |  |  |
|  |  | Enter in SBM2 mode. Wait TM_LFR_SCIENCE_SBM2_CWF_F2, TM_LFR_SCIENCE_SBM2_BP1_F0, TM_LFR_SCIENCE_SBM2_BP2_F0, TM_LFR_SCIENCE_SBM2_BP1_F1, TM_LFR_SCIENCE_SBM2_BP2_F1 (note durations are in TC_LFR_LOAD_SBM2_PAR). Ensure the verification packets: PID=79, CAT=12. |  |  |  | Verify the LFR FSW maintains, for each couple of APID and Destination ID, a TM sequence counter incremented by 1 when a packet is released and the FIRST SEQUENCE_CNT=0. |  |  |  |  |  |
|  |  | Packet 0xcfc (SBM1 and SBM2) valid the SEQUENCE_CNTcounter reaches (2^14)-1. |  |  |  | Verify the LFR stays healthy, and the counter wraps around from 2^14-1 to zero. |  |  |  | REQ-LFR-SRS-5241_Ed1 |  |
|  |  | Packet 0xcc4 (HK) valid the SEQUENCE_CNTcounter reaches (2^14)-1. |  |  |  | Verify the LFR stays healthy, and the counter wraps around from 2^14-1 to zero. |  |  |  | REQ-LFR-SRS-5241_Ed1 |  |
|  |  | Packet 0xcc1 (TM_LFR_TC_EXE_SUCCESS) and Packet 0xcc6 (TM_LFR_PARAMETER_DUMP TM_LFR_KCOEFFICIENTS_DUMP) valid the SEQUENCE_CNTcounter reaches (2^14)-1. |  |  |  | Verify the LFR stays healthy, and the counter wraps around from 2^14-1 to zero. |  |  |  | REQ-LFR-SRS-5241_Ed1 |  |
|  |  | Packet 0xccc valid the SEQUENCE_CNTcounter reaches (2^14)-1. |  |  |  | Verify the LFR stays healthy, and the counter wraps around from 2^14-1 to zero. |  |  |  | REQ-LFR-SRS-5241_Ed1 |  |
|  |  | Packet 0xcc1 ((TM_LFR_TC_EXE _CORRUPTED NOT_EXECUTABLE) valid the SEQUENCE_CNTcounter reaches (2^14)-1. |  |  |  | Verify the LFR stays healthy, and the counter wraps around from 2^14-1 to zero. |  |  |  | REQ-LFR-SRS-5241_Ed1 |  |
|  |  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0020_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5300_Ed2 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  |  | 34mn + 3610s |  |  |  |
| Purpose: |  |  | Check the LFR FSW maintains a local time, with: - a resolution of at least SY_RPW_TIME_RESOLUTION. -a relative accuracy of SY_RPW_TIME_ACCURACY. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Time management |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-340 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  | This verification could be thorough: -Rigorous verification is possible only with calibrated external equipments. -Functional verification is possible with all instruments (validation at system level). -A long delay avoids distortion measurement (the CPU load variation are not visible), but PC resynchronizations via the network, and delays due to treatments by the brick are possible. |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | time_resolution.py + set_time_wrap.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | The detail “resolution of at least SY_RPW_TIME_RESOLUTION)” of this constraint is respected by design with the use of CUC format. The PC clock is the reference for this test (a log delay avoid to fail the mesure). Numerical values are in AD03 (currently:SY_RPW_TIME_RESOLUTION = 1 ms, SY_RPW_TIME_ACCURACY = 500 us). The verif_internal_time_consistence tool (cf verif_fields.py) feature surveys the consistence between the pc time, and the lfr time. |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  |  |  | Comments | Status |
|  | Change some NORMAL parameters NORMAL MODE SWF=22s, ASM=4s, Wait at least 500s in each mode without TC_LFR_UPDATE_TIME and without time code reception by LFR. |  |  |  |  |  | Using TIME field of TM_LFR_* packets, verify the LFR FSW maintains a local time with a resolution relative accuracy of SY_RPW_TIME_ACCURACY (compare the delays: from the PC clock, and from the internal time. Using the accumulated deviation, calculate the average shift per second). |  |  |  |  |  |
|  | Launch LFR FSW Reach NORMAL mode Send a TC_LFR_UPDATE_TIME with CP_RPW_TIME=0x7FFFFE01 + valid time code Wait 3600s |  |  |  |  |  | the TIME field wraps around 0xFFFFFFFF to 0x80000000 MSB= 1 because we lost the synchronization (no timecode sent during the test) See on TM_LFR_HK, all TM_LFR_SCIENCE_NORMAL products are consistent. |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  |  | Assessment |  |  |  |  |
| SVS-0021_Ed2 |  |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5302_Ed1 |  |  |  |  |  |  |  |  |
| Component: |  |  |  |  |  |  |  |  |  | Version: V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HW:1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |  |  |
| End Time: |  |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration:3mn10 |  |  |  |  |  |  |  |
| Purpose: |  | Check the LFR FSW generates the TC execution report not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Response time |  |  |  |  |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-FS-131, AD10: 5-TCV-8 |  |  |  |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | SVS-0009 |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | tm_delay.py + just_tc_lfr_reset.py |  |  |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | No ack for TC_LFR_UPDATE_INFO/TC_LFR_UPDATE_TIME NB*.txt synthesizes whenever the timeout is not reached. |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  |  | Comments |  |  | Status |
|  | Use just_tc_lfr_reset.py Generate nominal TC_LFR_RESET. |  |  |  |  | Exit of LFR FSW. NO TM_LFR_TC_EXE* after time-out (4s) |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_COMMON_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_NORMAL_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_BURST_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_SBM1_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_SBM2_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_DUMP_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_KCOEFFICIENTS |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_DUMP_KCOEFFICIENTS |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_FBINS_MASK |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_LOAD_FILTER_PAR |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_ENTER_MODE. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_ENABLE_CALIBRATION. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate nominal TC_LFR_DISABLE_CALIBRATION. |  |  |  |  | Verify the LFR FSW generates one TC execution report (success) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_RESET. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_COMMON_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_NORMAL_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_BURST_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_SBM1_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_SBM2_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_DUMP_PAR. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_KCOEFFICIENTS |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_DUMP_KCOEFFICIENTS |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_FBINS_MASK |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_LOAD_FILTER_PAR |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_ENTER_MODE. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_ENABLE_CALIBRATION. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Generate failed TC_LFR_DISABLE_CALIBRATION. |  |  |  |  | Verify the LFR FSW generates one TC execution report (failure) not longer than SY_RPW_ACK_RESPONSE_TIME from the completion of the TC execution. |  |  |  |  |  |  |  |  |  |
|  | Ensure each ack has been provided; so provoke the TM_LFR_TC_EXE_* not seen at the previous step. |  |  |  |  | Verify all the TM_LFR_TC_EXE_* are seen for this TEST CASE. |  |  |  |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  | Assessment |  |  |  |
| SVS-0022_Ed2 |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5303_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration:52mn |  |  |  |  |  |  |
| Purpose: |  |  | Check the maximum rate of command packets generated to the LFR is SY_LFR_TC_MAX_RATE. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Command packets (DPU SW to RPW Analyzer SW) |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-IF-DPS-EQ-190 |  |  |  |  |  |  |  |
| restrictions |  |  |  | Verification is made only once. It is supposed to be sufficient. We suppose that there is no interaction between the all equipments on the network. |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | tc_maximum_rate.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | AD11: SY_LFR_TC_MAX_RATE = 5 commands per second. |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  | Comments | Status |
| - 1 | LFR is in SBM1. Send 1000 TC_LFR_LOAD_COMMON_PAR at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 2 | LFR is in BURST. Send 1000 TC_LFR_LOAD_NORMAL_PAR at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 3 | LFR is in SBM1. Send 1000 TC_LFR_LOAD_BURST_PAR at the SY_LFR_TC_MAX_RATE rate . |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 4 | LFR is in BURST. Send 1000 TC_LFR_LOAD_SBM1_PAR at the SY_LFR_TC_MAX_RATE rate . |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 5 | LFR is in SBM1. Send 1000 TC_LFR_LOAD_SBM2_PAR at the SY_LFR_TC_MAX_RATE rate . |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 6 | LFR is in SBM1. Send 1000 TC_LFR_DUMP_PAR at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 7 | Send 1000 TC_LFR_ENTER_MODE at the SY_LFR_TC_MAX_RATE rate. Change the CP_LFR_ENTER_MODE_TIME field for to avoid rejected TCs. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 8 | LFR is in SBM1. Send 1000 TC_LFR_UPDATE_INFO at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 9 | LFR is in SBM1. Send 1000 TC_LFR_ENABLE_CALIBRATION at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 10 | LFR is in SBM1. Send 1000 TC_LFR_DISABLE_CALIBRATION at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 11 | Send 1000 TC_LFR_UPDATE_TIME at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 12 | Send 1000 TC_LFR_LOAD_KCOEFFICIENTS at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 13 | Send 1000 TC_LFR_DUMP_KCOEFFICIENTS at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 14 | Send 1000 TC_LFR_LOAD_FBINS_MASK at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 15 | Send 1000 TC_LFR_LOAD_FILTER_PAR at the SY_LFR_TC_MAX_RATE rate. |  |  |  |  | Verify the LFR FSW is healthy. |  |  |  |  |  |
| - 16 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0026_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5241_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 12h30 |  |  |  |
| Purpose: |  |  | All the counters (error counters, packet counters,etc) managed by LFR FSW shall restart at 0 when they reached their maximum value |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | HK counter management |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-201 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | tm_sequence_counter_loop.py and tm_sequence_counter_xx.py (hk,dump,science) SVS-0019 |  |  |  |  |  |  |  |
| output data |  |  |  | update_time_and_info_cnt_wrap.py (SVS-0078) check_tc_exe_and_reject_cnt_wrap.py update_timecode_missing_cnt_wrap.py update_time_not_synchro_cnt_wrap.py update_timecode_it_and_ctr_cnt_wrap.py update_timecode_erroneous_cnt_wrap.py |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
| - 1 | .Check SEQUENCE_CNT for each couple (APID,Destination ID) |  |  |  | Max of SEQUENCE_CNT is 16383 either 2^14 -1 |  |  |  |  | See SVS-0019 (step 9 to 12) |  |
| - 2 | Check HK_LFR_UPDATE_INFO_TC_CNT |  |  |  | Max of HK_LFR_UPDATE_INFO_TC_CNT is 65535 either 2^16 -1 |  |  |  |  | See SVS-0078 (step3) |  |
| - 3 | Check HK_LFR_UPDATE_TIME_TC_CNT |  |  |  | Max of HK_LFR_UPDATE_TIME_TC_CNT is 65535 either 2^16 -1 |  |  |  |  | See SVS-0078 (step3) |  |
| - 4 | Check HK_LFR_EXE_TC_CNT and HK_LFR_REJ_TC_CNT |  |  |  | Max of HK_LFR_EXE _TC_CNT and HK_LFR_REJ_TC_CNT is 65535 either 2^16 -1 |  |  |  |  | check_tc_exe_and_reject_cnt_wrap.py around 7h30 |  |
| - 5 | Check HK_LFR_LE_CNT |  |  |  | Max of HK_LFR_LE_CNT and HK_LFR_ME_CNT is 65535 either 2^16 -1 |  |  |  |  | SVS-0078 (step3) update_time_and_info_cnt_wrap.py |  |
| - 6 | Check HK_LFR_ME_CNT |  |  |  | Max of HK_LFR_LE_CNT and HK_LFR_ME_CNT is 65535 either 2^16 -1 |  |  |  |  | Stress Spacewire errors are too difficult to produce |  |
| - 7 | Check HK_LFR_DPU_SPW_PKT_RCV_CNT and HK_LFR_DPU_SPW_PKT_SENT_CNT |  |  |  | Max of HK_LFR_DPU_SPW_PKT_RCV_ CNT and HK_LFR_DPU_SPW_PKT_SENT_CNT is 65535 either 2^16 -1 |  |  |  |  | HK_LFR_DPU_SPW_PKT_RCV_CNT SVS-0078 (step3) update_time_and_info_cnt_wrap.py HK_LFR_DPU_SPW_PKT_SENT_CNT test de charge SVS-0900 |  |
| - 8 | Check HK_LFR_DPU_TICK_OUT_CNT |  |  |  | Max of HK_LFR_DPU_TICK_OUT_CNT is 255 either 2^8 -1 |  |  |  |  | update_timecode_missing_cnt_wrap.py |  |
| - 9 | Check 8 spacewire errors HK_LFR_DPU_SPW_PARITY HK_LFR_DPU_SPW_DISCONNECT HK_LFR_DPU_SPW_ESCAPE HK_LFR_DPU_SPW_CREDIT HK_LFR_DPU_SPW_WRITE_SYNC HK_LFR_DPU_SPW_EARLY_EOP HK_LFR_DPU_SPW_INVALID_ADDR HK_LFR_DPU_SPW_EEP Check 1 spacewire error HK_LFR_DPU_SPW_RX_TOO_BIG |  |  |  | Max of each counter is 255 either 2^8 -1 |  |  |  |  | spw_failure.py (SVS-0064) Too difficult to produce a lot of these 8 spacewire specific errors. check_too_big_cnt_wrap.py |  |
| - 10 | Check 4 timing errors HK_LFR_TIMECODE_MISSING HK_LFR_TIMECODE_IT HK_LFR_TIME_NOT_SYNCHRO HK_LFR_TIMECODE_CTR HK_LFR_TIMECODE_ERRONEOUS HK_LFR_TIMECODE_INVALID |  |  |  | Max of each counter is 255 either 2^8 -1 |  |  |  |  | update_timecode_missing_cnt_wrap.py and update_time_not_synchro_cnt_wrap.py update_timecode_it_and_ctr_cnt_wrap.py update_timecode_erroneous_cnt_wrap.py Too difficult to produce a lot of.this error (SVS-0064) someone. |  |
| - 11 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0027_Ed2 |  |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements REQ-LFR-SRS-5413_Ed1 REQ-LFR-SRS-5411_Ed2 REQ-LFR-SRS-5412_Ed2 |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 30mn + 10mn+10mn |  |  |  |
| Purpose: |  |  | Check According to [AD09], the SpW protocol header of the packets exchanged is made up of 4 bytes: Target Logical Address. Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Concerning Target Logical Address: Check all the SpW packets produced toward LFR have the Destination Logical Address field set to SY_LFR_DPU_LA. Check all the SpW packets produced by LFR have the Destination Logical Address field set to SY_DPU_LFR_LA. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | DPU / RPW Analyzer SW communication protocol |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SpaceWire – CCSDS packet transfer protocol ECSS-E-ST-50-53C, SSS-IF-DPS-EQ-175 and SSS-IF-DPS-EQ-071 and SSS-IF-DPS-EQ-071 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  | Verification is made only once. It is supposed to be sufficient. |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | ccsds_header_fields_asm_4s.py (step1 to 4, 9 to 11) ccsds_header_fields_step1.py for SBM1 (5,6) ccsds_header_fields_step2.py for SBM2 (7,8) |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | This rule is checked by the Python treatment, for each TC (resp.TM) received (resp. sent) by the LFR. ASM=4s and other parameters are default values All LFR TC are implemented. We can check the generated file TM.csv with excel. We can check the generated file TM.csv with excel. Stress tests use all default parameters (ASM=3600s) . |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  |  | In STANDBY, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP TM_LFR_KCOEFFICIENTS_DUMP received, verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In NORMAL, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Wait 600s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2 received, . Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In NORMAL, with SY_LFR_N_CWF_LONG_F3 = 1 send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Wait 600s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2, TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2 received Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In BURST, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Wait 50s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_BURST_BP1_F0, TM_LFR_SCIENCE_BURST_BP1_F1, TM_LFR_SCIENCE_BURST_BP2_F0, TM_LFR_SCIENCE_BURST_BP2_F1, TM_LFR_SCIENCE_BURST_CWF_F2 received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In SBM1, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. ASM = 4s Wait 600s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_SBM1_BP1_F0, TM_LFR_SCIENCE_SBM1_BP2_F0, TM_LFR_SCIENCE_SBM1_CWF_F1 received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In SBM1, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. ASM = 4s SY_LFR_N_CWF_LONG_F3=1 Wait 600s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2, TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_SBM1_BP1_F0, TM_LFR_SCIENCE_SBM1_BP2_F0, TM_LFR_SCIENCE_SBM1_CWF_F1 received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In SBM2, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. ASM=4s Wait 600s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_SBM2_BP1_F0, TM_LFR_SCIENCE_SBM2_BP1_F1, TM_LFR_SCIENCE_SBM2_BP2_F0, TM_LFR_SCIENCE_SBM2_BP2_F1, TM_LFR_SCIENCE_SBM2_CWF_F2 received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | In SBM2, send each TC. Ensure Destination Logical Address field is set to SY_LFR_ DPU LA, Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. ASM = 4s SY_LFR_N_CWF_LONG_F3=1 Wait 600s |  |  |  | For each TM_LFR_HK, TM_LFR_PARAMETER_DUMP, TM_LFR_KCOEFFICIENTS_DUMP, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2, TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_SBM2_BP1_F0, TM_LFR_SCIENCE_SBM2_BP1_F1, TM_LFR_SCIENCE_SBM2_BP2_F0, TM_LFR_SCIENCE_SBM2_BP2_F1, TM_LFR_SCIENCE_SBM2_CWF_F2 received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | Ensure TM_LFR_TC_EXE_NOT_EXECUTABLE, TM_LFR_TC_EXE_SUCCESS have been sent in previous steps. |  |  |  | For M_LFR_TC_EXE_NOT_EXECUTABLE, TM_LFR_TC_EXE_NOT_IMPLEMENTED, TM_LFR_TC_EXE_SUCCESS received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | Provoke a TM_LFR_TC_EXE_INCONSISTENT: send a TC_LFR_ENTER_MODE with CP_LFR_MODE field = 0xF. |  |  |  | For TM_LFR_TC_EXE_INCONSISTENT received Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | Provoke a TM_LFR_TC_EXE_CORRUPTED: send a TC_LFR_ENTER_MODE with corrupted CRC. |  |  |  | For TM_LFR_TC_EXE_CORRUPTED received, Verify: Destination Logical Address field set to SY_DPU_LFR_LA (0xfe or 0x01) Protocol ID = 0x02 meaning CCSDS. Reserved byte = 0x00. User Application byte = 0x00. Ensure all the defined TCs are sent toward LFR with the destination logical address is SY_LFR_DPU_LA (0xfe) For each TM, verify the destination logical address is SY_DPU_LFR_LA (0x01) |  |  |  |  |  |  |
|  |  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0028_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5502_Ed1 REQ-LFR-SRS-5500_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 25s |  |  |  |
| Purpose: |  |  | Check, in the STANDBY mode, the LFR FSW accepts commands to configure the HW and the SW. The LFR shall handle the following modes STANDBY NORMAL BURST SBM1 SBM2 |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Equipment mode management |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-250 – SSS-CP-EQS-230 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | LFR is in STANDBY mode when RPW is in SERVICE mode; see § “LFR mode management” (§ 5.5.1). |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | configure_standby.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  | Comments | Status |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_COMMON_PAR command. |  |  |  |  | Verify the LFR FSW accepts TC_LFR_LOAD_COMMON_PAR command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_NORMAL_PAR command. |  |  |  |  | Verify the LFR FSW accepts TC_LFR_LOAD_NORMAL_PAR command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_BURST_PAR command. |  |  |  |  | Verify the LFR FSW accepts TC_LFR_LOAD_BURST_PAR command. C |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_SBM1_PAR command. |  |  |  |  | Verify the LFR FSW accepts TC_LFR_LOAD_SBM1_PAR command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_SBM2_PAR command. |  |  |  |  | Verify the LFR FSW accepts TC_LFR_LOAD_SBM2_PAR command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_UPDATE_TIME command. |  |  |  |  | Verify the LFR FSW accepts TC_LFR_UPDATE_TIME command. No acknowledge |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_ENABLE_CALIBRATION command. |  |  |  |  | Verify the LFR FSW accepts the command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_DISABLE_CALIBRATION command. |  |  |  |  | Verify the LFR FSW accepts the command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_UPDATE_INFO command. |  |  |  |  | Verify the LFR FSW accepts the command. No ackknowledge |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_KCOEFFICIENTS command. |  |  |  |  | Verify the LFR FSW accepts the command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_DUMP_KCOEFFICIENTS command. |  |  |  |  | Verify the LFR FSW accepts the command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_FBINS_MASK command. |  |  |  |  | Verify the LFR FSW accepts the command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | Ensure LFR FSW is in STANDBY mode. Send a TC_LFR_LOAD_FILTER_PAR command. |  |  |  |  | Verify the LFR FSW accepts the command. TM_LFR_EXE_SUCCESS |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |  |  |
| SVS-0029_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5503_Ed1 REQ-LFR-SRS-5548_Ed1 REQ-LFR-SRS-5500_Ed1 REQ-LFR-SRS-5514_Ed1 |  |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V1 |  |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 1h10 + 15h |  |  |  |  |  |
| Purpose: |  |  | Check, in the NORMAL mode, the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. The LFR FSW shall handle commands for configuring the different science sub-mode parameter sets. - NORMAL mode parameters: TC_LFR_LOAD_NORMAL_PAR The LFR shall handle the following modes STANDBY NORMAL BURST SBM1 SBM2 |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Equipment mode management and Equipment configuration management |  |  |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-260, REQ-RPW-SYS-0303 SSS-CP-EQS-190 SSS-CP-EQS-230 SSS-CP-EQS-370 |  |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | LFR is in NORMAL mode when RPW is in SURVEY_NORMAL (_C) mode; see § “LFR mode management” (§ 5.5.1). |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | normal_mode_parameter_set.py (step1 to step9) and normal_mode_parameter_set_nominal.py (ASM=3600s) + two long test with timegen with timegen |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | This TESTCASE verifies: -the case of nominal values -the cases with TC_LFR_LOAD_NORMAL_PAR sent any mode (except NORMAL). -Steps must be ensure rule is applied for CWF_F3 an CWF_LONG_F3 SY_LFR_N_SWF_L = 2048 (not to be changed) |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  |  |  | Expected Results | Comments |  | Status |
|  | Use normal_mode_parameter_set.py Launch the FSW LFR. Only ASM=4s for this test Send TC_LFR_LOAD_NORMAL_PAR, in STANDBY mode with SY_LFR_N_ASM_P=4 , send a TC_LFR_DUMP_PAR to valid the parameter send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 600s (1.2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1). |  |  |  |  |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set (default values). TM_LFR_PARAMETER_DUMP shows : SY_LFR_N_SWF_L=2048, SY_LFR_N_SWF_P=300(s), SY_LFR_N_ASM_P=4(s), SY_LFR_N_BP_P0=4(s), SY_LFR_N_BP_P1=20(s), SPARE=0x0, SY_LFR_N_CWF_LONG_F3=0 |  |  |  |
|  | In STANDBY mode, send TC_LFR_LOAD_NORMAL_PAR with: SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P= (default value) /2 SY_LFR_N_ASM_P= (default value) /2 SY_LFR_N_BP_P0= (default value) /2 SY_LFR_N_BP_P1= (default value) /2 SY_LFR_N_CWF_LONG_F3 = default value send a TC_LFR_DUMP_PAR to valid the parameter Send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 600s 2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1). |  |  |  |  |  |  |  | Verify the LFR FSW Rejects this TC With TM_LFR_TC_EXE_INCONSISTENT with PA_RPW_BYTE=16 = SY_LFR_N_BP_P0 PA_RPW_RCV_VALUE=2 TM_LFR_PARAMETER_DUMP shows : SY_LFR_N_SWF_L=2048, SY_LFR_N_SWF_P=300(s), SY_LFR_N_ASM_P=4(s), SY_LFR_N_BP_P0=4(s), SY_LFR_N_BP_P1=20(s), SY_LFR_N_CWF_LONG_F3=0 |  |  |  |
|  | In STANDBY mode, send TC_LFR_LOAD_NORMAL_PAR to test maximum rate with: SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P= 22 SY_LFR_N_ASM_P= 4 SY_LFR_N_BP_P0= default value SY_LFR_N_BP_P1= default value SY_LFR_N_CWF_LONG_F3 = 1 send a TC_LFR_DUMP_PAR to valid the parameter Send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1). |  |  |  |  |  |  |  | TM_LFR_PARAMETER_DUMP shows : SY_LFR_N_SWF_L=2048, SY_LFR_N_SWF_P=22(s), SY_LFR_N_ASM_P=4(s), SY_LFR_N_BP_P0=4(s), SY_LFR_N_BP_P1=20(s), SY_LFR_N_CWF_LONG_F3=1 Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. |  |  |  |
|  | In NORMAL mode, send TC_LFR_LOAD_NORMAL_PAR with default values: Send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1) of previous values ​​accepted. |  |  |  |  |  |  |  | Reject with TM_LFR_TC_EXE_NOT_EXECUTABLE. Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set following SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P= 22 SY_LFR_N_ASM_P= 300 SY_LFR_N_BP_P0= default value SY_LFR_N_BP_P1= default value SY_LFR_N_CWF_LONG_F3 = 0 |  |  |  |
|  | In SBM1 mode, send TC_LFR_LOAD_NORMAL_PAR with: default value Send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1) of previous values ​​accepted. |  |  |  |  |  |  |  | Reject with TM_LFR_TC_EXE_NOT_EXECUTABLE. Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set following SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P= 22 SY_LFR_N_ASM_P= 300 SY_LFR_N_BP_P0= default value SY_LFR_N_BP_P1= default value SY_LFR_N_CWF_LONG_F3 = 1 |  |  |  |
|  | In SBM2 mode, send TC_LFR_LOAD_NORMAL_PAR with default value: Send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1) of previous values ​​accepted. |  |  |  |  |  |  |  | Reject with TM_LFR_TC_EXE_NOT_EXECUTABLE. Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set following SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P= 22 SY_LFR_N_ASM_P= 300 SY_LFR_N_BP_P0= default value SY_LFR_N_BP_P1= default value SY_LFR_N_CWF_LONG_F3 = 1 |  |  |  |
|  | In STANDBY mode, send TC_LFR_LOAD_NORMAL_PAR with: SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P=0xffff (s) SY_LFR_N_ASM_P=0xffff (s) SY_LFR_N_BP_P0=0xff (s) SY_LFR_N_BP_P1=0xff (s). SY_LFR_N_CWF_LONG_F3 = 1 Then entering in NORMAL mode with maximum values |  |  |  |  |  |  |  | Verify the LFR FSW accept the TC_LFR_LOAD_NORMAL_PAR. We obtained TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi i=0,1,2 For SWF et ASM products, the time is too long to wait. | Validate the permitted ranges (currently, not the period stream). |  |  |
|  | In STANDBY mode, send TC_LFR_LOAD_NORMAL_PAR with: SY_LFR_N_SWF_L=2048 SY_LFR_N_SWF_P= 22(s) SY_LFR_N_ASM_P=4 (s) SY_LFR_N_BP_P0=4 (s) SY_LFR_N_BP_P1=20 (s). SY_LFR_N_CWF_LONG_F3 = 0 Then entering in NORMAL mode with minimum values |  |  |  |  |  |  |  | Verify the LFR FSW accept the TC_LFR_LOAD_NORMAL_PAR. Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. |  |  |  |
|  | Extra case: In STANDBY mode, send TC_LFR_LOAD_NORMAL_PAR with default value:. SY_LFR_N_SWF_L=default value=2048 SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=4 (s) SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = default value Then entering in NORMAL mode. |  |  |  |  |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. For ASM=3600 we don’t wait |  |  |  |
|  | A long test is done during 15 hours in NORMAL MODE with defaults values. SY_LFR_N_SWF_L=default value SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=defaultvalue SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = default value We also use the timegen software (send CTR in nominal behavior). |  |  |  |  |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. |  |  |  |
|  | A long test is done during 14 hours in NORMAL MODE with defaults values. SY_LFR_N_SWF_L=default value SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=default value SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  |  |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. |  |  |  |
|  | Use normal_mode_parameter_set_nominal.py Launch the FSW LFR. Without TC_LFR_LOAD_NORMAL_PAR, in STANDBY mode , send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 3960s (1.2*max(SY_LFR_N_SWF_P, SY_LFR_N_ASM_P, SY_LFR_N_BP_P0, SY_LFR_N_BP_P1). Go to standby mode Send TC_LFR_LOAD_NORMAL_PAR with SY_LFR_N_CWF_LONG_F3=1 send TC_LFR_ENTER_MODE with CP_LFR_MODE= NORMAL. With HK, ensure the LFR FSW is in the NORMAL mode. Wait at least 3960s |  |  |  |  |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the NORMAL mode parameter set. |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |
| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |  |  |  |
| SVS-0030_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |  |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5504_Ed1 REQ-LFR-SRS-5548_Ed1 REQ-LFR-SRS-5500_Ed1 REQ-LFR-SRS-5514_Ed1 |  |  |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 V1 |  |  |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 4mn |  |  |  |  |  |  |
| Purpose: |  |  | Check, in the BURST mode, the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. The LFR FSW shall handle commands for configuring the different science sub-mode parameter sets. - BURST mode parameters: TC_LFR_LOAD_BURST_PAR The LFR shall handle the following modes STANDBY NORMAL BURST SBM1 SBM2 |  |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Equipment mode management and Equipment configuration management |  |  |  |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-270 – REQ-RPW-SYS-0305 SSS-CP-EQS-190 – SSS-CP-EQS-230 SSS-CP-EQS-370 |  |  |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | LFR is in BURST mode when RPW is in SURVEY_BURST (_C) mode; see § “LFR mode management” (§ 5.5.1). |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | burst_mode_parameter_set.py + long test with timegen |  |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  |  | Comments | Status |  |  |
|  | Launch the FSW LFR In standby mode no TC LOAD_BURST_PAR sent Enter in Burst mode with default values SY_LFR_B_BP_P0=1s SY_LFR_B_BP_P1=5s Wait 31.5sec |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 Use TC_LFR_DUMP_PAR to check parameters configuration |  |  |  |  |  |  |  |  |  |
| - 2 | In standby mode send TC_LFR_LOAD_BURST_PAR with these values SY_LFR_B_BP_P0=4s SY_LFR_B_BP_P1=6s Enter in Burst Mode Wait 31.5sec |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. TM_LFR_EXE_INCONSISTENT on SY_LFR_B_BP_P0 parameter (PA_RPW_BYTE_POSITION=10 and PA_RPW_RCV_VALUE=4) Default values are always set. TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 3 | In standby mode send TC_LFR_BURST_PAR with maximum solicitation = default values SY_LFR_B_BP_P0=1s SY_LFR_B_BP_P1=5s Enter in Burst Mode Wait 31.5s |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. Default values are set TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 4 | LFR is in the BURST mode. Send a TC_LFR_BURST_PAR Wait 31.5s |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. TM_LFR_EXE_NOT_EXECUTABLE Values are set by step 3 = Default values TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 5 | In standby mode send TC_LFR_BURST_PAR with SY_LFR_B_BP_P0=0s SY_LFR_B_BP_P0=5s Enter in Burst Mode Wait 31.5s |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. TM_LFR_EXE_INCONSISTENT on SY_LFR_B_BP_P0 parameter PA_RPW_BYTE_POSITION=10 and PA_RPW_RCV_VALUE=0. Values are set by step 3 = Default values TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 6 | In standby mode send TC_LFR_BURST_PAR with SY_LFR_B_BP_P0=1s SY_LFR_B_BP_P0=0s Enter in Burst Mode Wait 31.5s |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. TM_LFR_EXE_INCONSISTENT on SY_LFR_B_BP_P1 parameter (PA_RPW_BYTE_POSITION=11 and PA_RPW_RCV_VALUE=0 ) Values are set by step 3 = Default values TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 7 | In standby mode send TC_LFR_BURST_PAR with SY_LFR_B_BP_P0=1s SY_LFR_B_BP_P0=15s Enter in Burst Mode Wait 31.5s |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. Values are set TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 8 | In standby mode send TC_LFR_BURST_PAR with default values SY_LFR_B_BP_P0=1s SY_LFR_B_BP_P0=5s Enter in Burst Mode Wait 31.5s |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. Values are set TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 9 | A long test is done during 14 hours in BURST MODE with defaults values. SY_LFR_B_BP_P0=1s SY_LFR_B_BP_P0=5s We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the LFR FSW sends one single data stream whose content corresponds to the BURST mode parameter set. Values are set TM_LFR_SCIENCE_BURST_BP1_F0 and F1 TM_LFR_SCIENCE_BURST_BP2_F0 and F1 TM_LFR_SCIENCE_BURST_CWF_F2 |  |  |  |  |  |  |  |  |  |
| - 10 | End |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0031_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5505_Ed1 REQ-LFR-SRS-5548_Ed1 REQ-LFR-SRS-5500_Ed1 REQ-LFR-SRS-5514_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 1h 05 |  |  |  |
| Purpose: |  |  | Check, in the SBM1 mode, the LFR FSW sends properly data streams. The LFR FSW shall handle commands for configuring the different science sub-mode parameter sets. - SBM1 mode parameters: TC_LFR_LOAD_SBM1_PAR The LFR shall handle the following modes STANDBY NORMAL BURST SBM1 SBM2 |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management and Equipment configuration management |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-280, REQ-RPW-SYS-0303 REQ-RPW-SYS-0305 SSS-CP-EQS-190 SSS-CP-EQS-230 SSS-CP-EQS-370 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  | LFR is in SBM1 mode when RPW is in SBM_DETECTION (_C) mode, and when RPW is in SBM1_DUMP (_C); see § “LFR mode management” (§ 5.5.1). |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | sbm1_mode_parameter_set_nominal.py (ASM=3600s) + sbm1_mode_parameter_set.py + long test with timegen |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Use sbm1_mode_parameter_set_nominal.py Launch the FSW LFR In standby mode No TC_LFR_LOAD_NORMAL_PAR (default values) noTC_LFR_LOAD_SBM1_PAR sent Enter in SBM1 mode with default values SY_LFR_S1_BP_P0=0.25s SY_LFR_S1_BP_P1=1s Wait 3960 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. For normal mode, we use default values. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
|  | Use sbm1_mode_parameter_set.py Launch the FSW LFR In standby mode TC_LFR_LOAD_NORMAL_PAR with default values except for SY_LFR_N_ASM_P=4s aTC_LFR_LOAD_SBM1_PAR sent SY_LFR_S1_BP_P0=1s SY_LFR_S1_BP_P1=4s Enter in SBM1 mode Wait 420 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. For normal mode, we use default values. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 2 | In standby mode TC_LFR_LOAD_SBM1_PAR sends with default values SY_LFR_S1_BP_P0=0.25s SY_LFR_S1_BP_P1=1s And LOAD_NORMAL_MODE 2048, 22s, 4s,4,20s CWF_LONG_F3 =0 Enter in SBM1 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 3 | In standby mode TC LOAD_SBM1_PAR sends with default values SY_LFR_S1_BP_P0=0.25s SY_LFR_S1_BP_P1=1s And LOAD_NORMAL_MODE 2048, 22s, 4s,4,20s CWF_LONG_F3= 1 Enter in SBM1 mode Wait 235sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. Maximum solicitation in Normal mode and SBM1 TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 4 | In SBM1 mode TC LOAD_SBM1_PAR sends SY_LFR_S1_BP_P0=4s SY_LFR_S1_BP_P1=1s Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. TM_LFR_EXE_NOT_EXECUTABLE Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 5 | In standby mode TC LOAD_SBM1_PAR sends with one bad value SY_LFR_S1_BP_P0=0 SY_LFR_S1_BP_P1=1s Enter in SBM1 mode Wait 235 sec |  |  |  | TM_LFR_EXE_INCONSISTENT on SY_LFR_S1_BP_P0 parameter (PA_RPW_BYTE_POSITION=10 and PA_RPW_RCV_VALUE=0) Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 6 | In standby mode TC LOAD_SBM1_PAR sends with one bad value SY_LFR_S1_BP_P0=0.25 SY_LFR_S1_BP_P1=0s Enter in SBM1 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. TM_LFR_EXE_NOT_INCONSISTENT on sy_lfr_s1_bp_p1 parameter (PA_RPW_BYTE_POSITION=11 and PA_RPW_RCV_VALUE=0). Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 7 | In standby mode TC LOAD_SBM1_PAR sends with default values SY_LFR_S1_BP_P0=0.75 SY_LFR_S1_BP_P1=2s Enter in SBM1 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. TM_LFR_EXE_NOT_INCONSISTENT on sy_lfr_s1_bp_p0 parameter (PA_RPW_BYTE_POSITION=10 and PA_RPW_RCV_VALUE=3). 3=3*0.25 Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 8 | In standby mode TC LOAD_SBM1_PAR sends with max values SY_LFR_S1_BP_P0=0xff SY_LFR_S1_BP_P1=0xff Enter in SBM1 mode Wait 357 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. SY_LFR_S1_BP_P0=63.75s SY_LFR_S1_BP_P1=255s TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 9 | In standby mode TC LOAD_NORMAL_PAR sends with default values TC LOAD_SBM1_PAR sends with default values Enter in SBM1 mode Wait 420 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. All default values of parameters are used (SBM1 +NORMAL) No ASM because default value is 3600s TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 10 | A long test is done during 3 hours in SBM1 MODE with defaults values. SY_LFR_S1_BP_P0=0.25s SY_LFR_S1_BP_P0=1s in NORMAL MODE with defaults values. SY_LFR_N_SWF_L=default value SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=default value SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = 0 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. All default values of parameters are used (SBM1 +NORMAL) TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 11 | A long test is done during 3 hours in SBM1 MODE with defaults values. SY_LFR_S1_BP_P0=0.25s SY_LFR_S1_BP_P0=1s in NORMAL MODE with defaults values. SY_LFR_N_SWF_L=default value SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=default value SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM1 mode parameter set. All default values of parameters are used (SBM1 +NORMAL) TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM1_CWF_F1 TM_LFR_SCIENCE_SBM1_BP1_F0 TM_LFR_SCIENCE_SBM1_BP2_F0 |  |  |  |  |  |  |
| - 12 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS0032_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5506_Ed1 REQ-LFR-SRS-5548_Ed1 REQ-LFR-SRS-5500_Ed1 REQ-LFR-SRS-5514_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V3 V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 1h05 |  |  |  |
| Purpose: |  |  | Check, in the SBM2 mode, the LFR FSW sends properly data streams. The LFR FSW shall handle commands for configuring the different science sub-mode parameter sets. - SBM2 mode parameters: TC_LFR_LOAD_SBM2_PAR The LFR shall handle the following modes STANDBY NORMAL BURST SBM1 SBM2 |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management and Equipment configuration management |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-290, REQ-RPW-SYS-0303 REQ-RPW-SYS-0305 SSS-CP-EQS-190 SSS-CP-EQS-230 SSS-CP-EQS-370 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  | LFR is in SBM2 mode when RPW is in SBM2_ACQUISITION (_C); see § “LFR mode management” (§ 5.5.1). |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | Sbm2_mode_parameter_set_nominal.py (ASM=3600s) + sbm2_mode_parameter_set.py + long test with timegen |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | V3 |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Use sbm2_mode_parameter_set_nominal.py Launch the FSW LFR In standby mode No TC_LFR_LOAD_NORMAL_PAR (default values) noTC_LFR_LOAD_SBM2_PAR sent Enter in SBM1 mode with default values SY_LFR_S2_BP_P0=1s SY_LFR_S2_BP_P1=5s Wait 3960 sec. |  |  |  | Verify the LFR FSW sends high cadence data stream whose content corresponds to the SBM2 mode parameter set. For normal mode, we use default values. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | Use sbm2_mode_parameter_set.py Launch the FSW LFR In standby mode TC_LFR_LOAD_NORMAL_PAR with default values except for SY_LFR_N_ASM_P=4s no TC_LFR_LOAD_SBM2_PAR sent Enter in SBM2 mode with default values SY_LFR_S2_BP_P0=4s SY_LFR_S2_BP_P1=20s Wait 420 sec |  |  |  | Verify the LFR FSW sends high cadence data stream whose content corresponds to the SBM2 mode parameter set. For normal mode, we use default values. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In standby mode TC_LFR_LOAD_SBM2_PAR sends with default values SY_LFR_S2_BP_P0=1 SY_LFR_S2_BP_P1=5s And LOAD_NORMAL_MODE 2048, 22s, 4s,4,20s CWF_LONG_F3 =1 Enter in SBM1 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends high cadence data stream whose content corresponds to the SBM2 mode parameter set. For normal mode, we use default values. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In SBM2 mode TC LOAD_SBM2_PAR sends SY_LFR_S2_BP_P0=4s SY_LFR_S2_BP_P1=5s Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_EXE_NOT_EXECUTABLE Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In standby mode TC LOAD_SBM2_PAR sends with one bad value SY_LFR_S2_BP_P0=0 SY_LFR_S2_BP_P1=5s Enter in SBM2 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_EXE_NOT_INCONSISTENT on sy_lfr_s2_bp_p0 parameter (PA_RPW_BYTE_POSITION=10 and PA_RPW_RCV_VALUE=0) Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In standby mode TC LOAD_SBM2_PAR sends with one bad value SY_LFR_S2_BP_P0=1s SY_LFR_S2_BP_P1=0s Enter in SBM2 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_EXE_NOT_INCONSISTENT on sy_lfr_s2_bp_p1 parameter (PA_RPW_BYTE_POSITION=11 and PA_RPW_RCV_VALUE=0) Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In standby mode TC LOAD_SBM2_PAR sends with one bad value SY_LFR_S2_BP_P0=3s SY_LFR_S2_BP_P1=20s Enter in SBM2 mode Wait 235 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_EXE_NOT_INCONSISTENT on sy_lfr_s2_bp_p0 parameter (PA_RPW_BYTE_POSITION=10 and PA_RPW_RCV_VALUE=3) Keep the values used in step 3 TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In standby mode TC LOAD_SBM2_PAR sends with max values. SY_LFR_S2_BP_P0=0xff SY_LFR_S2_BP_P1=0xff Enter in SBM2 mode Wait 357 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | In standby mode TC LOAD_NORMAL_PAR sends with default values TC LOAD_SBM2_PAR sends with default values Enter in SBM2 mode Wait 420 sec |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. No ASM because default value is 3600s TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM2 MODE with defaults values. SY_LFR_S2_BP_P0=1s SY_LFR_S2_BP_P0=5s in NORMAL MODE with defaults values. SY_LFR_N_SWF_L=default value SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=default value SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = 0 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM2 MODE with defaults values. SY_LFR_S2_BP_P0=1s SY_LFR_S2_BP_P0=5s in NORMAL MODE with defaults values. SY_LFR_N_SWF_L=default value SY_LFR_N_SWF_P=default value SY_LFR_N_ASM_P=default value SY_LFR_N_BP_P0=default value SY_LFR_N_BP_P1=default value. SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the LFR FSW sends a high cadence data stream whose content corresponds to the SBM2 mode parameter set. TM_LFR_SCIENCE_NORMAL_SWF_Fi TM_LFR_SCIENCE_NORMAL_CWF_F3 TM_LFR_SCIENCE_NORMAL_BP1_Fi TM_LFR_SCIENCE_NORMAL_BP2_Fi TM_LFR_SCIENCE_NORMAL_ASM_Fi TM_LFR_SCIENCE_SBM2_CWF_F2 TM_LFR_SCIENCE_SBM2_BP1_F0 TM_LFR_SCIENCE_SBM2_BP1_F1 TM_LFR_SCIENCE_SBM2_BP2_F0 TM_LFR_SCIENCE_SBM2_BP2_F1 |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0033_Ed1 |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5507_Ed1 REQ-LFR-SRS-5508_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  | Version: | V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 2h30 |  |  |  |
| Purpose: |  | Check mode transitions correspond to the activation by the LFR FSW of the set of configuration parameters corresponding to the final mode. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  | Equipment mode management |  |  |  |  |  |  |  |  |
| dependencies |  | SSS-CP-EQS-300 SSS-CP-EQS-310 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  | transition_activation_4s.py then analyzeCuc() to check |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  | Note TM_LFR_PARAMETER_DUMP (after TC_LFR_DUMP_PAR) provides the parameters values. For each step, wait a convincing delay. For each mode, theoretical delay is T=max(Time between two snapshots, time between two ASM, time between two products, ...); concrete delay should be >2*T. TC_LFR_ENTER_MODE with CP_LFR_MODE=current mode are tested here. |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Launch the LFR FSW. Ensure the mode is STANDBY. TC_LFR_LOAD_NORMAL_PAR is sent with SY_LFR_N_ASM_P= 4s Realize the STANDBY->NORMAL modes transition. Ensure the current mode is NORMAL Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of NORMAL mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the STANDBY->BURST modes transition. Ensure the current mode is BURST. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of BURST mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the STANDBY->SBM1 modes transition. Ensure the current mode is SBM1. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM1 mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the STANDBY->SBM2 modes transition. Ensure the current mode is SBM2. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM2 mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM2->NORMAL modes transition. Ensure the current mode is NORMAL. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of NORMAL mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the NORMAL ->NORMAL modes transition. Ensure the current mode is NORMAL. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of NORMAL mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the NORMAL ->BURST modes transition. Ensure the current mode is BURST. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of BURST mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the BURST -> NORMAL modes transition. Ensure the current mode is NORMAL. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of NORMAL mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the NORMAL ->SBM1 modes transition. Ensure the current mode is SBM1. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM1 mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM1-> NORMAL modes transition. Ensure the current mode is NORMAL. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of NORMAL mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the NORMAL ->SBM2 modes transition. Ensure the current mode is SBM2. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM2 mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM2->BURST modes transition. Ensure the current mode is BURST. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of BURST mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the BURST-> BURST modes transition. Ensure the current mode is BURST. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of BURST mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the BURST-> SBM1 modes transition. Ensure the current mode is SBM1. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM1mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM1-> BURST modes transition. Ensure the current mode is BURST. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of BURST mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the BURST->SBM2 modes transition. Ensure the current mode is SBM2. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM2mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM2->SBM1 modes transition. Ensure the current mode is SBM1. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM1mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM1-> SBM1 modes transition. Ensure the current mode is SBM1. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM1 mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM1->SBM2 modes transition. Ensure the current mode is SBM2. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM2mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM2->SBM2 modes transition. Ensure the current mode is SBM2. Send TC_LFR_DUMP_PAR. Wait sufficient delay (see note above). |  |  | Verify the data stream is compliant with default values of SBM2 mode. In TM_LFR_PARAMETER_DUMP, verify parameters are consistent with current mode. |  |  |  |  |  |  |
|  | Realize the SBM2->STANDBY modes transition. Ensure the current mode is SBM2. Wait sufficient delay (see note above). |  |  | Verify there’s no science data stream. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0034_Ed2 |  |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5509_Ed1 REQ-LFR-SRS-5508_Ed1 REQ-LFR-SRS-5592_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  | 3.2.0.24 |  |  |  |  |  |  |  |  |
| HW: |  |  |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm |  |  |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm |  |  |  |  | Test Duration: |  | 2h02 + 10mn |  |  |  |
| Purpose: |  | Check the synchronization of the LFR mode transition on the due date, using its internal time. The equipment flight software shall ignore the synchronization bit of the CP_LFR_ENTER_MODE_TIME parameter (most significant bit) when they handle a TC_LFR_ENTER_MODE command. |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Equipment mode management |  |  |  |  |  |  |  |  |  |
| dependencies |  |  | Data time-stamping mechanism, SSS-CP-EQS-320 SSS-CP-EQS-310 SSS-CP-EQS-328 |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | activateLfrModeTiming.py (step 1&2) activateLfrModeNominal Cuc (step 3&4). |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | For the internal time, the synchronized/de-synchronized cases are treated consecutively. |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Ensure the LFR FSW is synchronized (MSB of the TIME fields is 0). For that, we send one TC_LFR_UPDATE_TIME + a valid time code . Go to SBM2 mode and Realize all the allowed transitions. |  |  |  |  | For each transition: Verify the TM_LFR_SCIENCE_* are compliant with LFR mode. With PA_LFR_ACQUISITION_TIME field of CWF products, verify transition occurs on the due date. |  |  |  |  |  |  |
|  | Ensure the FSW LFR is not synchronized (MSB of the TIME fields is 1). For that, we wait during SY_RPW_DELAY_WITHOUT_CTR*1,1s Go to SBM2 made and Realize all the allowed transitions. |  |  |  |  | For each transition: Verify the TM_LFR_SCIENCE_* are compliant with LFR mode. With PA_LFR_ACQUISITION_TIME field of CWF products, verify transition occurs on the due date. |  |  |  |  |  |  |
|  | MSB of CP_LFR_ENTER_MODE_TIME = 0 The CP_LFR_ENTER_MODE_TIME is the interne time + delay; realize all the allowed transitions with msb=0 for delay=1, 2, 3, 4s the fine time of the CP_LFR_ENTER_MODE_TIME is 0. |  |  |  |  | Check the synchronization of the LFR mode transition on the due date. For delay 1,2,3, all mode transitions are accepted. For delay > 3s , the transition is refused and a TM_LFR_TC_EXE_NOT_EXECUTABLE is generated. HK_LFR_MODE of TM_LFR_TC_EXE_NOT_EXECUTABLE showns the current working mode and not the asked mode. The MSB bit of the CP_LFR_ENTER_MODE_TIME has no-effect level. Transition at due date is realized. LFR FSW executes the mode transition immediately. HK_LFR_MODE of TM_LFR_TC_EXE_SUCCESS is equal to the asked mode. |  |  |  |  | REQ-LFR-SRS-5592_Ed1 |  |
|  | MSB of CP_LFR_ENTER_MODE_TIME = 1 The CP_LFR_ENTER_MODE_TIME is the interne time + delay; realize all the allowed transitions with msb=1 for delay=1, 2, 3, 4s . the fine time of the CP_LFR_ENTER_MODE_TIME is 0. |  |  |  |  | Check the synchronization of the LFR mode transition on the due date.. For delay > 3s , the transition is refused and a TM_LFR_TC_EXE_NOT_EXECUTABLE is generated. HK_LFR_MODE of TM_LFR_TC_EXE_NOT_EXECUTABLE showns the current working mode and not the asked mode. The MSB bit of the CP_LFR_ENTER_MODE_TIME has no-effect level. Transition at due date is realized. LFR FSW executes the mode transition immediately. HK_LFR_MODE of TM_LFR_TC_EXE_SUCCESS is equal to the asked mode. |  |  |  |  | REQ-LFR-SRS-5592_Ed1 |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0035_Ed1 |  |  |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  | REQ-LFR-SRS-5511_Ed1 REQ-LFR-SRS-5512_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  |  |  | Version: |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  |  |  |  |  |
| Purpose: |  | Check LFR FSW synchronizes the continuous waveform acquisitions on 1 s, with a precision higher than 500 μs, by using the SpW time code. For snapshot waveforms : check that LFR FSW centers, with a precision higher than 1 ms, this time window on the reference time that is transmitted as a parameter of the mode activating command. |  |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Acquisition synchronization |  |  |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-340 and SSS-CP-EQS-350 REQ-RPW-SYS-0350, REQ-RPW-SYS-0351 |  |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  | time_tests_cwf.pro + time_tests_swf.pro (T.Chust IDL programs) |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | CTC-512 (SSS-CP-EQS-340) and CTC-200 (SSS-CP-EQS-350) |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Realize a mode transition terminating in NORMAL mode: realize an acquisition. |  |  |  |  |  | Ensure that the frame rate is consistent with the Normal mode parameters : - Verify the continuous waveform acquisitions are synchronized on the second, with a precision higher than 500 μs (SSS-CP-EQS-340). - Verify that the snapshot waveforms time windows (F0, F1 and F2) are centered with a precision higher than 1ms and correctly positioned compared to the reference time that is transmitted as a parameter of mode activating command (SSS-CP-EQS-350). |  |  |  |  |  |  |
|  | Realize a mode transition terminating in BURST mode: realize an acquisition. |  |  |  |  |  | Ensure that the frame rate is consistent with the Burst mode parameters. Ensure that the frame rate is consistent with the Normal mode parameters : - Verify the continuous waveform acquisitions are synchronized on the second, with a precision higher than 500 μs (SSS-CP-EQS-340). |  |  |  |  |  |  |
|  | Realize a mode transition terminating in SBM1 mode; realize an acquisition. |  |  |  |  |  | Ensure that the frame rate is consistent with the SBM1 mode parameters. - Verify the continuous waveform acquisitions are synchronized on the second, with a precision higher than 500 μs (SSS-CP-EQS-340). - Verify that the snapshot waveforms time windows (F0, F1 and F2) are centered with a precision higher than 1ms and correctly positioned compared to the reference time that is transmitted as a parameter of mode activating command (SSS-CP-EQS-350). |  |  |  |  |  |  |
|  | Realize a mode transition terminating in SBM2 mode; realize an acquisition. |  |  |  |  |  | Ensure that the frame rate is consistent with the SBM2 mode parameters. - Verify the continuous waveform acquisitions are synchronized on the second, with a precision higher than 500 μs (SSS-CP-EQS-340). - Verify that the snapshot waveforms time windows (F0, F1 and F2) are centered with a precision higher than 1ms and correctly positioned compared to the reference time that is transmitted as a parameter of mode activating command (SSS-CP-EQS-350). |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0036_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5553_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 4mn 30 |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to receive and process the shared status/HK at regular time intervals (≤ 2000 ms) as TC_LFR_UPDATE_INFO packets. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Inter-equipment shared data TC_xxx_UPDATE_INFO packet. SSS-CP-EQS-351 |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-DAS-544: When its transmission is enabled, in all the modes except STANDBY, the DAS shall generate every SY_DPU_INFO_PKT_GEN_PER milliseconds (≤ 2000), toward the analyzers (LFR, THR, TDS), a TC_xxx_UPDATE_INFO packet. |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  | REQ-LFR-SRS-5564_Ed1 |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | update_info.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | No mode constraint and no delay constraint. |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  | Comments | Status |
|  | For each mode For delay 0.5,1,1.5,2,2.5 Generate a TC_LFR_UPDATE_INFO shared status/HK packet. |  |  |  |  | Verify the LFR FSW receives and processes the TC_LFR_UPDATE_INFO packet. In standby mode, LFR accepts TC_LFR_UPDATE_INFO. |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0037_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5541_Ed1 and REQ-LFR-5542_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  |  |  |  |  |
| Purpose: |  |  | The LFR FSW shall also report anomalies and errors (including SpaceWire errors) by using its periodic HK report and by managing the following parameters: - Low level error counter: incremented each time a new low level error / anomaly is detected by the equipment flight software. - Medium level error counter: incremented each time a new medium level error / anomaly is detected by the equipment flight software. - Last error report id: the error report id corresponds to the category of the error (AHB, SpaceWire, Buffer management, etc.); the last error report id parameter contains the id of the last error that has occurred. - Last error code: in a given error category, each error or anomaly that can occur is identified by an error code; the last error code parameter contains the code of the last error that has occurred. - Time of the last error. - Individual error counters: each individual error identified by its category and its code is associated to a specific error counter. The LFR FSW shall report the 2 levels of severity: low and medium . |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Inter-equipment shared data |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-120 and SSS-CP-EQS-130 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  | spw_failure.py (SVS-0064) starting_time.py (SVS-0011) starting_time_set_time.py starting_time_start.py starting_time_after.py (SVS-0057) update_timecode_missing_cnt_wrap.py update_time_and_info_not_synchro_cnt_wrap.py update_timecode_it_and_ctr_cnt_wrap check_rx_too_big_cnt_wrap (SVS-0026) |  |  |  |  |  |  |  |  |
| test script |  |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
| - 1 | Low error management |  |  |  | HK_LFR_LE_CNT is the sum of these errors counters: spacewire errors HK_LFR_DPU_SPW_PARITY, HK_LFR_DPU_SPW_DISCONNECT, HK_LFR_DPU_SPW_ESCAPE HK_LFR_DPU_SPW_CREDIT HK_LFR_DPU_SPW_WRITE_SYNC, specific counters HK_LFR_AHB_CORRECTABLE timing HK_LFR_TIMECODE_ERRONEOUS HK_LFR_TIMECODE_MISSING HK_LFR_TIMECODE_INVALID HK_LFR_TIME_TIMECODE_IT HK_LFR_TIME_NOT_SYNCHRO HK_LFR_TIME_TIMECODE_CTR The last error detected updates these fields HK_LFR_LAST_ER_RID HK_LFR_ER_CODE HK_LFR_LAST_ER_TIME |  |  |  |  | See SVS-0064 spw_failure.py No test See SVS-0011 and starting_time_*.py (SVS-0057) and update_timecode_* (SVS-0026) |  |
| - 2 | medium error management |  |  |  | HK_LFR_ME_CNT is the sum of these errors counters: spacewire errors HK_LFR_DPU_SPW_EARLY_EOP, HK_LFR_DPU_SPW_INVALID_ADDR, HK_LFR_DPU_SPW_EEP, HK_LFR_DPU_SPW_RX_TOO_BIG The last error detected updates these fields HK_LFR_LAST_ER_RID HK_LFR_ER_CODE HK_LFR_LAST_ER_TIME |  |  |  |  | See SVS-0064 spw_failure.py and check_rx_too_big_cnt_wrap (SVS-0026) |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0038_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5555_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 45mn |  |  |  |
| Purpose: |  |  | Check the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Inter-equipment shared data |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-353, REQ-RPW-SYS-160 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | 5 scripts, one by mode update_info_xxx.py (normal_long, long,sbm1,sbm2) |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | The procedure step described actions for each mode |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | For NORMAL mode, SBM1 mode and SBM2 mode send a TC_LFR_LOAD_NORMAL_PAR to set SY_LFR_N_ASM_P=4s Enter into the mode Send a TC_LFR_UPDATE_INFO With CP_DPU_BIAS_ON_OFF = 1 Wait the delay to see all TM_LFR_SCIENCE |  |  |  | Verify the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. List of the fields in TM_LFR_SCIENCE* PA_BIA_MODE_MUX_SET PA_BIA_MODE_HV_ENABLED PA__BIA_MODE_BIAS1_ENABLED PA__BIA_MODE_BIAS2_ENABLED PA__BIA_MODE_BIAS3_ENABLED PA_BIA_ON_OFF In this step, only PA_BIA_ON_OFF is updated = 1 |  |  |  |  |  |  |
|  | Send a TC_LFR_UPDATE_INFO With CP_BIA_MODE_MUX_SET = 0,1,2,3,4,5,5,7 Wait the delay to see all TM_LFR_SCIENCE |  |  |  | Verify the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. List of the fields in TM_LFR_SCIENCE* PA_BIA_ON_OFF = 1 And PA_BIA_MODE_MUX_SET = [0,7] |  |  |  |  |  |  |
|  | Send a TC_LFR_UPDATE_INFO With CP_BIA_MODE_HV_ENABLED = 1 Wait the delay to see all TM_LFR_SCIENCE |  |  |  | Verify the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. List of the fields in TM_LFR_SCIENCE* PA_BIA_ON_OFF = 1 PA_BIA_MODE_MUX_SET = 7 PA_BIA_MODE_HV_ENABLED = 1 |  |  |  |  |  |  |
|  | Send a TC_LFR_UPDATE_INFO With CP_BIA_MODE_BIAS1_ENABLED = 1 Wait the delay to see all TM_LFR_SCIENCE |  |  |  | Verify the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. List of the fields in TM_LFR_SCIENCE* PA_BIA_ON_OFF = 1 PA_BIA_MODE_MUX_SET = 7 PA_BIA_MODE_HV_ENABLED = 1 PA_ BIA_MODE_BIAS1_ENABLED =1 |  |  |  |  |  |  |
|  | Send a TC_LFR_UPDATE_INFO With CP_BIA_MODE_BIAS2_ENABLED = 1 Wait the delay to see all TM_LFR_SCIENCE |  |  |  | Verify the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. List of the fields in TM_LFR_SCIENCE* PA_BIA_ON_OFF = 1 PA_BIA_MODE_MUX_SET = 7 PA_BIA_MODE_HV_ENABLED = 1 PA_ BIA_MODE_BIAS1_ENABLED =1 PA_ BIA_MODE_BIAS2_ENABLED =1 |  |  |  |  |  |  |
|  | Send a TC_LFR_UPDATE_INFO With CP_BIA_MODE_BIAS3_ENABLED = 1 Wait the delay to see all TM_LFR_SCIENCE |  |  |  | Verify the LFR FSW extracts the relevant parameters from the shared status/HK packets for inserting them into their scientific TM packets. List of the fields in TM_LFR_SCIENCE* PA_BIA_ON_OFF = 1 PA_BIA_MODE_MUX_SET = 7 PA_BIA_MODE_HV_ENABLED = 1 PA_ BIA_MODE_BIAS1_ENABLED =1 PA_ BIA_MODE_BIAS2_ENABLED =1 PA_ BIA_MODE_BIAS3_ENABLED =1 |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0040_Ed2 |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5515_Ed1 REQ-LFR-SRS-5573_Ed1 REQ-LFR-SRS-5516_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  | Version: | V1 V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  | 35mn |  |  |  |
| Purpose: |  |  | Check the LFR FSW transmits the science data as packets compliant to the PUS service n°21. Check the LFR FSW always sets the segmentation grouping flag of the scientific TM packets to the value “Stand-alone packet”.(Value = 3) |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Science data acquisition and processing |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-GEN-FS-010, SSS-CP-EQS-380 SSS-CP-EQS-415 SSS-CP-EQS-390 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | service21_consistency.py and service21_consistency_asm_4s.py |  |  |  |  |  |  |
| output data |  |  |  | service21_consistency.py |  |  |  |  |  |  |
| notes |  |  |  | Used verif_fields.py for compliance. We only played service21_consistency_asm_4s.py |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  | Comments | Status |
|  | In standby mode, Send a TC_LFR_LOAD_NORMAL_PAR to set SY_LFR_ASM_P=4s Reach the NORMAL mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2 |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS =”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. Used default values of parameters except ASM=4s |  |  |  |  |  |
|  | Reach the BURST mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_BURST_CWF_F2, TM_LFR_SCIENCE_BURST_BP1_F0, TM_LFR_SCIENCE_BURST_BP2_F0, TM_LFR_SCIENCE_BURST_BP1_F1, TM_LFR_SCIENCE_BURST_BP2_F1. |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS_=”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. |  |  |  |  |  |
|  | Reach the SBM1 mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_SBM1_CWF_F1, TM_LFR_SCIENCE_SBM1_BP1_F0, TM_LFR_SCIENCE_SBM1_BP2_F0, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2. |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS_=”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. |  |  |  |  |  |
|  | Reach the SBM2 mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_SBM2_CWF_F2, TM_LFR_SCIENCE_SBM2_BP1_F0, TM_LFR_SCIENCE_SBM2_BP2_F0, TM_LFR_SCIENCE_SBM2_BP1_F1, TM_LFR_SCIENCE_SBM2_BP2_F1, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2. |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS_=”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. |  |  |  |  |  |
|  | Configure the TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 emission. Reach the NORMAL mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2 |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS =”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. |  |  |  |  |  |
|  | Reach the SBM1 mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_SBM1_CWF_F1, TM_LFR_SCIENCE_SBM1_BP1_F0, TM_LFR_SCIENCE_SBM1_BP2_F0, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2. |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS_=”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. |  |  |  |  |  |
|  | Reach the SBM2 mode. LFR FSW transmits each science data: TM_LFR_SCIENCE_SBM2_CWF_F2, TM_LFR_SCIENCE_SBM2_BP1_F0, TM_LFR_SCIENCE_SBM2_BP2_F0, TM_LFR_SCIENCE_SBM2_BP1_F1, TM_LFR_SCIENCE_SBM2_BP2_F1, TM_LFR_SCIENCE_NORMAL_SWF_F0, TM_LFR_SCIENCE_NORMAL_SWF_F1, TM_LFR_SCIENCE_NORMAL_SWF_F2, TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3, TM_LFR_SCIENCE_NORMAL_ASM_F0, TM_LFR_SCIENCE_NORMAL_ASM_F1, TM_LFR_SCIENCE_NORMAL_ASM_F2, TM_LFR_SCIENCE_NORMAL_BP1_F0, TM_LFR_SCIENCE_NORMAL_BP1_F1, TM_LFR_SCIENCE_NORMAL_BP1_F2, TM_LFR_SCIENCE_NORMAL_BP2_F0, TM_LFR_SCIENCE_NORMAL_BP2_F1, TM_LFR_SCIENCE_NORMAL_BP2_F2. |  |  |  | Verify packets are compliant to the PUS service n°21 (PUS_=”PUS_VERSION = 1”, SERVICE_TYPE = “SCIENCE_DATA_TRANSFER = 21”, SERVICE_SUBTYPE = “SCIENCE_REPORT = 3”. |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0041_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5519_Ed1 REQ-LFR-SRS-5524_Ed2 REQ-LFR-SRS-5513_Ed1 REQ-LFR-SRS-5569_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 20mn + 23h |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to generate continuous waveform in each science mode. Check the LFR FSW transmits to the DPU the continuous waveforms in data packet. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | LFR data products - continuous waveforms and LFR data packets - continuous waveforms |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-420 – SSS-CP-EQS-470 SSS-CP-EQS-360 |  |  |  |  |  |  |  |  |
| restrictions |  |  | Case STANDBY is not tested here. |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | cwf_generation.py + CTC-800 long tests + periodicity.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Launch LFR FSW LFR is in STANDBY mode . |  |  |  | Verify LFR doesn’t generate continuous waveforms. |  |  |  |  |  |  |
|  | LFR is in NORMAL mode without TC_LFR_LOAD_NORMAL_PAR Packet.; it generates continuous waveforms. |  |  |  | In TM_LFR_SCIENCE_NORMAL_CWF_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. After 168s (at least) in the current mode We receive a train of 4 CWF_F3 packets with a delta time of 42s between each one. |  |  |  |  |  |  |
|  | LFR is in BURST mode; it generates continuous waveforms. |  |  |  | In TM_LFR_SCIENCE_BURST_CWF_F2 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. After 11s in the current mode We receive a train of 8 CWF_F2 packets with a delta time of 1.315s between each one. |  |  |  |  |  |  |
|  | LFR is in SBM1 mode; it generates continuous waveforms. |  |  |  | In TM_LFR_SCIENCE_SBM1_CWF_F1 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. After 1.2s in the current mode We receive a train of 8 CWF_F1 packets with a delta time of 0.0820 between each one. In TM_LFR_SCIENCE_NORMAL_CWF_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. After 168s (at least) in the current mode We receive a train of 4 CWF_F3 packets with a delta time of 42s between each one. |  |  |  |  |  |  |
|  | LFR is in SBM2 mode; it generates continuous waveforms. |  |  |  | In TM_LFR_SCIENCE_SBM2_CWF_F2 Packet, verify continuous waveforms contains the following components: -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. After 11s in the current mode We receive a train of 8 CWF_F2 packets with a delta time of 1.315s between each one. In TM_LFR_SCIENCE_NORMAL_CWF_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. We receive a train of 4 CWF_F3 packets with a delta time of 42s between each one. The CWF_F3 dataflow is not be reset. We observe a delta time of 42s between the last CWF_F3 received in SBM1 and the first one of the train received in SBM2 mode. |  |  |  |  |  |  |
|  | LFR is in NORMAL mode after TC_LFR_LOAD_NORMAL_PAR Packet.; SY_LFR_N_CWF_LONG_F3 = 1 it generates continuous waveforms. |  |  |  | In TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. -Magnetic field components sampled at f3: B1_f3, B2_f3, B3_f3. After 168s (at least) in the current mode. We receive a train of 8 CWF_LONG_F3 packets with a delta time of 21s between each one. |  |  |  |  |  |  |
|  | LFR is in SBM1 mode; it generates continuous waveforms. SY_LFR_N_CWF_LONG_F3 = 1 |  |  |  | In TM_LFR_SCIENCE_SBM1_CWF_F1 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. After 1.2s in the current mode We receive a train of 8 CWF_F1 packets with a delta time of 0.0820 between each one. In TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. -Magnetic field components sampled at f3: B1_f3, B2_f3, B3_f3. The CWF_F3 dataflow is not be reset. We observe a delta time of 21s between the last CWF_LONG_F3 received in NORMAL mode and the first one of the train received in SBM1 mode. |  |  |  |  |  |  |
|  | LFR is in SBM2 mode; it generates continuous waveforms. SY_LFR_N_CWF_LONG_F3 = 1 |  |  |  | In TM_LFR_SCIENCE_SBM2_CWF_F2 Packet, verify continuous waveforms contains the following components: -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. After 11s in the current mode We receive a train of 8 CWF_F2 packets with a delta time of 1.315s between each one. In TM_LFR_SCIENCE_NORMAL_CWFLONG__F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. -Magnetic field components sampled at f3: B1_f3, B2_f3, B3_f3. The CWF_F3 dataflow is not be reset. We observe a delta time of 21s between the last CWF_LONG_F3 received in SBM1 mode and the first one of the train received in SBM2 mode. |  |  |  |  |  |  |
|  | A long test is done during 4h30 in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_NORMAL_CWF_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. |  |  |  |  |  |  |
|  | A long test is done during 15 hours in NORMAL MODE with SY_LFR_N_CWF_LONG_F3 = 0 SY_LFR_N_SWF_P = 22s SY_LFR_N_ASM_P = 4s We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. -Magnetic field components sampled at f3: B1_f3, B2_f3, B3_f3. |  |  |  |  |  |  |
|  | A long test is done during 14 hours in BURST MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_BURST_CWF_F2 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. |  |  |  |  |  |  |
|  | A long test is done during 2h30 in SBM1 MODE with defaults values. in NORMAL MODE with defaults values We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_SBM1_CWF_F1 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. In TM_LFR_SCIENCE_NORMAL_CWF_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM1 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_SBM1_CWF_F1 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. In TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. -Magnetic field components sampled at f3: B1_f3, B2_f3, B3_f3. |  |  |  |  |  |  |
|  | A long test is done during 1 hour in SBM2 MODE with defaults values. in NORMAL MODE with defaults values We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_SBM2_CWF_F2 Packet, verify continuous waveforms contains the following components: -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. In TM_LFR_SCIENCE_NORMAL_CWF_LONG_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. -Magnetic field components sampled at f3: B1_f3, B2_f3, B3_f3. |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM2 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | In TM_LFR_SCIENCE_SBM2_CWF_F2 Packet, verify continuous waveforms contains the following components: -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. In TM_LFR_SCIENCE_NORMAL_CWF_F3 packet, verify continuous waveforms contains the following components: -Electric field components sampled at f3: V_f3, E1_f3, E2_f3. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0042_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5520_Ed1 REQ-LFR-SRS-5525_Ed1 REQ-LFR-SRS-5513_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V1 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 42mn + 23h |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to generate waveform snapshots. Check the LFR FSW transmits to the DPU the waveform snapshot in data packets. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR data products - waveform snapshots and LFR data packets - waveform snapshots |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-430 SSS-CP-EQS-480 SSS-CP-EQS-360 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | swf_generation.py + verifs_fields.py long tests + periodicity.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | LFR is in STANDBY mode without TC_LFR_LOAD_NORMAL_PAR Packet. Wait ~330s. |  |  |  | The LFR FSW doesn’t generate waveform snapshots. |  |  |  |  |  |  |
|  | LFR realizes NORMAL mode transition (without TC_LFR_LOAD_NORMAL_PAR Packet). The LFR FSW generates waveform snapshots. |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. After 300s (at least ) in the current mode We receive a train of 7 NORMAL_SWF_Fi packets i=[0,2] F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s |  |  |  |  |  |  |
|  | LFR realizes BURST mode transition without TC_LFR_LOAD_NORMAL_PAR Packet. Wait ~330s. |  |  |  | The LFR FSW doesn’t generate waveform snapshots. |  |  |  |  |  |  |
|  | LFR realizes SBM1 mode transition (without TC_LFR_LOAD_NORMAL_PAR Packet). The LFR FSW generates waveform snapshots. |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 296 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 296 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. The SWF_Fi dataflow is not be reset. i=[0,2] We observe a delta time of right time between the last SWF_Fi received in SBM1 mode and the first one of the train received in NORMAL mode. F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s |  |  |  |  |  |  |
|  | LFR realizes SBM2 mode transition (without TC_LFR_LOAD_NORMAL_PAR Packet). The LFR FSW generates waveform snapshots. |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 296 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 296 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. The SWF_Fi dataflow is not be reset. I=[0,2] We observe a delta time of right time between the last SWF_Fi received in SBM2 mode and the first one of the train received in SBM1 mode. F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s |  |  |  |  |  |  |
|  | A long test is done during 4h30 in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. At 300s (at least) in the current mode We receive a train of 7 NORMAL_SWF_Fi packets i=[0,2] F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s |  |  |  |  |  |  |
|  | A long test is done during 15 hours in NORMAL MODE with SY_LFR_N_CWF_LONG_F3 = 0 SY_LFR_N_SWF_P = 22s SY_LFR_N_ASM_P = 4s We also use the timegen software (send CTR in nominal behavior). |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 22 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 22 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 22 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. At 38s (at least) in the current mode We receive a train of 7 NORMAL_SWF_Fi packets i=[0,2] F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s After the delta time between each first packet of a train is 22s. |  |  |  |  |  |  |
|  | A long test is done during 14 hours in BURST MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | The LFR FSW doesn’t generate waveform snapshots. |  |  |  |  |  |  |
|  | A long test is done during 2h30 in SBM1 MODE with defaults values. in NORMAL MODE with defaults values We also use the timegen software (send CTR in nominal behavior). |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. At 300s (at least) in the current mode We receive a train of 7 NORMAL_SWF_Fi packets i=[0,2] F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s |  |  |  |  |  |  |
|  | A long test is done during 1 hour in SBM2 MODE with defaults values. in NORMAL MODE with defaults values We also use the timegen software (send CTR in nominal behavior). |  |  |  | With TM_LFR_SCIENCE_NORMAL_SWF_F0, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f0: V_f0, E1_f0, E2_f0. -Magnetic field components sampled at f0: B1_f0, B2_f0, B3_f0. With TM_LFR_SCIENCE_NORMAL_SWF_F1, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f1: V_f1, E1_f1, E2_f1. -Magnetic field components sampled at f1: B1_f1, B2_f1, B3_f1. With TM_LFR_SCIENCE_NORMAL_SWF_F2, verify the waveform snapshot generated contains the following components: -2048 samples every 300 seconds. -Electric field components sampled at f2: V_f2, E1_f2, E2_f2. -Magnetic field components sampled at f2: B1_f2, B2_f2, B3_f2. At 300s (at least) in the current mode We receive a train of 7 NORMAL_SWF_Fi packets i=[0,2] F0: delta time between is 810/811 fine time = 0.03149s F1: delta time between is 1300 fine time = 0.0742s F2: delta time between is 3000 fine time = 0.1875s |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0043_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5521_Ed1 REQ-LFR-SRS-5526_Ed1 REQ-LFR-SRS-5513_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 9mn + 23h |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to generate averaged spectral matrixes. Check the LFR FSW transmits to the DPU the averaged spectral matrixes in data packets. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR data products - averaged spectral matrixes and LFR data packets - averaged spectral matrixes |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-440 SSS-CP-EQS-490 SSS-CP-EQS-360 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | Asm_generation_4s.py + verif_fields.py with tm_lfr_period rule Asm_generation.py + asm_generation_long_period_Normal/SBM1/SBM2.py (See long tests) + periodicity.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Use asm_generation_asm_4s.py Send a TC_LFR_LOAD_NORMAL_PAR with SY_LFR_ASM_P=4s LFR is in NORMAL mode. The LFR FSW generates averaged spectral matrixes. Wait 100s |  |  |  | Verify averaged spectral matrixes generated: - every 4s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | LFR is in SBM1 mode. The LFR FSW generates averaged spectral matrixes. |  |  |  | Verify averaged spectral matrixes generated: - every 4s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. the NORMAL_ASM_Fi dataflow is reset.i=[0,2] |  |  |  |  |  |  |
|  | LFR is in SBM2 mode. The LFR FSW generates averaged spectral matrixes. |  |  |  | Verify averaged spectral matrixes generated: - every 4s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. the NORMAL_ASM_Fi dataflow is reset.i=[0,2] |  |  |  |  |  |  |
|  | A long test is done during 4h30 in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify averaged spectral matrixes generated: - every 3600s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | A long test is done during 15 hours in NORMAL MODE SY_LFR_N_CWF_LONG_F3 =0 SY_LFR_N_SWF_P = 22s SY_LFR_N_ASM_P = 4s We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify averaged spectral matrixes generated: - every 3600s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | A long test is done during 2h30 in SBM1 MODE with defaults values. in NORMAL MODE with defaults values We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify averaged spectral matrixes generated: - every 3600s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM1 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify averaged spectral matrixes generated: - every 3600s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | A long test is done during 1 hour in SBM2 MODE with defaults values. in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify averaged spectral matrixes generated: - every 3600s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM2 MODE with defaults values. in NORMAL MODE with defaults values, except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify averaged spectral matrixes generated: - every 3600s seconds: -Averaged spectral matrix from the EM data stream at f0: ASM_f0 -Averaged spectral matrix from the EM data stream at f1: ASM_f1 -Averaged spectral matrix from the EM data stream at f2: ASM_f2. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0044_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5522_Ed2 REQ-LFR-SRS-5527_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 50mn + 23h |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to generate the set of basic parameters “set 1” Check the LFR FSW transmits to the DPU the set of basic parameters 1 in data packets. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | LFR data products - basic parameters set 1 and LFR data packets - basic parameters set 1 |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-450 SSS-CP-EQS-500 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  | LFR_packet_decom-linux-x86_64 (C++ LPP decom program) |  |  |  |  |  |  |  |  |
| test script |  |  | bp_generation_all_modes.py + verif_fields.py with tm_lfr_period rule + long tests + periodicity |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | PE_fn is the spectral power of E field from the electric data stream (2E) at fn; 12 or 24 bins PB_fn is the spectral power of B field from the magnetic data stream (3B) at fn; 12 or 24 bins nvec_fn is the wave normal vector from the magnetic data stream (3B) at fn; 12 or 24 bins ellip_fn is the wave ellipticity from the magnetic data stream (3B) at fn; 12 or 24 bins dop_fn is the degree of polarization from the magnetic data stream (3B) at fn; 12 or 24 bins Sz_fn is the normalized z-Poynting flux from the EM data stream (2E +3B) at fn; 12 or 24 bins Vphi_fn is the phase speed from the EM data stream (2E +3B) at fn; 12 or 24 bins Wait 650s in each mode |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | LFR is in NORMAL mode. The LFR FSW generates set of basic parameters “set 1”. |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1 -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2 |  |  |  |  |  |  |
|  | LFR is in BURST mode. The LFR FSW generates set of basic parameters “set 1”. |  |  |  | Verify the set of basic parameters generated: Every 1 second: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1 |  |  |  |  |  |  |
|  | LFR is in SBM1 mode. The LFR FSW generates set of basic parameters “set 1”. |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1 -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2 Every 0.25 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. |  |  |  |  |  |  |
|  | LFR is in SBM2 mode. The LFR FSW generates set of basic parameters “set 1”. |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2. Every 1 second: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. |  |  |  |  |  |  |
|  | A long test is done during 4h30 in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2. |  |  |  |  |  |  |
|  | A long test is done during 15 hours in NORMAL MODE with SY_LFR_N_CWF_LONG_F3 = 0 SY_LFR_N_SWF_P = 22s SY_LFR_N_ASM_P = 4s We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2. |  |  |  |  |  |  |
|  | A long test is done during 14 hours in BURST MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 1 second: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1 |  |  |  |  |  |  |
|  | A long test is done during 2h30 in SBM1 MODE with defaults values. in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1 -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2 Every 0.25 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM1 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1 -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2 Every 0.25 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0 |  |  |  |  |  |  |
|  | A long test is done during 1 hour in SBM2 MODE with defaults values. in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2. Every 1 second: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM2 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 4 seconds: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. -Basic parameter 1 at f2: PE_f2, PB_f2, nvec_f2, ellip_f2, dop_f2, Sz_f2, Vphi_f2. Every 1 second: -Basic parameter 1 at f0: PE_f0, PB_f0, nvec_f0, ellip_f0, dop_f0, Sz_f0, Vphi_f0. -Basic parameter 1 at f1: PE_f1, PB_f1, nvec_f1, ellip_f1, dop_f1, Sz_f1, Vphi_f1. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0045_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5523_Ed2 REQ-LFR-SRS-5528_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 50mn + 23h |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to generate the set of basic parameters “set 2” Check LFR FSW transmits to the DPU the set of basic parameters 2 in data packets |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | LFR data products - basic parameters set 2 and LFR data packets - basic parameters set 2 |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-510 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  | LFR_packet_decom-linux-x86_64 (C++ LPP decom program) |  |  |  |  |  |  |  |  |
| test script |  |  | bp_generation_all_modes. + verif_fields.py with tm_lfr_period rule long tests + periodicity.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | auto_fn corresponds to 5 auto variances from the EM data stream (2E +3B) at fn ; 12 or 24 bins cross_fn corresponds to 10 complex cross correlations from the EM data stream (2E +3B) at fn ; 12 or 24 bins |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | LFR is in NORMAL mode. The LFR FSW generates set of basic parameters “set 2”. |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 |  |  |  |  |  |  |
|  | LFR is in BURST mode. The LFR FSW generates set of basic parameters “set 2”. |  |  |  | Verify the set of basic parameters generated: Every 5 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 |  |  |  |  |  |  |
|  | LFR is in SBM1 mode. The LFR FSW generates set of basic parameters “set 2”. |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 Every 1 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 |  |  |  |  |  |  |
|  | LFR is in SBM2 mode. The LFR FSW generates set of basic parameters “set 2”. |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 Every 5 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 |  |  |  |  |  |  |
|  | A long test is done during 4h30 in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 |  |  |  |  |  |  |
|  | A long test is done during 15 hours in NORMAL MODE with SY_LFR_N_CWF_LONG_F3 = 0 SY_LFR_N_SWF_P = 22s SY_LFR_N_ASM_P = 4s We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 |  |  |  |  |  |  |
|  | A long test is done during 14 hours in BURST MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 5 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 |  |  |  |  |  |  |
|  | A long test is done during 2h30 in SBM1 MODE with defaults values. in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 Every 1 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM1 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 Every 1 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 |  |  |  |  |  |  |
|  | A long test is done during 1 hours in SBM2 MODE with defaults values. in NORMAL MODE with defaults values. We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 Every 5 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 |  |  |  |  |  |  |
|  | A long test is done during 3 hours in SBM2 MODE with defaults values. in NORMAL MODE with defaults values except to SY_LFR_N_CWF_LONG_F3 = 1 We also use the timegen software (send CTR in nominal behavior). |  |  |  | Verify the set of basic parameters generated: Every 20 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 -Basic parameter 2 at f2: auto_f2, cross_f2 Every 5 seconds: -Basic parameter 2 at f0: auto_f0, cross_f0 -Basic parameter 2 at f1: auto_f1, cross_f1 |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0053_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5558_Ed1 REQ-LFR-SRS-5556_Ed1 REQ-LFR-SRS-5557_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 23mn |  |  |  |
| Purpose: |  |  | Check the LFR FSW reports in its periodic HK packet (TM_LFR_HK) the enable/disable status of the calibration function. Check upon reception of a TC_LFR_ENABLE_CALIBRATION, the LFR FSW enables the LFR calibration function. Check upon reception of a TC_LFR_DISABLE_CALIBRATION, the LFR FSW disables the LFR calibration function. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | LFR calibration function |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-524 - SSS-CP-EQS-522 SSS-CP-EQS-523 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | hkCalEnabled_signal.py + Acceptance tests |  |  |  |  |  |  |  |  |
| output data |  |  | See plots into Calibration report. |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Run the LFR FSW wait 5s |  |  |  | After the boot sequence the Field HK_LFR_CALIB_ENABLED = 0 (DISABLED) |  |  |  |  |  |  |
|  | Send a TC_LFR_LOAD_NORMAL_PAR with SWF_P = 22s For each mode: -LFR receipts a TC_LFR_ENABLE_CALIBRATION. . . and stays 250 sec Wait 10s LFR receipts a TC_LFR_DISABLE_CALIBRATION |  |  |  | TC is acknowledged by a TM_LFR_TC_EXE_SUCCESS. Verify the LFR FSW enables the LFR calibration function (generation of the calibration signal for the SCM). Start the calibration signal Verify the LFR FSW reports in its periodic HK packet (TM_LFR_HK) the enable status of the calibration function. Field HK_LFR_CALIB_ENABLED = 1 when TC_LFR_ENABLE_CALIBRATION TC is acknowledged by a TM_LFR_TC_EXE_SUCCESS. Verify the LFR FSW disabled the LFR calibration function (generation of the calibration signal for the SCM). Stop the calibration signal Field HK_LFR_CALIB_ENABLED = 0 when TC_LFR_DISABLE_CALIBRATION |  |  |  |  | No mode constraint Only Normal, sbm1,sbm2 generate SWF products |  |
|  | For each mode Send TC_LFR_ENABLE_CALIBRATION Wait 3s Send TC_LFR_ENABLE_CALIBRATION Wait 3s Send TC_LFR_DISABLE_CALIBRATION Wait 3s Send TC_LFR_DISABLE_CALIBRATION |  |  |  | Each TC is acknowledged by a TM_LFR_TC_EXE_SUCCESS. In TM_LFR_HK Field HK_LFR_CALIB_ENABLED = 1 when TC_LFR_ENABLE_CALIBRATION Field HK_LFR_CALIB_ENABLED = 0 when TC_LFR_DISABLE_CALIBRATION |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0054_Ed2 |  |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5530_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: |  | V1 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  |  | 4s |  |  |  |  |
| Purpose: |  |  | Check the LFR FSW is ready to accept commands within SY_LFR_DELAY_ACC_TC milliseconds after the boot process has been completed. (500ms) |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Startup phase |  |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-010 |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | AD11 |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | delay_after_boot_process.py |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Tc_lfr_reset performs exit(0) |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Reset the LFR board By the Python script : -FSW is loaded, and not started. Realize as fast as possible the actions below: -Run the FSW, -Launch a TC_LFR_ENTER_MODE |  |  |  |  |  | Taking into account the technical uncertainty verify: -LFR FSW stays healthy during the boot process and startup phase. -when the boot process has just been completed, the LFR FSW accepts commands after SY_LFR_DELAY_ACC_TC. TC accepts with a TM_LFR_TC_EXE_SUCCESS Use in TM_LFR_HK HK_LFR_LAST_EXE_TC_TYPE=181 HK_LFR_LAST_EXE_SUBTYPE=41 HK_LFR_LAST_EXE_TC_TIME. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0055_Ed2 |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  | REQ-LFR-SRS-5531_Ed1 REQ-LFR-SRS-5535_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  | 5s |  |  |  |  |
| Purpose: |  | Check after successful time synchronization just after Startup phase, the LFR FSW enables the generation of its periodic housekeeping reports. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Startup phase |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-020 SSS-CP-EQS-060 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | synchro_startup_phase.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | The test case does not check the TC_LFR_RESET case. Use excel one file.csv and check that HK is compliant to PUS service 3 with verif_fields.py |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | -Launch the FSW Send a TC_LFR_UPDATE_TIME + time-code - Wait the generation of its periodic housekeeping reports.. |  |  | Verify the LFR FSW enables the generation of its periodic housekeeping reports. First TM_LFR_HK has its time=0x8xxxxxxxx After synchronization , the TM_LFR_HK has its time=0x00000002xxxx time is updated by CP-RPW_TIME of TC_LFR_UPDATE_TIME and MSB = 1 for synchronization HK_LFR_DPU_SPW_TICK_OUT_CNT=1, HK_LFR_DPU_SPW_LAST_TIMC=1 HK_LFR_UPDATE_TIME_TC_CNT=1 |  |  |  |  |  |  |
|  | See previous action. |  |  | Verify the LFR FSW sends its HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  | REQ-LFR-SRS-5535 |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0056_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5532_Ed1 REQ-LFR-SRS-5535_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration: |  | 37s |  |  |  |  |
| Purpose: |  | After Startup phase, check if no time synchronization has occurred within SY_LFR_TIME_SYN_TIMEOUT, the LFR FSW initiates the generation of its housekeeping reports using a non-synchronized time value. |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | Startup phase |  |  |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-030 SSS-CP-EQS-060 |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | timeout_synchro_after_boot.step01.py and timeout_synchro_after_boot.step02.py and and timeout_synchro_after_boot.step03.py |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | SY_LFR_TIME_SYN_TIMEOUT (Analyzer time synchronization timeout) is in AD11, 2000ms The TEST CASE does not treat the startup case after a TC_LFR_RESET. |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  |  | Comments | Status |
|  | FSW is loaded, and not started. Launch the Python script. Start the FSW (run): neither TC_LFR_UPDATE_TIMEs, neither time code. |  |  |  | SY_LFR_TIME_SYN_TIMEOUT after startup, verify the LFR FSW initiates the generation of its housekeeping reports using a non-synchronized time value: in TM_LFR_HK packets: MSB of TIME field is set to 1 .b1 (bit). in TM_LFR_HK HK_LFR_TIMECODE_MISSING=1 because automatic timecode is not launched. HK_LFR_LAST_ER_RID = LE_LFR_TIMEC (42129) HK_LFR_LAST_ER_CODE = MISSING (21) No timecodes: HK_LFR_DPU_SPW_TICK_OUT_CNT=0, HK_LFR_DPU_SPW_LAST_TIMC=0 HK_LFR_TIME_NOT_SYNCHRO= 0 this field is not updated because LFR must be synchronize first. HK_LFR_LE_CNT = 1 Verify the LFR FSW sends its HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  | Time of the first TM_LFR_HK= 0x80000002xxxx REQ-LFR-SRS-5535_Ed1 |  |
|  | FSW is loaded, and not started. Launch the Python script. Start the FSW (run): TC_LFR_UPDATE_TIMEs without time code. |  |  |  | SY_LFR_TIME_SYN_TIMEOUT after startup, verify in TM_LFR_HK packets: MSB of TIME field is set to 1 b1 (bit). in TM_LFR_HK HK_LFR_TIMECODE_MISSING=1 HK_LFR_TIME_TIMECODE_IT = 20 HK_LFR_LAST_ER_RID = LE_LFR_TIMEC (42129) HK_LFR_LAST_ER_CODE = MISSING (21) HK_LFR_TIME_NOT_SYNCHRO=0 this field is not updated because LFR must be synchronize first. No timecodes: HK_LFR_DPU_SPW_TICK_OUT_CNT=0, HK_LFR_DPU_SPW_LAST_TIMC=0 HK_LFR_UPDATE_TIME_CNT = 1 HK_LFR_LE_CNT = 1 Verify the LFR FSW sends its HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  | Time of the first TM_LFR_HK= 0x80000002xxxx REQ-LFR-SRS-5535_Ed1 |  |
|  | Exit of socexplorer. FSW is loaded, and not started. Launch the Python script. Start the FSW (run):20 time codes without TC_LFR_UPDATE_TIME. After 20 timecodes we stop it. |  |  |  | SY_LFR_TIME_SYN_TIMEOUT after startup, verify in TM_LFR_HK packets: MSB of TIME field is set to 1 b1 (bit). send 20 timecodes: HK_LFR_DPU_SPW_TICK_OUT_CNT=20, HK_LFR_DPU_SPW_LAST_TIMC=20 HK_LFR_TIME_TIMECODE_IT = 20 HK_LFR_LAST_ER_RID = LE_LFR_TIME (42119) HK_LFR_LAST_ER_CODE = TIMECODE_IT (24) HK_LFR_UPDATE_TIME_CNT is not incremented = 0 HK_LFR_LE_CNT =20 After automatic timecode stop HK_LFR_TIME_MISSING = 1 HK_LFR_LAST_ER_RID = LE_LFR_TIMEC (42129) HK_LFR_LAST_ER_CODE = MISSING (21) HK_LFR_LE_CNT =21 Verify the LFR FSW sends its HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  | Time of the first TM_LFR_HK= 0x80000002xxxx REQ-LFR-SRS-5535_Ed1 |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0057_Ed1 |  |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5533_Ed1 REQ-LFR-SRS-5534_Ed1 REQ-LFR-SRS-5535_Ed1 REQ-LFR-SRS-5230_Ed1 REQ-LFR-SRS-5214_Ed1 REQ-LFR-SRS-5215_Ed1 REQ-LFR-SRS-5216_Ed1 REQ-LFR-SRS-6002_Ed1 REQ-LFR-SRS-6102_Ed1 REQ-LFR-SRS-6105_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V1 /V3++ |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration: |  | 5mn30 |  |  |  |  |
| Purpose: |  |  | Check the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. Check the LFR FSW provides in HK report SW and HW status Check the LFR FSW sends its HK report as a packet compliant to the PUS service n°3. |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment HK reporting |  |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-040 SSS-CP-EQS-050 SSS-CP-EQS-060 SSS-CP-FS-520 SSS-CP-FS-150 SSS-CP-FS-160 SSS-CP-170 SSS-CP-EQS-752 SSS-CP-EQS-763 SSS-CP-EQS-766 |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  | REQ-LFR-SRS-5215_Ed1 (“The mode of housekeeping packet generation shall be the periodic mode.”) |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | periodical_hk.py hk_reporting (used by verif_fields) for analysis. 3 scripts for timecode management: starting_time_set_time.py, (step7) , starting_time_timecode_start.py (step 8) starting_time_timecode_start_after.py (step 9) |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | all tests verified these requirements |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  |  | Comments | Status |
|  | Launch LFR FSW and wait end of boot sequence no automatic timecode |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR after the boot sequence: TIME = 0x80000002xxxx HK_LFR_MODE = STANDBY HK_LFR_DPU_SPW_ENABLED: ENABLED = 1, HK_LFR_DPU_SPW_LINK_STATE: RUN = 5, HK_LFR_SC_POTENTIEL_FLAG: ON = 1 SY_LFR_PAS_FILTER_ENABLED: DISABLED =0, SY_LFR_WATCHDOG_ENABLED: ENABLED = 1, HK_LFR_CALIB_ENABLED: DISABLED = 0, HK_LFR_RESET_CAUSE: POWER_ON = 1 HK_LFR_SC_POTENTIEL_FLAG: ON = 1 Default parameters values: SY_LFR_BW=1, SY_LFR_SP0=0, SY_LFR_SP1=0, SY_LFR_R0=0, SY_LFR_R1=0, SY_LFR_R2=0 Specific parameters HK_LFR_VHDL_SM=0, HK_LFR_VHDL_IIR=0, HK_LFR_VHDL_CAL=0 HK_LFR_SC_V_F3, HK_LFR_SC_E1_F3, HK_LFR_SC_E2_F3 Error counters HK_LFR_LE_CNT = 1 Time counters : HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=1, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 HK_LFR_TIMECODE_MISSING because no automatic timecode. 16 S/C reaction wheel frequencies are unavailable - FLAG = 0 (DISABLED) The fields are HK_LFR_SC_RWi_Fj_FLAG with i=[1,4] and j=[1,4] A lot of fields to zero (HK_LFR_DPU_SPW_PKT_RCV_CNT=0, HK_LFR_DPU_SPW_PKT_SENT_CNT=0…. HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
|  | LFR is in STANDBY mode. Send all the TC. |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. HK_LFR_MODE = standby HK_LFR_CALIB_ENABLED: ENABLED = 1 when TC_LFR_ _ENABLE_CALIBRATION is sent HK_LFR_CALIB_ENABLED: DISABLED = 0, when TC_LFR_DISABLE_CALIBRATION is sent SY_LFR_PAS_FILTER_ENABLED: ENABLED = 1 when TC_LFR_LOAD_FILTER_PAR is sent with SY_LFR_PAS_FILTER_ENABLED_D =1 updated parameters values SY_LFR_BW, SY_LFR_SP0, SY_LFR_SP1,SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 if TC_LFR_LOAD_COMMON_PAR is sent updated HK_LFR_SC_RWi_Fj_FLAG with i=[1,4] and j=[1,4] if TC_LFR_UPDATE_INFO is sent (CP_RPW_SC_RWi_Fj = Nan or float value) FLAG=0 if Nan FLAG=1 if float value HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
|  | LFR is in NORMAL mode. Send all the TC. |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. HK_LFR_MODE = NORMAL HK_LFR_CALIB_ENABLED: ENABLED = 1 when TC_LFR_ _ENABLE_CALIBRATION is sent HK_LFR_CALIB_ENABLED: DISABLED = 0, when TC_LFR_DISABLE_CALIBRATION is sent SY_LFR_PAS_FILTER_ENABLED: ENABLED = 1 when TC_LFR_LOAD_FILTER_PAR is sent with SY_LFR_PAS_FILTER_ENABLED_D =1 updated parameters values SY_LFR_BW, SY_LFR_SP0, SY_LFR_SP1,SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 if TC_LFR_LOAD_COMMON_PAR is sent updated HK_LFR_SC_RWi_Fj_FLAG with i=[1,4] and j=[1,4] if TC_LFR_UPDATE_INFO is sent (CP_RPW_SC_RWi_Fj = Nan or float value) FLAG=0 if Nan FLAG=1 if float value HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
|  | LFR is in BURST mode. Send all the TC. |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. HK_LFR_MODE = BURST HK_LFR_CALIB_ENABLED: ENABLED = 1 when TC_LFR_ _ENABLE_CALIBRATION is sent HK_LFR_CALIB_ENABLED: DISABLED = 0, when TC_LFR_DISABLE_CALIBRATION is sent SY_LFR_PAS_FILTER_ENABLED: ENABLED = 1 when TC_LFR_LOAD_FILTER_PAR is sent with SY_LFR_PAS_FILTER_ENABLED_D =1 updated parameters values SY_LFR_BW, SY_LFR_SP0, SY_LFR_SP1,SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 if TC_LFR_LOAD_COMMON_PAR is sent updated HK_LFR_SC_RWi_Fj_FLAG with i=[1,4] and j=[1,4] if TC_LFR_UPDATE_INFO is sent (CP_RPW_SC_RWi_Fj = Nan or float value) FLAG=0 if Nan FLAG=1 if float value HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
|  | LFR is in SBM1 mode. Send all the TC. |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. HK_LFR_MODE = SBM1 HK_LFR_CALIB_ENABLED: ENABLED = 1 when TC_LFR_ _ENABLE_CALIBRATION is sent HK_LFR_CALIB_ENABLED: DISABLED = 0, when TC_LFR_DISABLE_CALIBRATION is sent SY_LFR_PAS_FILTER_ENABLED: ENABLED = 1 when TC_LFR_LOAD_FILTER_PAR is sent with SY_LFR_PAS_FILTER_ENABLED_D =1 updated parameters values SY_LFR_BW, SY_LFR_SP0, SY_LFR_SP1,SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 if TC_LFR_LOAD_COMMON_PAR is sent updated HK_LFR_SC_RWi_Fj_FLAG with i=[1,4] and j=[1,4] if TC_LFR_UPDATE_INFO is sent (CP_RPW_SC_RWi_Fj = Nan or float value) FLAG=0 if Nan FLAG=1 if float value HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
|  | LFR is in SBM2 mode. Send all the TC. |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. HK_LFR_MODE = SBM2 HK_LFR_CALIB_ENABLED: ENABLED = 1 when TC_LFR_ _ENABLE_CALIBRATION is sent HK_LFR_CALIB_ENABLED: DISABLED = 0, when TC_LFR_DISABLE_CALIBRATION is sent SY_LFR_PAS_FILTER_ENABLED: ENABLED = 1 when TC_LFR_LOAD_FILTER_PAR is sent with SY_LFR_PAS_FILTER_ENABLED_D =1 updated parameters values SY_LFR_BW, SY_LFR_SP0, SY_LFR_SP1,SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 if TC_LFR_LOAD_COMMON_PAR is sent updated HK_LFR_SC_RWi_Fj_FLAG with i=[1,4] and j=[1,4] if TC_LFR_UPDATE_INFO is sent (CP_RPW_SC_RWi_Fj = Nan or float value) FLAG=0 if Nan FLAG=1 if float value HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
|  | Run LFR FSW Send a automatic timecode Send a TC_LFR_UPDATE_TIME = PC current time with a valid time code Wait 10 s |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. After the boot Error counters HK_LFR_LE_CNT = 1 last error is HK_LFR_ERR_RID=42119, HK_LFR_ER_CODE= 24 , HK_LFR_ER_TIME= time Time counters : HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=1, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 HK_LFR_DPU_SPW_TICK_OUT_CNT is incremented HK_LFR_DPU_SPW_LAST_TIMC=timecode timecode analyze will updated HK_LFR_TIME_TIMECODE_ IT = 1 and last error is HK_LFR_ERR_RID=42119, HK_LFR_ER_CODE= 24 , HK_LFR_ER_TIME= time HK_LFR_LE_CNT is incremented LFR is synchronized HK_LFR_UPDATE_TIME_TC_CNT is incremented HK_LFR_DPU_SPW_TICK_OUT_CNT is incremented HK_LFR_DPU_SPW_LAST_TIMC=2 HK_LFR_TIME_TIMECODE_ IT = is incremented and last error is HK_LFR_ERR_RID=42119, HK_LFR_ER_CODE= 24 , HK_LFR_ER_TIME= time HK_LFR_LE_CNT is incremented HK_LFR_DPU_SPW_TICK_OUT_CNT is incremented HK_LFR_DPU_SPW_LAST_TIMC=timecode_ HK_LFR_TIME_TIMECODE_ IT is incremented HK_LFR_TIME_TIMECODE_CTR is incremented last error is HK_LFR_ERR_RID=42119, HK_LFR_ER_CODE= 26 , HK_LFR_ER_TIME= time HK_LFR_LE_CNT is incremented End of test HK_LFR_LE_CNT=33 HK_LFR_DPU_SPW_TICK_OUT_CNT=17 HK_LFR_DPU_SPW_LAST_TIMC=17 HK_LFR_TIMECODE_MISSING=1 HK_LFR_TIME_TIMECODE_IT=17 HK_LFR_TIME_TIMECODE_CTR=15 |  |  |  |  |  |  |  |
|  | Reset LFR Run LFR FSW and launch automatic timecode during boot sequence wait 10s |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. After the boot Error counters HK_LFR_LE_CNT = 0 last error is HK_LFR_ERR_RID=0, HK_LFR_ER_CODE= 0 , HK_LFR_ER_TIME= 0 Time counters : HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=0, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 End of test No timecode error. HK_LFR_DPU_SPW_TICK_OUT_CNT=16, HK_LFR_DPU_SPW_LAST_TIMC=16 |  |  |  |  |  |  |  |
|  | Reset LFR Run LFR FSW after boot sequence launch automatic timecode wait 10s |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. After the boot Error counters HK_LFR_LE_CNT = 1 last error is HK_LFR_ERR_RID=42119, HK_LFR_ER_CODE= 24 , HK_LFR_ER_TIME= time Time counters : HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=1, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 HK_LFR_TIME_TIMECODE_IT is incremented last error is HK_LFR_ERR_RID=42119, HK_LFR_ER_CODE= 24 , HK_LFR_ER_TIME= time HK_LFR_LE_CNT is incremented HK_LFR_DPU_SPW_TICK_OUT_CNT is incremented HK_LFR_DPU_SPW_LAST_TIMC=timecode End of test HK_LFR_LE_CNT=13 HK_LFR_DPU_SPW_TICK_OUT_CNT=12, HK_LFR_DPU_SPW_LAST_TIMC=12 HK_LFR_TIMECODE_MISSING=1 HK_LFR_TIME_TIMECODE_IT=12 |  |  |  |  |  |  |  |
|  | Long tests (15h) |  |  |  |  | Verify the LFR FSW provides periodically a TM_LFR_HK packet (HK report) reflecting the status of the LFR. HK report as a packet compliant to the PUS service n°3 (PACKET_CATEGORY=”HK_ROUTINE = 4”, PUS_VERSION=”PUS_VERSION = 1”, SERVICE_TYPE=”HOUSEKEEPING_AND_DIAGNOSTIC_DATA_REPORTING = 3)”. |  |  |  |  |  |  |  |
| - 11 | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0058_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5232_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V3 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 24s |  |  |  |  |
| Purpose: |  |  | When the watchdog reaches 0, the LFR FSW will stop. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Watchdog management |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-540 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | test_watchdog.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | LFR watchdog is implemented at software level and the only action is to exit (0). |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Run Fsw, in standby mode rwrite 0 into the watchdog register |  |  |  |  | LFR FSW stops. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0059_Ed2 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5544_Ed2 REQ-LFR-SRS-5208_Ed1 REQ-LFR-SRS-5210_Ed1 REQ-LFR-SRS-5202_Ed1 REQ-LFR-SRS-5545_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 V1 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 24s |  |  |  |  |
| Purpose: |  |  | Check the LFR FSW produces the command acknowledgment packets. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment command feedback |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-150 SSS-CP-FS-085 SSS-CP-FS-095 SSS-CP-FS-100 SSS-CP-EQS-160 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | ack.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Command acknowledgment packets are compliant to the PUS service n°1. See REQ-LFR-SRS-5543_Ed2 TM_LFR_TC_EXE_ERROR command is validated by design All LFR TC are implemented |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | In each mode, transmit each command to LFR in case of execution success and not executable TC. |  |  |  |  | Verify the LFR FSW produces the TM_LFR_TC_EXE_SUCCESS, TM_LFR_TC_EXE_ NOT_EXECUTABLE, command acknowledgment packet. |  |  |  |  |  |  |
|  | Transmit a command to LFR in case of error detected during the acceptance stage verifications (Illegal APID, Illegal packet type, Illegal packet subtype, wrong CRC, wrong or incomplete length). |  |  |  |  | Verify the LFR FSW produces the TM_LFR_TC_EXE_CORRUPTED command acknowledgment packet. |  |  |  |  |  |  |
|  | Transmit a command to LFR in case of wrong or inconsistent field (in the header fields or in the data fields. |  |  |  |  | Verify the LFR FSW produces the TM_LFR_TC_EXE_INCONSISTENT command acknowledgment packet. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0060_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5536_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V1 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 39h |  |  |  |  |
| Purpose: |  |  | The generation period of the HK equipment report packets is fixed to SY_LFR_HK_SAMPLING_PER = 1 second. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment HK reporting |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-070 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | test_mode_burst.py test_mode_normal.py test_mode_SBMx_12h.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | periodicity.py checks the generation period of HK report fixed to SY_LFR_HK_SAMPLING_PER = 1 verif_fields.py checks all TM_LFR_HK in all validation tests. |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
| - 1 | LFR is in NORMAL Mode during 13:40 hours |  |  |  |  | Verify the periodicity of TM_LFR_HK mean of all HK delta times = 0.99999443s median of all HK delta times : 0.999985s 52339 TM_LFR_HK generated |  |  |  |  |  |  |
| - 2 | LFR is BURST Mode during 12hours |  |  |  |  | Verify the periodicity of TM_LFR_HK Mean of all HK delta times : 0.9999928759 s Median of all HK delta times : 1.0.s 43200 TM_LFR_HK generated |  |  |  |  |  |  |
| - 3 | LFR is SBM1 Mode during hours then SBM2 during 6hours |  |  |  |  | Verify the periodicity of TM_LFR_HK mean of all HK = 0.99999308s median of all HK delta times : 1.0s 44425 TM_LFR_HK generated |  |  |  |  |  |  |
| - 4 | LFR is in NORMAL MODE (3630s) then LFR is in SBM1 MODE (1800s) then LFR is in NORMAL MODE (3600s) then LFR is in SBM2 MODE (1800s) then LFR is in NORMAL MODE (3600s) then LFR is in SBM1 MODE (1800s) then LFR is in SBM2 MODE (1800s) then LFR is in SBM1 MODE (1830s) then LFR is in NORMAL MODE (3600s) |  |  |  |  | Verify the periodicity of TM_LFR_HK mean of all HK = 0.99999450 median of all HK delta times : 0.999985s 23459 TM_LFR_HK generated |  |  |  |  |  |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0061_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5540_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | N/A |  |  |  |  |
| Purpose: |  |  | The LFR FSW shall report the normal progress of operations and activities having an operational significance by updating the value of the suitable status parameter in its periodic HK report. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment Event reporting |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-110 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | all tests |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Only dynamic fields are described and this list is not exhaustive. verif_fields.py checks all TM_LFR_HK in all validation tests. |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
| - 1 | Common parameters sent by TC_LFR_LOAD_COMMON_PAR |  |  |  |  | in TM_LFR_HK, SY_LFR_BW, SY_LFR_SP0, SY_LFR_SP1,SY_LFR_R0, SY_LFR_R1,SY_LFR_R2 |  |  |  |  | See SVS-0095 |  |
| - 2 | Calibration mode activity sent by TC_LFR_ENABLE_CALIBRATION or TC_LFR_DISABLE_CALIBRATION |  |  |  |  | in TM_LFR_HK HK_LFR_CALIB_ENABLED |  |  |  |  | See SVS-0053 |  |
| - 3 | LFR mode sent by TC_LFR_ENTER_MODE |  |  |  |  | in TM_LFR_HK HK_LFR_MODE = current working mode |  |  |  |  | See SVS-0034 |  |
| - 4 | TC_LFR_LOAD_FILTER_PAR |  |  |  |  | in TM_LFR_HK SY_LFR_PAS_FILTER_ENABLED_D |  |  |  |  | See SVS-0096 SVS-1202 |  |
| - 5 | Specific LFR status |  |  |  |  | in TM_LFR_HK HK_LFR_VHDL_SM HK_LFR_VHDL_IIR HK_LFR_VHDL_CAL HK_LFR_SC_V_F3 HK_LFR_SC_E1_F3 HK_LFR_SC_E2_F3 |  |  |  |  | all tests CTC-800 |  |
| - 6 | TC management ( acceptance, execution, statistics) |  |  |  |  | in TM_LFR_HK TC_LFR_UPDATE_xxx_CNT (TIME, INFO) execution TC HK_LFR_EXE_xxxx Reject TC HK_LFR_REJ_xxxx |  |  |  |  | See SVS-0003 to SVS-0010 |  |
| - 7 | Errors management (statistics anomalies) TC are managed in step 5 |  |  |  |  | in TM_LFR_HK Timing errors Spacewire errors Anomaly_statistics in HK_LFR_LE_CNT and HK_LFR_ME_CNT |  |  |  |  | SVS-0012 to SVS-0013 SVS-0026 SVS-0064 |  |
| - 8 | resource statistics |  |  |  |  | in TM_LFR_HK HK_LFR_CPU_LOAD HK_LFR_CPU_LOAD_MAX HK_LFR_CPU_LOAD_AVE HK_LFR_Q_SD_FIFO_SIZE_MAX, HK_LFR_Q_RV_FIFO_SIZE_MAX, HK_LFR_Q_P0_FIFO_SIZE_MAX, HK_LFR_Q_P1_FIFO_SIZE_MAX HK_LFR_Q_P2_FIFO_SIZE_MAX about spacewire interface HK_LFR_DPU_SPW_PKT_RCV_CNT, HK_LFR_DPU_SPW_PKT_SENT_CNT HK_LFR_DPU_SPW_TICK_OUT_CNT HK_LFR_DPU_SPW_LAST_TIMC |  |  |  |  | All tests |  |
| - 9 | Reaction Wheels Frequencies TC_LFR_UPDATE_INFO field CP_RPW_SC_RWi_Fi i=[1,4] and j=[1,4] either 16 frequencies |  |  |  |  | In TM_LFR_HK HK_LFR_SC_RWi_Fj_FLAG i=[1,4] and j=[1,4] |  |  |  |  | SVS-1100 |  |
| - 10 | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0062_Ed1 |  |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5561_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration: |  | N/A |  |  |  |  |
| Purpose: |  |  | Check the LFR procedure after the loss of the SpW connection, if the analyzer has failed to (re-)establish the connection within a SY_LFR_DPU_CONNECT_TIMEOUT timeout period. |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | SpaceWire link monitoring |  |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-153 |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | Manually tested. |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Inspection is better to valid this requirement |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  |  | Comments | Status |
|  | Realize a loss of the SpW connection (disconnect the SpW). Fail the LFR analyzer to establish the connection within a SY_LFR_DPU_CONNECT_TIMEOUT timeout period. |  |  |  |  | Verify the LFR FSW: -Resets the SpW interface. -Resets the connection timeout period. -Starts again the connection process. |  |  |  |  |  |  |  |
|  | Fail the LFR analyzer to re-establish the connection within a SY_LFR_DPU_CONNECT_TIMEOUT timeout period. |  |  |  |  | Verify the LFR FSW: -Resets the SpW interface. -Resets the connection timeout period. -Starts again the connection process. |  |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0063_Ed1 |  |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5562_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration: |  | N/A |  |  |  |  |
| Purpose: |  |  | Check the LFR FSW performs up to SY_LFR_DPU_CONNECT_ATTEMPT attempts of connection. |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | SpaceWire link monitoring |  |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-154 |  |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | Manually |  |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Minicom tool shows the attemps of connection |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  |  | Comments | Status |
|  | The LFR FSW realizes SY_LFR_DPU_CONNECT_ATTEMPT-1 unsuccessful attempts of connection. |  |  |  |  | The LFR FSW tries one more time a connection. |  |  |  |  |  | Used Minicom |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0064_Ed1 |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  | REQ-LFR-SRS-5563_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 2mn |  |  |  |
| Purpose: |  | Check, after SY_LFR_DPU_CONNECT_ATTEMPT unsuccessful attempts of connection, the LFR FSW enters into STANDBY. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  | SpaceWire link monitoring |  |  |  |  |  |  |  |
| dependencies |  |  | SSS-CP-EQS-155 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  | spw_failure.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | WARNING: We disconnect the spacewire plug manually so it ‘s difficult to obtain always the same SPW errors. |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  | Comments | Status |
|  | Run the LFR FSW with automatic timecodes For each mode The LFR FSW realizes SY_LFR_DPU_CONNECT_ATTEMPT unsuccessful attempts of connection. |  |  |  | Verify after SY_LFR_DPU_CONNECT_ATTEMPT unsuccessful attempts of connection, the LFR FSW enters into STANDBY. We observed spacewire errors in the TM_LFR_HK HK_LFR_DPU_SPW_PARITYHK_LFR_DPU_SPW_DISCONNECT HK_LFR_DPU_SPW_ ESCAPE and HK_LFR_LE_CNT is updated. Sometimes HK_LFR_DPU_SPW_EARLY_EOP and HK_LFR_ME_CNT is updated. HK_LFR_TIMECODE_ERRONEOUS and HK_LFR_TIMECODE_INVALID are incremented. The last error kept into HK_LFR_ER_RID HK_LFR_ER_CODE HK_LFR_ER_TIME |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0065_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / N / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5551_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 27mn |  |  |  |
| Purpose: |  |  | Check the LFR FSW allows dumping in TM_LFR PARAMETER_DUMP packets, upon the reception of TC_LFR_DUMP_PAR command, all their functional and operational configuration parameters. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment parameter dump |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-215 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | Reference_values_for_diff.asc files to diff with extract.py output |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | Send_nominal_tc_dump_all_modes.py + extract.py (step1) Send_modified_tc_dump_all_modes.py + extract.py (step2) Send_bad_tc_dump_all_modes.py + extract.py (step3 step 4) |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | TC created for R3 and R3+ are tested |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | This step must be launched after boot or reboot of fsw to ensure nominal mode and SEQUENCE_CNT verification. Send a TC_LFR_DUMP_PAR to valid default parameters after the boot Default parameters: In each LFR modes,LFR receipts TC_LFR_DUMP_PAR command send send a TC_LFR_UPDATE_INFO send a TC_LFR_DUMP_PAR send a TC_LFR_LOAD_FBINS_MASKS send a TC_LFR_DUMP_PAR send a TC_LFR_LOAD_FILTER_PAR send a TC_LFR_DUMP_PAR |  |  |  | After the boot Verify the LFR FSW dumps in TM_LFR PARAMETER_DUMP packets all their functional and operational configuration parameters (software and hardware) for each mode and with default parameters. Default values for section SY_LFR_COMMON_PARAMETERS SY_LFR_NORMAL_PARAMETERS SY_LFR_BURST_PARAMETERS SY_LFR_SBM1_PARAMETERS SY_LFR_SBM2_PARAMETERS SY_LFR_RW_MASK SY_LFR_FBINS_MASKS SY_LFR_FILTER_PARAMETERS if a TC_LFR_UPDATE_INFO is sent with default values (CP_RPW_SC_RWi_Fi = Nan with i=[1,4] ans j =[1,4] On TM_LFR_PARAMETER_DUMP no changed on section SY_LFR_RW_MASK (PA_LFR_RW_MASK_Fi_WORDj with i=[1,4] ans j =[1,4] if a TC_LFR_LOAD_FBINS_MASK is sent with default values (FBINS_Fi_WORDj with i=[1,4] ans j =[1,4] (bit=1) On TM_LFR_PARAMETER_DUMP no changed on section SY_LFR_RW_MASK (SY_LFR_FBINS_Fi_WORDj with i=[1,4] ans j =[1,4] (bit=1) if a TC_LFR_LOAD_FILTER_PAR is sent with default values SY_LFR_PAS_FILTER_ENABLED_D = ENABLED =1 On TM_LFR_PARAMETER_DUMP Changed on SY_LFR_PAS_FILTER_ENABLED_D field = ENABLED. After the boot SY_LFR_PAS_FILTER_ENABLED_D = DISABLED until a TC_LFR_LOAD_FILTER should be sent. |  |  |  |  | We use diff between trace extract and a reference file with values expected |  |
|  | Modified parameters : This step must be launched after boot or reboot of fsw For each mode Send a TC_LFR_COMMON_MODE with SY_LFR_BW=0 SY_LFR_SP0=1 SY_LFR_SP1=1 SY_LFR_R0=1 SY_LFR_R1=1 SY_LFR_R2=1 Send TC_LFR_DUMP_PAR Send a TC_LFR_COMMON_MODE with default values Send TC_LFR_DUMP_PAR For each mode send the allowed TC_LFR_LOAD_*_PAR commands with coherent sample values (non default) and TC_LFR_DUMP_PAR For each mode send a TC_LFR_UPDATE_INFO with a floating value (not a Nan) CP_RPW_RW1_F1 and CP_RPW_RW1_F2 and a TC_LFR_DUMP_PAR For each mode send a TC_LFR_LOAD_FBINS_MASK with FBINS_F0_WORDi = 0 i = [1,4] and a TC_LFR_DUMP_PAR For each mode send a TC_LFR_LOAD_FILTER_PAR with SY_LFR_PAS_FILTER_ENABLED = ENABLED SY_LFR_PAS_FILTER_TBAD = 3.0 SY_LFR_PAS_FILTER_OFFSET = 1 SY_LFR_PAS_FILTER_SHIFT=0 and a TC_LFR_DUMP_PAR |  |  |  | Verify the LFR FSW dumps in TM_LFR PARAMETER_DUMP packets all their functional and operational configuration parameters (software and hardware) for each mode and that they are equal to injected ones. Updated parameters for section SY_LFR_COMMON_PARAMETERS SY_LFR_NORMAL_PARAMETERS SY_LFR_BURST_PARAMETERS SY_LFR_SBM1_PARAMETERS SY_LFR_SBM2_PARAMETERS SY_LFR_RW_MASK SY_LFR_FBINS_MASKS SY_LFR_FILTER_PARAMETERS |  |  |  |  | We use diff between trace extract and a reference file with values expected |  |
|  | Bad parameters : This step must be launched after boot or reboot of fsw Go to Standby mode we first load non default parameters and then try to load non authorized values(out of range or impossible) with all TC_LFR_LOAD_*_PAR commands and we send TC_LFR_DUMP_PAR for each. send a TC_LFR_UPDATE_INFO with a negative rw frequency (CP_RPW_SC_RWi_Fi with i=[1,4] and j=[1,4] and send a TC_LFR_DUMP_PAR send a TC_LFR_LOAD_FBINS_MASK and send a TC_LFR_DUMP_PAR send a TC_LFR_LOAD_KCOEFFICIENTS with one KCOEFF with negative value and send a TC-LFR_DUMP_PAR send a TC_LFR_LOAD_FILTER_PAR with a bad value for each parameter (Modulus, Tbad, shift, offset , inconsistent Modulus vs shift + offset) TC_LFR_DUMP_PAR |  |  |  | Verify that parameters contained in the TM_LFR PARAMETER_DUMP packets have not been updated to bad ones and not forced to default. Each Parameter of the TC_LFR_LOAD_*_PAR is ent with a bad value and coherence between parameters is also tested. TC_LFR_LOAD_COMMON_PAR hasn’t bad parameter value. TC_LFR_LOAD_FBINS_MASK hasn’t bad parameter value. |  |  |  |  | We use diff between trace extract and a reference file used in step 3 |  |
|  | For each scientific mode we first load non default parameters and then try to load non authorized values(out of range or impossible) with all TC_LFR_LOAD_*_PAR commands and we send TC_LFR_DUMP_PAR for each. send a TC_LFR_UPDATE_INFO with a negative rw frequency (CP_RPW_SC_RWi_Fi with i=[1,4] and j=[1,4] and send a TC_LFR_DUMP_PAR send a TC_LFR_LOAD_FBINS_MASK and send a TC_LFR_DUMP_PAR send a TC_LFR_LOAD_FILTER_PAR inconsistent Modulus vs shift + offset) and send a TC_LFR_DUMP_PAR |  |  |  | Verify that parameters contained in the TM_LFR PARAMETER_DUMP packets have not been updated to bad ones and not forced to default TC_LFR_LOAD_COMMON_PAR hasn’t bad parameter value. TC_LFR_LOAD_FBINS_MASK hasn’t bad parameter value. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0066_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5552_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 13s |  |  |  |
| Purpose: |  |  | Check, upon reception of a TC_LFR_RESET command, the LFR FSW forces the reboot of the LFR: the initial state after a reset or after a boot is then the STANDBY mode. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Software reset |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-220 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | just_tc_lfr_reset.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Tc_lfr_reset performs exit(0) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | LFR FSW receipts a TC_LFR_RESET command. |  |  |  | This TC is not acknowledged because exit (0) is performed. Environment test displays A TM_LFR_TC_EXE_* is not seen after time-out (4s). |  |  |  |  |  |  |
|  | Boot the LFR. |  |  |  | Verify the initial state after a boot is the STANDBY mode. |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0067_Ed1 |  |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5517_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 4mn5 |  |  |  |
| Purpose: | Check the LFR FSW uses its internal time to time-stamp all the packets. |  |  |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  |  | Software reset (NOT IMPLEMENTED) |  |  |  |  |  |  |  |
| dependencies |  |  |  |  | SSS-CP-EQS-400 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  |  | activeLfrinternaltime.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  |  | Begin in SBM2 mode during 6s after boot Reach each mode LFR FSW receipts a S/C time by TC_LFR_UPDATE_TIME+ time code Wait 5s |  |  |  | Verify the LFR FSW time-stamps all the HK packets. Verify the LFR FSW time-stamps all the science data packets. MSB time = 1 Verify the LFR FSW time-stamps all the HK packets Verify the LFR FSW time-stamps all the science data packets. MSB time = 0 |  |  |  |  |  |  |
|  |  | Reach each mode Wait 1.1*SY_RPW_DELAY_WITHOUT_CTR. |  |  |  | Verify the LFR FSW time-stamps all the HK packets. The first TM_LFR_HK after the synchronization lost has the fields HK_LFR_TIME_NOT_SYNCHRO and HK_LFR_LE_CNT incremented by 1. Verify the LFR FSW time-stamps all the science data packets. MSB time = 1 lost synchro |  |  |  |  |  |  |
|  |  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0069_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5565_Ed1 REQ-LFR-SRS-5592_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: | 1.1.91 StarDUndee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 3mn10 |  |  |  |
| Purpose: |  |  | Check the LFR FSW executes the mode transition on the next sharp second compared to current LFR time (current coarse time +1) when the time given in parameter of the TC_LFR_ENTER_MODE packet is equal to 0. The equipment flight software shall ignore the synchronization bit of the CP_LFR_ENTER_MODE_TIME parameter (most significant bit) when they handle a TC_LFR_ENTER_MODE command. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-321 SSS-CPEQS-328 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | activateLfrMode0.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step |  | Actions |  |  |  | Expected Results |  |  |  | Comments | Status |
|  |  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME =0. |  |  |  | Verify LFR FSW executes the mode transition immediately, without waiting for time synchronization. HK_LFR_MODE of TM_HK_LFR is equal to the asked mode. PA_LFR_ACQUISITION_TIME of TM_LFR_SCIENCE_CWF_Fx allows to valid this transition mode |  |  |  |  |  |
|  |  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME= 0 with msb=1. |  |  |  | Verify LFR SWF executes the mode transition immediately, without waiting for time synchronization. HK_LFR_MODE of TM_HK_LFR is equal to the asked mode. PA_LFR_ACQUISITION_TIME of TM_LFR_SCIENCE_CWF_Fx allows to valid this transition mode The MSB bit of the CP_LFR_ENTER_MODE_TIME has no-effect level. LFR FSW executes the mode transition immediately. HK_LFR_MODE of TM_LFR_TC_EXE_SUCCESS is equal to the asked mode. |  |  |  | REQ-LFR-SRS-5592_Ed1 |  |
|  |  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0070_Ed1 |  |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5566_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration: |  | 6mn5 |  |  |  |  |
| Purpose: |  |  |  | Check the LFR FSW rejects the TC_LFR_ENTER_MODE packet if the CP_LFR_ENTER_MODE_TIME parameter is lower than the current time. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  |  |  | Equipment mode management |  |  |  |  |  |  |  |
| dependencies |  |  |  |  |  | SSS-CP-EQS-322 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  |  |  | activateLfrModeRetroactive.py (steps 1,2), |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  |  |  |  | Expected Results |  | Comments | Status |
|  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME-current TIME=-1(fine time forced to 0), -1/0x10000. |  |  |  |  |  |  |  |  | Verify LFR FSW rejects the TC_LFR_ENTER_MODE packet. and one TM_LFR_EXE_NOT_EXECUTABLE is generated. Field HK_LFR_MODE of TM_LFR_EXE_NOT_EXECUTABLE showns the current working mode . |  |  |  |
|  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME-current TIME=-1(fine time forced to 0), -1/0x10000. with msb=1. |  |  |  |  |  |  |  |  | Verify LFR FSW rejects the TC_LFR_ENTER_MODE packet. and one TM_LFR_EXE_NOT_EXECUTABLE is generated. Field HK_LFR_MODE of TM_LFR_EXE_NOT_EXECUTABLE showns the current working mode . |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0071_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5567_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 5mn10 |  |  |  |  |
| Purpose: |  |  | Verify the LFR FSW accepts all the transition between modes excepting if the destination mode is equal to the source mode. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-325 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | activateLfrMode.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  |  |  | Expected Results |  | Comments | Status |
|  | Realize all the mode transitions by the complete sequence: 4->0->0->1->0->2->0->3->0->4->1->1->2->1->3->1->4->2->2->3->2->4->3->3->4->4->0. Notice: -Sends comply with timing constraints. -For the four Science sub-modes: don’t send the TC_LFR_LOAD_xxx_PAR before the TC_LFR_ENTER_MODE. |  |  |  |  |  |  |  | Verify the LFR FSW: -accepts all the transition between modes excepting if the destination mode is equal to the source mode, in this case TM_LFR_EXE_NOT_EXECUTABLE. -the HK_LFR_MODE field from TM_LFR_HK is consistent -the TM_LFR_SCIENCE_* are consistent. |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0073_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5575_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 1h10 |  |  |  |
| Purpose: |  |  | Check the LFR FSW is able to change the value of SY_LFR_N_SWF_L and SY_LFR_N_SWF_P parameters upon reception of the TC_LFR_LOAD_NORMAL_PAR packet. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-431 |  |  |  |  |  |  |  |
| restrictions |  |  |  | SY_LFR_N_SWF_L ne peut etre changer. Reste à sa valeur par défaut 2048 |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | loadNorm.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Realize the steps below for each initial mode without SCIENCE_NORMAL stream, and for each final mode with SCIENCE_NORMAL stream. |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
|  | Standby mode: We try to change SY_LFR_N_SWF_L to value <> 2048 ( we try 1024). |  |  |  | Verify that TM_LFR_TC_EXE_INCONSISTENT is received with PA_RPW_BYTE_POSITION = 10 and PA_RPW_RCV_VALUE = 0. Also, we have to see by TM_LFR_PARAM_DUMP that SY_LFR_N_SWF_L is still 2048 |  |  |  |  |  |  |
| - 2 | For mode NORMAL SBM1 SBM2 Change only SY_LFR_N_SWF_P (22s) and default value for SY_LFR_N_SWF_L Stay in the mode during 1.1* SY_LFR_N_SWF_P seconds |  |  |  | Verify LFR has changed the value of SY_LFR_N_SWF_P parameters: by the TM_LFR_PARAM_DUMP, by the SCIENCE_NORMAL stream analysis. We observe one sequence of 7 packets for TM_LFR_SCIENCE_SWF_F0 around 41s after the transition mode. Same thing for TM_LFR_SCIENCE_SWF_F1 Same thing for TM_LFR_SCIENCE_SWF_F2 |  |  |  |  |  |  |
| - 3 | Change SY_LFR_N_SWF_P with default Value. default value for SY_LFR_N_SWF_L) |  |  |  | Verify LFR has changed the value of SY_LFR_N_SWF_P parameters: by the TM_LFR_PARAM_DUMP, by the SCIENCE_NORMAL stream analysis. |  |  |  |  |  |  |
| - 4 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0074_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5571_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 6mn |  |  |  |
| Purpose: |  |  | Check the LFR FSW rejects the TC_LFR_ENTER_MODE packet if the CP_LFR_ENTER_MODE_TIME parameter is greater than the current time plus 3 seconds. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-323 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | activateLfrModeOver.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | CP_LFR_ENTER_MODE_TIME = Due date in seconds (fine_time= 0) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  | Comments | Status |
|  |  |  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME=current TIME+3s. |  |  |  | This TC is validated by a TM_LFR_TC_EXE_SUCCESS. HK_LFR_MODE of TM_LFR_TC_EXE_SUCCESS is equal to the asked mode. Transition at due date is realized. |  |  |  |  |  |  |  |  |  |
|  |  |  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME= current TIME+4s. |  |  |  | Verify the LFR FSW rejects the TC_LFR_ENTER_MODE packet. and one TM_LFR_EXE_NOT_EXECUTABLE is generated. Field HK_LFR_MODE showns the current working mode . |  |  |  |  |  |  |  |  |  |
|  |  |  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME=current TIME+3s, and . with msb=1. |  |  |  | This TC is validated by a TM_LFR_TC_EXE_SUCCESS. HK_LFR_MODE of TM_LFR_TC_EXE_SUCCESS is equal to the asked mode. Transition at due date is realized. MSB bit of TIME has no-effect level. |  |  |  |  |  |  |  |  |  |
|  |  |  | For each transition modes, send TC with CP_LFR_ENTER_MODE_TIME= current TIME+4s and . with msb=1. |  |  |  | Verify the LFR FSW rejects the TC_LFR_ENTER_MODE packet. and one TM_LFR_EXE_NOT_EXECUTABLE is generated. Field HK_LFR_MODE showns the current working mode . MSB bit of TIME has no-effect level. |  |  |  |  |  |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0076_Ed1 |  |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5222_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |  |
| HW: |  | StarDundee |  |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  |  | Test Duration: |  | 5mn40 |  |  |  |
| Purpose: |  |  | Check if no SpW time code is received by the LFR FSW for a delay greater than SY_RPW_DELAY_WITHOUT_CTR, the LFR FSW indicates this by setting the MSB of the time field of each packet to 1. |  |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Time management |  |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-410; AD06 R-789 |  |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | time_msb1.py |  |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  |  | Comments | Status |
| - 1 | Launch LFR FSW, after the boot sequence, no timecode and no TC_LFR_UPDATE_TIME |  |  |  |  | in TM_LFR_HK Only one timing error is traced, HK_LFR_ TIMECODE_MISSING at this end of boot sequence So HK_LFR_LE_CNT=1, HK_LFR_LAST_ER_RID: LE_LFR_TIMEC = 42129, HK_LFR_LAST_ER_CODE: MISSING = 21, HK_LFR_LAST_ER_TIME=0x800000022f12, HK_LFR_DPU_SPW_TICK_OUT_CNT=0, HK_LFR_DPU_SPW_LAST_TIMC=0, HK_LFR_TIMECODE_ERRONEOUS=0, HK_LFR_TIMECODE_MISSING=1, HK_LFR_TIMECODE_INVALID=0, HK_LFR_TIME_TIMECODE_IT=0, HK_LFR_TIME_NOT_SYNCHRO=0, HK_LFR_TIME_TIMECODE_CTR=0 TIME of TM_LFR_HK has its MSB bit set to 1 |  |  |  |  |  |  |
| - 2 | Realize, for each mode: LFR is synchronized at t0 (TC_LFR_UPDATE_TIME + valid time-code) LFR receives neither time codes, neither CTR for a delay greater than SY_RPW_DELAY_WITHOUT_CTR. |  |  |  |  | Verify TIME field of all TMs: -coarse and fine part are consistent with the clock of the PC. -MSB is 0 before t0+SY_RPW_DELAY_WITHOUT_CTR. HK_LFR_DPU_SPW_TICK_OUT_CNT and HK_LFR_DPU_SPW_LAST_TIMC are updated but -MSB is 1 after t0+SY_RPW_DELAY_WITHOUT_CTR. in TM_LFR_HK HK_LFR_LE_CNT is incremented of 1 HK_LFR_TIME_NOT_SYNCHRO=1 |  |  |  |  |  |  |
| - 3 | Go to standby |  |  |  |  | in TM_HK_LFR 5 valid timecodes sent HK_LFR_DPU_SPW_TICK_OUT_CNT=5, HK_LFR_DPU_SPW_LAST_TIMC=5, HK_LFR_TIME_NOT_SYNCHRO=5 HK_LFR_TIMECODE_MISSING=6 (one after the boot sequence + 5) the last error is HK_LFR_LAST_ER_RID: LE_LFR_TIME = 42119, HK_LFR_LAST_ER_CODE: NOT_SYNCHRO = 25, HK_LFR_LAST_ER_TIME=0x8000003d64f3 |  |  |  |  |  |  |
| - 3 | End |  |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  |  | Assessment |  |  |
| SVS-0077_Ed1 |  |  |  |  |  | DD/MM/YYYY |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5574_Ed1 REQ-LFR-SRS-5564_Ed1 |  |  |  |  |  |
| Component: |  |  |  |  |  | Version: | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Operator: |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  |  | Test Duration: |  | 6mn |  |  |  |
| Purpose: |  |  | Check upon reception of a TC_LFR_UPDATE_INFO packet, the LFR FSW increments the HK_LFR_UPDATE_INFO_TC_CNT counter only if the packet is correct and has been accepted. Check the LFR FSW does not acknowledge the TC_LFR_UPDATE_INFO packets |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment command feedback |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-141 SSS-CP-EQS-354 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | update_info_cnt.py + + update_time_and_info_cnt_wrap.py (step3) |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  | Fully tested by REQ-LFR-SRS-5543_Ed1 (SVS-0081) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  | Expected Results |  |  |  |  | Comments | Status |
| - 1 | After boot, all rw frequencies are set to Nan Send a TC_LFR_DUMP_PAR and wait TM_LFR_PARAMETER_DUMP |  |  |  | All rw frequencies are set to Nan (all PA_LFR_RW_MASK_Fi_WORDj are set to 0xFFFFFFFF. I=[1,4] and j=[1,4] On TM_LFR_HK, HK_LFR_SC_RWi_Fj are DISABLED (=0). I=[1,4] and j=[1,4] |  |  |  |  |  |  |
| - 2 | In each mode send the sequence Send nominal TC_LFR_UPDATE_INFO |  |  |  | On TM_LFR_HK Verify the LFR FSW increments the HK_LFR_UPDATE_INFO_TC_CNT counter. |  |  |  |  |  |  |
| - 3 | Send erroneous TC_LFR_UPDATE_INFO (inconsistent parameters) CP_LFR_MODE_COPY = [5,15] CP_TDS_MODE_COPY =[6,15] CP_THR_MODE_COPY =[[4,15] |  |  |  | On TM_LFR_HK Verify the LFR FSW does not increment the HK_LFR_UPDATE_INFO_TC_CNT counter. HK_LFR_LAST_REJ_x Fields of the TM_LFR_HK not updated. |  |  |  |  |  |  |
|  | Send erroneous TC_LFR_UPDATE_INFO (inconsistent parameters) a negative rw frequency CP_RPW_SC_RW1_F1 = -1.0 send a TC_LFR_DUMP_PAR |  |  |  | Verify the LFR FSW does not increment the HK_LFR_UPDATE_INFO_TC_CNT counter. HK_LFR_LAST_REJ_x Fields of the TM_LFR_HK not updated. On TM_LFR_PARAMETER_DUMP All rw frequencies are set to Nan (all PA_LFR_RW_MASK_Fi_WORDj are set to 0xFFFFFFFF. I=[1,4] and j=[1,4] On TM_LFR_HK TM_LFR_HK, HK_LFR_SC_RWi_Fj are DISABLED (=0). I=[1,4] and j=[1,4] |  |  |  |  |  |  |
| - 4 | Send erroneous TC_LFR_UPDATE_INFO (for instance: wrong .CRC)). |  |  |  | Verify the LFR FSW does not increment the HK_LFR_UPDATE_INFO_TC_CNT counter. HK_LFR_LAST_REJ_x Fields of the TM_LFR_HK not updated. |  |  |  |  | Accurate tests in SVS-00081 |  |
| - 5 | Use update_time_and_info_cnt_wrap.py Reach the HK_LFR_UPDATE_INFO_TC_CNT maximal value. Send nominal TC_LFR_UPDATE_INFO |  |  |  | Verify the LFR FSW increments the HK_LFR_UPDATE_INFO_TC_CNT counter by wrapping around 0 (after 65535). |  |  |  |  | See SVS-0078 (step3) |  |
| - 6 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0078_Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5572_Ed1 REQ-LFR-SRS-5241_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 Stardundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 50s + 3h50 |  |  |  |
| Purpose: |  |  | Check upon reception of a TC_LFR_UPDATE_TIME packet, the LFR FSW increment s the HK_LFR_UPDATE_TIME_TC_CNT counter only if the packet is correct and has been accepted. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment command feedback |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-142 SSS-CP-FS-201 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | update_time_cnt.py + update_time_and_info_cnt_wrap.py (step3) |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  | Comments | Status |
| - 1 | In each mode: Send nominal TC_LFR_UPDATE_TIME without time code. |  |  |  |  | Verify the LFR FSW increments the HK_LFR_UPDATE_TIME_TC_CNT counter. |  |  |  |  |  |
| - 2 | In each mode: Send erroneous TC_LFR_UPDATE_TIME (for instance: wrong .CRC), with nominal time codes. |  |  |  |  | Verify the LFR FSW does not increment the HK_LFR_UPDATE_TIME_TC_CNT counter. HK_LFR_LAST_REJ_x Fields of the TM_LFR_HK not updated. |  |  |  |  |  |
| - 3 | Use update_time_and_info_cnt_wrap.py Reach the HK_LFR_UPDATE_TIME_TC_CNT maximal value Send nominal TC_LFR_UPDATE_TIME ( no time code) Reach the HK_LFR_UPDATE_INFO_TC_CNT maximal value |  |  |  |  | Verify the LFR FSW increments the HK_LFR_UPDATE_TIME_TC_CNT counter by wrapping around 0, and HK_LFR_UPDATE_INFO_TC_CNT counter by wrapping around 0 |  |  |  | 0x10004 TC ~3h50 REQ-LFR-SRS-5241_Ed1 |  |
| - 4 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  | Date |  |  |  | Assessment |  |  |
| SVS-0079 _Ed1 |  |  |  |  | DD/MM/YYYY |  |  |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  | RB Tested requirements |  |  |  |  |  |  |
| N/A |  |  |  |  | REQ-LFR-SRS-5568_Ed1 |  |  |  |  |  |  |
| Component: |  |  |  |  | Version: |  | V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |  |
| SW: |  | 3.2.0.24 |  |  |  |  |  |  |  |  |  |
| HW: |  | 1.1.91 StarDundee |  |  |  |  |  |  |  |  |  |
| Start time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |  |  |  |
| End Time: |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: |  |  | 49mn |  |  |  |
| Purpose: |  |  | Check, when it performs a NORMAL / BURST mode transition, the LFR FSW stops the acquisition processing which is in progress if any. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR NORMAL / BURST mode transition |  |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-525 |  |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | Normal2Burst _swf_22s_asm_4s.py + extract.py |  |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  |  | Comments | Status |
| - 1 | Realize the below scenario: t1: TC_LFR_ENTER_MODE immediat to NORMAL. Ensure LFR processes acquisitions. (Wait SWF or CWF products) t2: TC_LFR_ENTER_MODE immediat to BURST as short as possible. t3: TC_LFR_ENTER_MODE immediat to NORMAL. Ensure LFR processes acquisitions. |  |  |  |  | Using the TIME_ACQUISITION of TMs emetted after t3, verify the LFR FSW has stopped the previous acquisition processing |  |  |  |  |  |
| - 3 | End |  |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0081_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5543_Ed3 REQ-LFR-SRS-5564_Ed3 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V2 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 35s |  |  |  |
| Purpose: |  | The equipment flight software shall acknowledge all the commands it receives from the DPU expected the both following commands for which no TM_LFR_TC_EXE_yyy packets shall be generated: TC_LFR_UPDATE_INFO, TC_LFR_UPDATE_TIME. - The parameters HK_LFR_EXE_TC_CNT to HK_LFR_LAST_REJ_TC_TIME shall not be updated upon reception of TC_LFR_UPDATE_INFO and TC_LFR_UPDATE_TIME. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Command Feedback |  |  |  |  |  |  |
| dependencies |  |  |  | AD05, SSS-CP-EQS-140 SSS-CP-EQS-354 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  | Acceptance of the command fails. |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | loop_update.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | -criterias of acceptance failed are listed in REQ-LFR-SRS-5204 -CRC should not add extra inconsistency; so, it should be recalculated |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | For each mode send a TC_LFR_UPDATE_INFO nominal. |  |  |  |  | Verify: -FSW does not reply TM_LFR_TC_EXE_xxx. -The parameters HK_LFR_EXE_TC_CNT to HK_ LFR _LAST_REJ_TC_TIME are not updated. HK_LFR_UPDATE_INFO_TC_CNT is incremented |  |  |  |  |
| - 2 | TC_LFR_UPDATE_INFO with error detected during the acceptance stage verifications. ( bad CRC, bad PID, bad CAT, bad type, bad SUB-TYPE, bad length packet, bad source_id , bad length vs type/sub-type). |  |  |  |  | Verify: -FSW does not reply TM_LFR_TC_EXE_xxx. -The parameters HK_LFR_EXE_TC_CNT to HK_ LFR _LAST_REJ_TC_TIME are not updated. Except when type et sub-type are bad, LFR FSW doen’t identify a TC_UPDATE_INFO and in this case, LFR FSW sends a TM_LFR_TC_EXE_CORRUPTED. |  |  |  |  |
| - 3 | Send a TC_LFR_UPDATE_TIME nominal |  |  |  |  | Verify: -FSW does not reply TM_LFR_TC_EXE_xxx. -The parameters HK_LFR_EXE_TC_CNT to HK_ LFR _LAST_REJ_TC_TIME are not updated. HK_LFR_UPDATE_TIME_TC_CNT is incremented |  |  |  |  |
| - 4 | TC_LFR_UPDATE_TIME with error detected during the acceptance stage verifications ( bad CRC, bad PID, bad CAT, bad type, bad SUB-TYPE, bad length packet, bad source_id , bad length vs type/sub-type). |  |  |  |  | Verify: -FSW does not reply TM_LFR_TC_EXE_xxx. -The parameters HK_LFR_EXE_TC_CNT to HK_ LFR _LAST_REJ_TC_TIME are not updated. Except when type et sub-type are bad, LFR FSW doen’t identify a TC_UPDATE_TIME and in this case, LFR FSW sends a TM_LFR_TC_EXE_CORRUPTED. |  |  |  |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0082_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5580_Ed1 REQ-LFR-SRS-5569_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V3 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 16s |  |  |  |
| Purpose: |  | The LFR flight software shall report in its periodic HK packet (TM_LFR_HK) the availability of the electric field components sampled at f3. VHDL starts acquisition immediately after power-on and hardware init sequence. After flight software boot sequence and as soon as the filters are nominally working (stable) : electrical values written by VHDL in dedicated registers are read by LFR Flight software and following actions are performed: - HK_LFR_SC_POTENTIAL_FLAG is set to 1 - HK_LFR_SC_V_F3, HK_LFR_SC_E1_F3, HK_LFR_SC_E2_F3 are filled This implies that even in STANDBY mode, flag is set to 1 and fields are filled with valid values. Check each second, the LFR FSW puts in its TM_LFR_HK packet a mean of the 16 last values of the electric field components sampled at f3: V_f3, E1_f3, E2_f3. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR data for S/C potential computation |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-533 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | survey_potential_computation.py + CTC-800 |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | Never observed HK_LFR_SC_POTENTIAL_FLAG = 0 because, this case is during some cycles in the boot sequence. Some plots of results can be seen in Calibration report (RPW-MEB-LFR-RPT-00168-LPP-01-02-LFR_Calibration_report) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | Launch LFR FSW For each mode Read the 3 VHDL registers F3_V,F3_E1,F3_E2 wait 10s TM_LFR_HK packets. |  |  |  |  | After the boot sequence, LFR is in STANDBY, Verify the first one TM_LFR_HK HK_LFR_SC_POTENTIAL_FLAG is set to 1 HK_LFR_SC_V_F3,HK_LFR_SC_E1_F3, HK_LFR_SC_E2_F3 are filled with the registers values . Registers addresses are 0x80000fe4,0x 80000fe8, 0x80000fec Verify each second, the LFR FSW puts in its TM_LFR_HK packet a mean of the 16 last values of the electric field components sampled at f3: V_f3, E1_f3,E2_f2. |  |  |  |  |
| - 2 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0083_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5581_Ed1 REQ-LFR-SRS-5582_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V3 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 35s |  |  |  |
| Purpose: |  | Upon reception of a TC_LFR_LOAD_FBINS_MASK, the LFR flight software shall be able to update the internal mask used for avoiding some frequency bins in the computation of the basic parameters: - 1 mask f0 , 128 bits = 16 bytes - 1 mask f1 , 16 bytes - 1 mask f2 , 16 bytes The LFR flight software shall be able to dump in the TM_LFR_PARAMETER_DUMP packet the internal mask of frequency bins. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR frequency bins internal mask |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-527 SSS-CP-EQS-528 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | load_fbins_with_dump.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | Covered by Calbration tests CTC-610 to CTC-612 |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | Launch LFR FSW For each mode Send a TC_LFR_LOAD_FBINS_MASK with default values Send a TC_LFR_DUMP_PAR |  |  |  |  | TM_LFR_PARAMETER_DUMP contains FBINS_Fi_WORD1=0xffffffff FBINS_Fi_WORD2=0xffffffff FBINS_Fi_WORD3=0xffffffff FBINS_Fi_WORD4=0xffffffff Fi = F0,F1,F2 The default values are set to 1 |  |  | No mode constraint |  |
| - 2 | For each mode Send a TC_LFR_LOAD_FBINS_MASK with all bits set to 0 Send a TC_LFR_DUMP_PAR |  |  |  |  | TM_LFR_PARAMETER_DUMP contains FBINS_Fi_WORD1=0x FBINS_Fi_WORD2=0Xffffffff FBINS_Fi_WORD3=0Xffffffff FBINS_Fi_WORD4=0Xffffffff Fi = F0,F1,F2 |  |  | No mode constraint |  |
| - 3 | Use 3 calibration tests CTC-610,CTC-611 and CTC-612 to validate the effect of the internal mask on science data (BP2 products). For each frequency internal mask = 0 then Internal mask= 1 |  |  |  |  |  |  |  |  |  |
| - 4 | acceptance stage tests see SVS-0003, SVS-0005, SVS-0007 SVS-0008, SVS-0009 |  |  |  |  | no inconsistent parameter for this TC |  |  |  |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  | Assessment |  |  |
| SVS-0086_Ed1 |  |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5583_Ed1 REQ-LFR-SRS-5584_Ed1 |  |  |  |  |
| Component: |  |  |  |  |  | Version: V3 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Duration: 20mn |  |  |  |  |
| Purpose: |  | Upon reception of several TC_LFR_LOAD_KCOEFFICIENTS packets, the LFR flight software shall be able to update the inter-calibration factors (k-coefficients) used for computing the poynting flux and phase velocity estimators: - 32 coefficients (coded on 4 bytes) for 36 frequencies. Upon reception of a TC_LFR_DUMP_KCOEFFICIENTS packet, the LFR flight software shall be able to dump in two TM_LFR_KCOEFFICENTS_DUMP packets the inter-calibration factors (k-coefficients). |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR inter-calibration factors |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-529 SSS-CP-EQS-531 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | load_kcoeff_with_dump.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | Covered by Calibration tests CTC-700 |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  | Comments | Status |
| - 1 | Launch LFR FSW Send a TC_LFR_DUMP_KCOEFFICIENTS |  |  |  |  |  | 2 TM_LFR_KCOEFFICIENTS_DUMP expected First TM_LFR_KCOEFFICIENTS_DUMP contains from SY_LFR_KCOEFF_FREQUENCY = 0 to SY_LFR_KCOEFF_FREQUENCY = 29 for each one, 32 KCOEFFICIENTS with default values = 1 field SY_LFR_KCOEFF_i (1 to 32) Second TM_LFR_KCOEFFICIENTS_DUMP contains from SY_LFR_KCOEFF_FREQUENCY =30 to SY_LFR_KCOEFF_FREQUENCY = 35 for each one, 32 KCOEFFICIENTS with default values = 1 field SY_LFR_KCOEFF_i (1 to 32) |  | No mode constraint |  |
| - 2 | For each mode Send a TC_LFR_LOAD_KCOEFFICIENTS with default values KCOEFF_FREQ = 0 Send a TC_LFR_DUMP_KCOEFFICIENTS |  |  |  |  |  | 2 TM_LFR_KCOEFFICIENTS_DUMP expected First TM_LFR_KCOEFFICIENTS_DUMP contains from SY_LFR_KCOEFF_FREQUENCY = 0 to SY_LFR_KCOEFF_FREQUENCY = 29 for each one, 32 KCOEFFICIENTS with default values = 1 field SY_LFR_KCOEFF_i (1 to 32) Second TM_LFR_KCOEFFICIENTS_DUMP contains from SY_LFR_KCOEFF_FREQUENCY =30 to SY_LFR_KCOEFF_FREQUENCY = 35 for each one, 32 KCOEFFICIENTS with default values = 1 field SY_LFR_KCOEFF_i (1 to 32) |  | No mode constraint |  |
| - 3 | For each mode for each freq [1,35] Send a TC_LFR_LOAD_KCOEFFICIENTS with 19 Kcoeffs changed with representative values. Send a TC_LFR_DUMP_KCOEFFICIENTS |  |  |  |  |  | 2 TM_LFR_KCOEFFICIENTS_PAR , only the 19 Kcoefficients of the tested frequency are changed. At this test end, 19 kcoefficients of all frequencies are changed. |  | No mode constraint |  |
| - 4 | Use one calibration test CTC-710 to validate the effect inter-calibration factors on science products. For each frequency 32 factors changed |  |  |  |  |  |  |  |  |  |
| - 5 | to acceptance stage see SVS-0003, SVS-0005, SVS-0007 SVS-0008, SVS-0009 |  |  |  |  |  | KCOEFF_FREQ is the only parameter of this TC. Interval definition is [0,35]. |  |  |  |
| - 6 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  | Assessment |  |  |
| SVS-0088_Ed1 |  |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5590_Ed1 |  |  |  |  |
| Component: |  |  |  |  |  | Version: V2 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Duration: 1h30 |  |  |  |  |
| Purpose: |  | Upon reception of a TC_LFR_ENTER_MODE(SBM1) / TC_LFR_ENTER_MODE(SBM2) / TC_LFR_ENTER_MODE(NORMAL), the equipment flight software shall not re-initialize the NORMAL data flow if this one was already active.. |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-326 |  |  |  |  |  |  |
| Restrictions |  |  |  | For this test SWF period is set to 22s and ASM period to 4s |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  | LFR_packet_decom-linux-x86_64 (C++ LPP decom program) |  |  |  |  |  |  |
| test script |  |  |  | test_modes-noTimegen_allTransitions.py |  |  |  |  |  |  |
| output data |  |  |  | All TM are logged in log_data file and All dataflows are in record_data file (can be decommuted with LFR_packet_decom-linux-x86_64) |  |  |  |  |  |  |
| notes |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  | Comments | Status |
| - 1 | LFR goes from STANDBY mode to NORMAL during 600s |  |  |  |  |  | We should see all products of NORMAL dataflow starting with expected periodicity. |  |  |  |
| - 2 | LFR goes from NORMAL mode to SBM1 during 900s |  |  |  |  |  | We should see that waveform products of NORMAL dataflow periodicities are unchanged since step 1 e.g. not affected by mode transition. |  |  |  |
| - 3 | LFR goes from SBM1 mode to SBM2 during 900s |  |  |  |  |  | We should see that waveform products of NORMAL dataflow periodicities are unchanged since step 1 e.g. not affected by mode transition. |  |  |  |
| - 4 | LFR goes from SBM2 mode to NORMAL during 600s |  |  |  |  |  | We should see that waveform products of NORMAL dataflow periodicities are unchanged since step 1 e.g. not affected by mode transition. |  |  |  |
| - 5 | LFR goes from NORMAL mode to BURST during 600s |  |  |  |  |  | We should observe the termination of NORMAL dataflow started at step 1 e.g. no more NORMAL products. |  |  |  |
| - 6 | LFR goes from BURST mode to NORMAL during 600s |  |  |  |  |  | We should see all products of NORMAL dataflow starting with expected periodicity e.g. not the continuation of NORMAL dataflow stopped at step 5. |  |  |  |
| - 7 | LFR goes from NORMAL mode to SBM2 during 900s |  |  |  |  |  | We should see that waveform products of NORMAL dataflow periodicities are unchanged e.g. not affected by mode transition. |  |  |  |
| - 8 | LFR goes from SBM2 mode to STANDBY |  |  |  |  |  | We should observe termination of all dataflows. |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  | Date |  | Assessment |  |  |
| SVS-0089_Ed1 |  |  |  |  |  | DD/MM/YYYY |  | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  | RB Tested requirements |  |  |  |  |
| N/A |  |  |  |  |  | REQ-LFR-SRS-5591_Ed1 REQ-LFR-SRS-5501_Ed1 |  |  |  |  |
| Component: |  |  |  |  |  | Version: V3 |  |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Operator: |  |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm | Test Duration: 50mn |  |  |  |  |
| Purpose: |  | The LFR FSW shall stop to produce science packets as soon as they have sent the acknowledgment packet related to the mode transition to STANDBY. When the LFR FSW enters in the standby mode, it shall be stop the acquisitions if they are active. |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment mode management |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-327 / SSS-CP-EQS-240 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | transition_to_standby.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | Use ASM=4s for quick test |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  |  | Expected Results |  | Comments | Status |
| - 1 | Launch LFR FSW, mode is standby. Send a TC_LOAD_NORMAL_PAR with SY_N_ASM_P = 4s Reach Normal mode Send a TC_LFR_DUMP_PAR Wait t_n_aux Reach Standby and wait t_n_aux t_n_aux=360.s |  |  |  |  |  | After transition NORMAL -> STANDBY 17:21:21.719005, /!\Timeout=360.0s: ['TM_LFR_SCIENCE_NORMAL_SWF_F0', 'TM_LFR_SCIENCE_NORMAL_SWF_F1', 'TM_LFR_SCIENCE_NORMAL_SWF_F2', 'TM_LFR_SCIENCE_NORMAL_CWF_F3' 'TM_LFR_SCIENCE_NORMAL_ASM_F0', 'TM_LFR_SCIENCE_NORMAL_ASM_F1', 'TM_LFR_SCIENCE_NORMAL_ASM_F2', 'TM_LFR_SCIENCE_NORMAL_BP1_F0', 'TM_LFR_SCIENCE_NORMAL_BP1_F1', 'TM_LFR_SCIENCE_NORMAL_BP1_F2', 'TM_LFR_SCIENCE_NORMAL_BP2_F0', 'TM_LFR_SCIENCE_NORMAL_BP2_F1', 'TM_LFR_SCIENCE_NORMAL_BP2_F2'] not seen |  |  |  |
| - 2 | Reach BURST mode Send a TC_LFR_DUMP_PAR Wait t_b_aux Reach Standby and wait t_b_aux t_b_aux=12.6s |  |  |  |  |  | After transition BURST -> STANDBY 17:21:54.839855, /!\Timeout=12.6s: ['TM_LFR_SCIENCE_BURST_CWF_F2', 'TM_LFR_SCIENCE_BURST_BP1_F0', 'TM_LFR_SCIENCE_BURST_BP2_F0', 'TM_LFR_SCIENCE_BURST_BP1_F1', 'TM_LFR_SCIENCE_BURST_BP2_F1'] not seen. |  |  |  |
| - 3 | Reach SBM1 mode Send a TC_LFR_DUMP_PAR Wait t_1_aux Reach Standby and wait t_1_aux t_1_aux=360s |  |  |  |  |  | After transition SBM1 -> STANDBY 17:38:01.793427, /!\Timeout=360.0s: ['TM_LFR_SCIENCE_NORMAL_SWF_F0', 'TM_LFR_SCIENCE_NORMAL_SWF_F1', 'TM_LFR_SCIENCE_NORMAL_SWF_F2', 'TM_LFR_SCIENCE_NORMAL_CWF_F3', 'TM_LFR_SCIENCE_NORMAL_ASM_F0', 'TM_LFR_SCIENCE_NORMAL_ASM_F1', 'TM_LFR_SCIENCE_NORMAL_ASM_F2', 'TM_LFR_SCIENCE_NORMAL_BP1_F0', 'TM_LFR_SCIENCE_NORMAL_BP1_F1', 'TM_LFR_SCIENCE_NORMAL_BP1_F2', 'TM_LFR_SCIENCE_NORMAL_BP2_F0', 'TM_LFR_SCIENCE_NORMAL_BP2_F1', 'TM_LFR_SCIENCE_NORMAL_BP2_F2', 'TM_LFR_SCIENCE_SBM1_CWF_F1', 'TM_LFR_SCIENCE_SBM1_BP1_F0', 'TM_LFR_SCIENCE_SBM1_BP2_F0'] not seen. |  |  |  |
| - 4 | Reach SBM2 mode Send a TC_LFR_DUMP_PAR Wait t_2_aux Reach Standby and wait t_2_aux t_2_aux=360s |  |  |  |  |  | After transition SBM2 -> STANDBY 17:54:21.179829, /!\Timeout=360.0s: ['TM_LFR_SCIENCE_NORMAL_SWF_F0', 'TM_LFR_SCIENCE_NORMAL_SWF_F1', 'TM_LFR_SCIENCE_NORMAL_SWF_F2', 'TM_LFR_SCIENCE_NORMAL_CWF_F3', 'TM_LFR_SCIENCE_NORMAL_ASM_F0', 'TM_LFR_SCIENCE_NORMAL_ASM_F1', 'TM_LFR_SCIENCE_NORMAL_ASM_F2', 'TM_LFR_SCIENCE_NORMAL_BP1_F0', 'TM_LFR_SCIENCE_NORMAL_BP1_F1', 'TM_LFR_SCIENCE_NORMAL_BP1_F2', 'TM_LFR_SCIENCE_NORMAL_BP2_F0', 'TM_LFR_SCIENCE_NORMAL_BP2_F1', 'TM_LFR_SCIENCE_NORMAL_BP2_F2', 'TM_LFR_SCIENCE_SBM2_CWF_F2', 'TM_LFR_SCIENCE_SBM2_BP1_F0', 'TM_LFR_SCIENCE_SBM2_BP2_F0', 'TM_LFR_SCIENCE_SBM2_BP1_F1', 'TM_LFR_SCIENCE_SBM2_BP2_F1'] not seen. |  |  |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0090_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5518_Ed2 REQ-LFR-SRS-5407_Ed2 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V2 V1 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 2h5mn |  |  |  |
| Purpose: |  | Each data packet generated by the equipment flight software shall contain the absolute time (SCET) of the first sample contained in the packet (acquisition time). The time of the other samples are deduced from the time of the first sample.  The absolute time value shall be copied twice: 1) in the time field of the TM packet data field header (PUS header) and 2) in the TM packet source data auxiliary header (data fields).  If needed, some relative timestamps can be added in the data packet to tag data blocks. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Science data acquisition and processing |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-410 SSS-IF-DPS-EQ-160 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | generate_science_all_modes.py + extract_v1.bash |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| 1 | We generated all TM_LFR_SCIENCE_* in all science modes. MSB=1 |  |  |  |  | Verify: Absolute time is copied with the same value in time field of TM data auxiliary header AND in PUS header (PA_LFR_ACQUISITION_TIME). No relative timestamps are added. |  |  | extract_v1.bash does the verification |  |
| 2 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0091_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5289_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V3 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 10s |  |  |  |
| Purpose: |  | The RPW Flight Software shall explicitly configure the data and instruction caches at startup. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Cache configuration |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-FS-610 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | test_cache_state.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  | Reset the LFR |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
|  | Run socexplorer don’t launch LFR SFW Read the instruction cache status into the dedicated register Read the data cache status into the dedicated register |  |  |  |  | instruction cache status = Disabled data cache status = Disabled |  |  |  |  |
|  | Load and run the LFR FSW Read the instruction cache status into the dedicated register Read the data cache status into the dedicated register |  |  |  |  | instruction cache status = Enabled data cache status = Enabled |  |  |  |  |
|  | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0095_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-5550_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V2 |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 7h10 |  |  |  |
| Purpose: |  | Additionally, the equipment flight software, depending on the features of the equipment, shall have one or several other commands for configuring the parameters not directly linked to the science sub-mode configuration.  Common parameters: TC_LFR_LOAD_COMMON_PAR |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | Equipment configuration management |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-210 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | send_tc_load_common_all_modes.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | Launch LFR FSW Standby mode configure normal mode Send TC_LFR_NORMAL_PAR SY_LFR_N_SWF_P=22(s) SY_LFR_N_ASM_P=4 (s) Send a TC_LFR_DUMP_PAR DEFAULT_COMMON_PAR = 0x20 SY_LFR_BW = 1 SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 = 0 For each parameter combination send a TC_LFR_COMMON_PAR send a TC_DUMP_PAR wait 10s send TC_LFR_COMMON_PAR with DEFAULT_COMMON_PAR = 0x20 send a TC_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_HK contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 No TM_LFR_SCIENCE* products |  |  | No mode constraint |  |
| - 2 | Reach Normal mode DEFAULT_COMMON_PAR = 0x20 SY_LFR_BW = 1 SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 = 0 For each parameter combination send a TC_LFR_COMMON_PAR send a TC_DUMP_PAR wait 180s send TC_LFR_COMMON_PAR with DEFAULT_COMMON_PAR = 0x20 send a TC_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_HK contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_SCIENCE_NORMAL* products contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 |  |  | No mode constraint |  |
| - 3 | Reach Burst mode DEFAULT_COMMON_PAR = 0x20 SY_LFR_BW = 1 SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 = 0 For each parameter combination send a TC_LFR_COMMON_PAR send a TC_DUMP_PAR wait 30s send TC_LFR_COMMON_PAR with DEFAULT_COMMON_PAR = 0x20 send a TC_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_HK contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_SCIENCE_BURST* products contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 de |  |  | No mode constraint |  |
| - 4 | Reach SBM1 mode DEFAULT_COMMON_PAR = 0x20 SY_LFR_BW = 1 SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 = 0 For each parameter combination send a TC_LFR_COMMON_PAR send a TC_DUMP_PAR wait 180s send TC_LFR_COMMON_PAR with DEFAULT_COMMON_PAR = 0x20 send a TC_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_HK contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_SCIENCE_NORMAL* products contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_SCIENCE_SBM1* products contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 |  |  | No mode constraint |  |
| - 5 | Reach SBM2 mode DEFAULT_COMMON_PAR = 0x20 SY_LFR_BW = 1 SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 = 0 For each parameter combination send a TC_LFR_COMMON_PAR send a TC_DUMP_PAR wait 180s send TC_LFR_COMMON_PAR with DEFAULT_COMMON_PAR = 0x20 send a TC_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_HK contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_SCIENCE_NORMAL* products contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 TM_LFR_SCIENCE_SBM2* products contains the modified parameter into the dedicated field: SY_LFR_BW, SY_LFR_SP0 , SY_LFR_SP1, SY_LFR_R0, SY_LFR_R1, SY_LFR_R2 |  |  | No mode constraint |  |
| - 6 | to acceptance stage see SVS-0003, SVS-0005, SVS-0007 SVS-0008, SVS-0009 |  |  |  |  | No inconsistent parameter because each one is coded in one bit. No constraint mode |  |  |  |  |
| - 7 | Calibration test CTC-100 to validate the effect of these 5 parameters. |  |  |  |  |  |  |  |  |  |
| - 8 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-0096_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6003_Ed1 REQ-LFR-SRS-6005_Ed1 REQ-LFR-SRS-6104_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version: V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: 1mn 50 |  |  |  |
| Purpose: |  | Upon reception of a TC_LFR_LOAD_FILTER_PAR packet, the LFR FSW should be able to change - SY_LFR_RW_DELTA_F - The 32 bit float values (SY_LFR_RWi_Kj with i=[1,4] and j=[1,4] The LFR FSW should be able to dump in the TM_LFR_PARAMETER_DUMP - SY_LFR_RW_DELTA_F - The 32 bit float values (SY_LFR_RWi_Kj with i=[1,4] and j=[1,4] The LFR FSW should be able to dump in the TM_LFR_PARAMETER_DUMP - SY_LFR_PAS_FILTER_ENABLED - SY_LFR_PAS_FILTER_MODULUS - SY_LFR_PAS_FILTER_TBAD - SY_LFR_PAS_FILTER_OFFSET - SY_LFR_PAS_FILTER_SHIFT |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR Filtering of S/C reaction wheel emission frequencies |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-753 – SSS-CP-EQS-755 – SSS-CP-EQS-756 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  |  |  |  |  |  |  |  |
| prerequisite |  |  |  |  |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | change_param_load_filter_par.py |  |  |  |  |  |  |
| output data |  |  |  |  |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | Launch LFR FSW Standby mode Send a TC_LFR_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the default values SY_LFR_PAS_FILTER_ENABLED_D = DISABLED SY_LFR_PAS_FILTER_MODULUS = 4 SY_LFR_PAS_FILTER_TBAD = 1.0 SY_LFR_PAS_FILTER_OFFSET = 0 SY_LFR_PAS_FILTER_SHIFT = 0.5 SY_LFR_SC_RW_DELTA_F = 0.045 SY_LFR_RWi_K1 = 1 with i=[1,4] SY_LFR_RWi_K2 = 8 with i=[1,4] SY_LFR_RWi_K3 = 24 with i=[1,4] SY_LFR_RWi_K4 = 48 with i=[1,4] PA_LFR_RW_MASK_Fi_WORDj = 1 (DISABLED) with i=[0,2] and j=[1,4] either 32 bits by frequency |  |  | No mode constraint |  |
| - 2 | For each mode send a TC_LOAD_FILTER_PAR with SY_LFR_PAS_FILTER_ENABLED_D = DISABLED SY_LFR_SC_RW_DELTA_F = 1.0 SY_LFR_RWi_K1 = 2 with i=[1,4] SY_LFR_RWi_K2 = 16 with i=[1,4] SY_LFR_RWi_K3 = 48 with i=[1,4] SY_LFR_RWi_K4 = 96 with i=[1,4] send a TC_LFR_DUMP_PAR send a TC_LOAD_FILTER_PAR with SY_LFR_PAS_FILTER_ENABLED_D = ENABLED and previous values send a TC_LFR_DUMP_PAR send a TC_LOAD_FILTER_PAR with SY_LFR_PAS_FILTER_ENABLED_D =DISABLED and default values of others parameters send a TC_LFR_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the modified parameter into the dedicated field: SY_LFR_SC_RW_DELTA_F SY_LFR_RWi_K1 = 1 with i=[1,4] SY_LFR_RWi_K2 = 8 with i=[1,4] SY_LFR_RWi_K3 = 24 with i=[1,4] SY_LFR_RWi_K4 = 48 with i=[1,4] These fields are not updated PA_LFR_RW_MASK_Fi_WORDj = 1 (DISABLED) with i=[0,2] and j=[1,4] either 32 bits by frequency because the SY_LFR_PAS_FILTER_ENABLED_D = DISABLED |  |  | No mode constraint |  |
| - 3 | For each mode send a TC_LOAD_FILTER_PAR with SY_LFR_PAS_FILTER_ENABLED_D = ENABLED SY_LFR_PAS_FILTER_MODULUS = 6 SY_LFR_PAS_FILTER_TBAD = 4.0 SY_LFR_PAS_FILTER_OFFSET = 4 SY_LFR_PAS_FILTER_SHIFT = 1.0 send a TC_DUMP_PAR send a TC_LOAD_FILTER_PAR SY_LFR_PAS_FILTER_ENABLED_D = DISABLED with default values for others parameters send a TC_DUMP_PAR |  |  |  |  | TM_LFR_DUMP_PAR contains the default values SY_LFR_PAS_FILTER_ENABLED_D = ENABLED SY_LFR_PAS_FILTER_MODULUS = 6 SY_LFR_PAS_FILTER_TBAD = 4.0 SY_LFR_PAS_FILTER_OFFSET = 4 SY_LFR_PAS_FILTER_SHIFT = 1.0 TM_LFR_DUMP_PAR contains the default values SY_LFR_PAS_FILTER_ENABLED_D = ENABLED SY_LFR_PAS_FILTER_MODULUS = 4 SY_LFR_PAS_FILTER_TBAD = 1.0 SY_LFR_PAS_FILTER_OFFSET = 0 SY_LFR_PAS_FILTER_SHIFT = 0.5 |  |  |  |  |
| - 4 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1100_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6000_Ed1 (NORMAL mode part) + REQ-LFR-SRS-6020_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify S/C RW filtering efficiency on Basic Parameters (BP2) computation for NORMAL mode |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR filtering of S/C reaction wheel emission frequencies |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-750 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | SVS_1100.input |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | SVS_1100.py |  |  |  |  |  |  |
| output data |  |  |  | SVS_1100.output |  |  |  |  |  |  |
| notes |  |  | Automatic analysis is provided through lfrcompliance framework : SVS_1100_analysis.ipynb (Python notebook) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - Activate output to file of the LFR binary stream - Switch to NORMAL mode |  |  |  |  |  |  |  |  |  |
| - 2 | For each of the 88 ASM bins frequencies of F0 ( [1632Hz,9984Hz] with df=96Hz) : - Disable RW filtering - Start generation of the considered frequency on E1, E2, B1, B2 and B3 (1Vpp) - Wait 60 s (to get 3 F0 BP packets) - Activate RW filtering by setting all RWx_freqs to the injected frequency value. - Wait 60s (to get 3 F0 BP packets) |  |  |  |  | - We should get 6 F0 BP packets for each frequency. - We consider to check the 2nd and the 5th because they reflect a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the field AUTO of 2nd BP packet a important count rate (> 8x105)reflecting the DISABLE state of RW filtering. - We should see in the field AUTO of 5th BP packet a low count rate (< 5x104)reflecting the ENABLE state of RW filtering. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 3 | For each of the 104 ASM bins frequencies of F1 ( [96Hz,1744Hz] with df=16Hz) : - Disable RW filtering - Start generation of the considered frequency on E1, E2, B1, B2 and B3 (1Vpp) - Wait 60 s (to get 3 F1 BP packets) - Activate RW filtering by setting all RWx_freqs to the injected frequency value. - Wait 60s (to get 3 F1 BP packets) |  |  |  |  | - We should get 6 F1 BP packets for each frequency. - We consider to check the 2nd and the 5th because they reflect a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the field AUTO of 2nd BP packet a important count rate (> 8x105)reflecting the DISABLE state of RW filtering. - We should see in the field AUTO of 5th BP packet a low count rate (< 5x104)reflecting the ENABLE state of RW filtering. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 4 | For each of the 96 ASM bins frequencies of F2 ( [7Hz,102Hz] with df=1Hz) : - Disable RW filtering - Start generation of the considered frequency on E1, E2, B1, B2 and B3 (1Vpp) - Wait 60 s (to get 3 F2 BP packets) - Activate RW filtering by setting all RWx_freqs to the injected frequency value. - Wait 60s (to get 3 F2 BP packets) |  |  |  |  | - We should get 6 F2 BP packets for each frequency. - We consider to check the 2nd and the 5th because they reflect a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the field AUTO of 2nd BP packet a important count rate (> 8x105)reflecting the DISABLE state of RW filtering. - We should see in the field AUTO of 5th BP packet a low count rate (< 5x104)reflecting the ENABLE state of RW filtering. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1110_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6000_Ed1 (SBM1 mode part) + REQ-LFR-SRS-6020_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify S/C RW filtering efficiency on Basic Parameters (BP2) computation for SBM1 mode |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR filtering of S/C reaction wheel emission frequencies |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-750 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | SVS_1110.input |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | SVS_1110.py |  |  |  |  |  |  |
| output data |  |  |  | SVS_1110.output |  |  |  |  |  |  |
| notes |  |  | In SBM1 we only have BP2@F0. Automatic analysis is provided through lfrcompliance framework : SVS_1110_analysis.ipynb (Python notebook) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - Activate output to file of the LFR binary stream - Switch to SBM1 mode |  |  |  |  |  |  |  |  |  |
| - 2 | For each of the 88 ASM bins frequencies of F0 ( [1632Hz,9984Hz] with df=96Hz) : - Disable RW filtering - Start generation of the considered frequency on E1, E2, B1, B2 and B3 (1Vpp) - Wait 3 s (to get 3 F0 BP packets from SBM dataflow) - Activate RW filtering by setting all RWx_freqs to the injected frequency value. - Wait 3s (to get 3 F0 BP packets from SBM dataflow) |  |  |  |  | - We should get 6 F0 BP packets for each frequency. - We consider to check the 2nd and the 5th because they reflect a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the field AUTO of 2nd BP packet a important count rate (> 8x105)reflecting the DISABLE state of RW filtering. - We should see in the field AUTO of 5th BP packet a low count rate (< 5x104)reflecting the ENABLE state of RW filtering. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 3 | End |  |  |  |  |  |  |  |  |  |
| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1102_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6006_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify combination of S/C RW filtering masks AND FBINS_MASKS efficiency on Basic Parameters (BP2) computation |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR filtering of S/C reaction wheel emission frequencies |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-527 and SSS-CP-EQS-750 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | SVS_1102.input |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | SVS_1102.py |  |  |  |  |  |  |
| output data |  |  |  | SVS_1102.output |  |  |  |  |  |  |
| notes |  |  | Only sample case tests (combinations) are performed. |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - Activate output to file of the LFR binary stream - Switch to NORMAL mode |  |  |  |  |  |  |  |  |  |
| - 2 | We test here 2 frequencies that are far one from another: - Start generation of a sum of sinus (8Hz and 101Hz) on E1, E2, B1, B2 and B3 (1Vpp) - Activate RW filtering by setting all RWx_freqs to 8Hz - Load a FBINS_MASK corresponding to 101Hz - Wait 60 s (to get 3 F0 BP packets) |  |  |  |  | - We should get 3 F0 BP packets.We consider to check the 2nd because it reflects a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the fields AUTO of 2nd BP packet a low count rate for bins corresponding to 8Hz and 101Hz reflecting the action of both RW and FBINS masks combined. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 3 | We test here the same frequency for RW and FBINS: - Start generation of a sinus (8Hz ) on E1, E2, B1, B2 and B3 (1Vpp) - Activate RW filtering by setting all RWx_freqs to 8Hz - Load a FBINS_MASK corresponding to 8Hz - Wait 60 s (to get 3 F0 BP packets) |  |  |  |  | - We should get 3 F0 BP packets.We consider to check the 2nd because it reflects a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the fields AUTO of 2nd BP packet a low count rate for bins corresponding to 8Hz reflecting the action of both RW and FBINS masks combined. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 4 | We test here 2 frequencies that are that are contiguous: - Start generation of a sum of sinus (8Hz and 10Hz) on E1, E2, B1, B2 and B3 (1Vpp) - Activate RW filtering by setting all RWx_freqs to 8Hz - Load a FBINS_MASK corresponding to 10Hz - Wait 60 s (to get 3 F0 BP packets) |  |  |  |  | - We should get 3 F0 BP packets.We consider to check the 2nd because it reflects a stabilized period of acquisition (not impacted by a recent change of parameters). - We should see in the fields AUTO of 2nd BP packet a low count rate for bins corresponding to 8Hz and 10Hz reflecting the action of both RW and FBINS masks combined. |  |  | Most filtered values shoud be around 0. Because of some spreading effect over the masked bandwidth, some higher values should be observed but always below 5x104 |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1103_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6001_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify correct reporting in TM_LFR_HK of the available/unavailable state for each of the 16 S/C reaction wheel conveyed in TC_LFR_UPDATE_INFO |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR filtering of S/C reaction wheel emission frequencies |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-751 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | SVS_1103.input |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | SVS_1103.py |  |  |  |  |  |  |
| output data |  |  |  | SVS_1103.output |  |  |  |  |  |  |
| notes |  |  | Automatic analysis is provided through lfrcompliance framework : SVS_1103_analysis.ipynb (Python notebook) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - Activate output to file of the LFR binary stream - Switch to NORMAL mode |  |  |  |  |  |  |  |  |  |
| - 2 | For each of the 65536 combinations possible for the 16 S/C available state : - We load a combination of 16 values (nan or 1.0) through TC_LFR_UPDATE_INFO - We wait 2s (to get an HK packet) |  |  |  |  | - We should retrieve in the HK packet received after each TC_LFR_UPDATE_INFO the same values of states that were loaded. |  |  |  |  |
| - 3 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1109_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6004_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify the good reporting of RW masks for F0, F1 and F2 in TM_LFR_PARAMETER_DUMP |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR filtering of S/C reaction wheel emission frequencies |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-754 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | SVS_1109.input |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework |  |  |  |  |  |  |
| used test programs |  |  |  |  |  |  |  |  |  |  |
| test script |  |  |  | SVS_1109.py |  |  |  |  |  |  |
| output data |  |  |  | SVS_1109.output |  |  |  |  |  |  |
| notes |  |  | Because it would take too much time to check every possible combination of frequency, only sample test case are implemented : around 1200 frequencies have been carefully chosen to cover all ASM bandwidth and covering the maximum of different intersection cases for “center area” and “polluted bandwidth”. Automatic analysis is provided through lfrcompliance framework : SVS_1109_analysis.ipynb (Python notebook) |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - Activate output to file of the LFR binary stream - Switch to NORMAL mode - Parameter sc_rw_delta_f is set to default value 0.045Hz |  |  |  |  |  |  |  |  |  |
| - 2 | - We load through several TC_LFR_UPDATE_INFO frequencies chosen all over F0, F1 and F2 proper frequencies and out of bound frequencies (>10kHz) - After each TC_LFR_UPDATE_INFO,we send a TC_LFR_DUMP_PAR to get a TM_LFR_PARAMETER_DUMP |  |  |  |  | We should see F0, F1 and F2 MASKS compliant with frequencies and delta_f sent through TC_LFR_UPDATE_INFO. |  |  |  |  |
| - 3 | - We load through several TC_LFR_UPDATE_INFO frequencies chosen to be included, intersecting or outside “center area” of ASM bins (for F0, F1 and F2) - After each TC_LFR_UPDATE_INFO,we send a TC_LFR_DUMP_PAR to get a TM_LFR_PARAMETER_DUMP |  |  |  |  | We should see F0, F1 and F2 MASKS compliant with frequencies sent through TC_LFR_UPDATE_INFO. |  |  |  |  |
| - 4 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1201_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | REQ-LFR-SRS-6101_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify that no filtering process is applied when SY_LFR_PAS_FILTER_ENABLED is set to DISBALED. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR REAL TIME FILTERING OF SWA/PAS PERTURBATIONS |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-762 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | We use only noise at LFR inputs. |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework and LFR SGSE. |  |  |  |  |  |  |
| used test programs |  |  |  | lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualStarter.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualLoadFilterPar.py |  |  |  |  |  |  |
| test script |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  | Validation is made directly by monitoring ASM products on LFR SGSE |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - ASM period is set to 4s - Switch to NORMAL mode |  |  |  |  |  |  |  |  |  |
| - 2 | All LFR PAS filtering parameters are default (No TC_LOAD_FILTER_PAR is sent) e.g. PAS filtering is disabled |  |  |  |  | We should see noise on all ASM products (F0, F1 and F2 every 4s) |  |  |  |  |
| - 3 | - To compare with behavior when PAS filtering is enabled we load through TC_LOAD_FILTER_PAR following parameters : sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 4 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0 E.g. we consider a full perturbation of 4s |  |  |  |  | We should see a total flat response with no counts (amplitude 0) on all ASM products (F0, F1 and F2 every 4s) |  |  |  |  |
| - 4 | End |  |  |  |  |  |  |  |  |  |
| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1202_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | F0 part of REQ-LFR-SRS-6100_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify PAS filtering behavior and efficiency for F0 channel |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR REAL TIME FILTERING OF SWA/PAS PERTURBATIONS |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-761 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | 4s sweep signal repeated loop from 1536 Hz to 6144 Hz. |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework and LFR SGSE. Also, we use specific synchronized start script to ensure that ASM production process starts on the first timecode of on MODULUS (we ensure that we go to NORMAL mode when current coarse time%MODULUS =0 ) AND sweep signal loop is triggered at the same time. |  |  |  |  |  |  |
| used test programs |  |  |  | lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualStarter.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualLoadFilterPar.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/SynchroStartPASandTRIGGER.py |  |  |  |  |  |  |
| test script |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  | Validation is made directly by monitoring ASM products on LFR SGSE |  |  |  |  |  |  |
| notes |  |  |  |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - ASM period is set to 4s - Switch to NORMAL mode on the beginning of a MODULUS + sweep signal triggered at the same time. |  |  |  |  |  |  |  |  |  |
| - 2 | All LFR PAS filtering parameters are default (No TC_LOAD_FILTER_PAR is sent) e.g. PAS filtering is disabled |  |  |  |  | we see all the frequencies from 1536 Hz to 6144 Hz excited on the 4s ASM@F0 : |  |  |  |  |
| - 3 | PAS filtering is enabled with default parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.5 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW - Considering the 4s sweep represents 4 time unit of a perturbation, we should see on ASM products at F0 : a 1 unit hole (TBAD) of total flat response with no counts (amplitude 0) after ½ unit of signal (SHIFT): |  |  |  |  |
| - 4 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW - Considering the 4s sweep represents 4 time unit of a perturbation, we should see on ASM products at F0 : a 1 unit hole (TBAD) of total flat response with no counts (amplitude 0) at the beginning: |  |  |  |  |
| - 5 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1 sy_lfr_pas_filter_offset = 1 sy_lfr_pas_filter_shift = 0.5 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW - Considering the 4s sweep represents 4 time unit of a perturbation, we should see on ASM products at F0 : a 1 unit hole (TBAD) of total flat response with no counts (amplitude 0) after a 1 unit OFFSET and 1/2 unit of SHIFT: |  |  |  |  |
| - 6 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 0.5 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.5 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW -Considering the 4s sweep represents 4 time unit of a perturbation, we should see on ASM products at F0 : a 1/2 unit hole (TBAD) of total flat response with no counts (amplitude 0) after 1/2 unit of SHIFT: |  |  |  |  |
| - 7 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1.0 sy_lfr_pas_filter_offset = 3 sy_lfr_pas_filter_shift = 0.5 This step is dedicated to test limit case e.g. when a perturbation is considered to be spread over 2 modulus. |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW - Considering the 4s sweep represents 4 time unit of a perturbation, we should see on ASM products at F0 : a 1 unit hole(TBAD) of total flat response with no counts (amplitude 0) but this hole should be splitted like 0.5 unit at beginning and 0.5 unit at the end because of a 3 unit OFFSET and 0.5 unit SHIFT : |  |  |  |  |
| - 8 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1203_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | F2 part of REQ-LFR-SRS-6100_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify PAS filtering behavior and efficiency for F2 channel. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR REAL TIME FILTERING OF SWA/PAS PERTURBATIONS |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-761 |  |  |  |  |  |  |
| restrictions |  |  |  | For F2 channel, only 4 instant spectral matrices (e.g. 4 FFT) are used to compute a 4s ASM. |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | 4s custom signal with 0.5s of 16Hz at the end of 1st second, 0.5s of 16Hz and 2.5s of 100hz : PAS_perturbation_0.5s-0Hz_1s-16Hz_2.5s-100Hz.csv @2048Hz sampling rate: |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework and LFR SGSE. Also, we use specific synchronized start script to ensure that ASM production process starts on the first timecode of on MODULUS (we ensure that we go to NORMAL mode when current coarse time%MODULUS =0 ) AND sweep signal loop is triggered at the same time. |  |  |  |  |  |  |
| used test programs |  |  |  | lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualStarter.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualLoadFilterPar.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/SynchroStartPASandTRIGGER.py |  |  |  |  |  |  |
| test script |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  | Validation is made directly by monitoring ASM products on LFR SGSE |  |  |  |  |  |  |
| notes |  |  | Wave form generation configuration made with Digilent WaveForms2015 GUI |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - ASM period is set to 4s - Switch to NORMAL mode on the beginning of a MODULUS + custom signal triggered at the same time. |  |  |  |  |  |  |  |  |  |
| - 2 | All LFR PAS filtering parameters are default (No TC_LOAD_FILTER_PAR is sent) e.g. PAS filtering is disabled |  |  |  |  | We should see the 2 peaks at 16 Hz and 100 Hz on the 4s ASM@F2 with expected amplitudes e.g because of different signals length and integration periods : 100 Hz peak should be around 3 times stronger than 16Hz peak. |  |  |  |  |
| - 3 | PAS filtering is enabled with default parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.5 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW -We should see the filtering of the 16Hz peak AND an attenuation of the 100Hz Peak (~10%). This is because the 16Hz perturbation affects the 2 first SM so we should drop a part of the 100Hz signal. |  |  |  |  |
| - 4 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 0.49 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.5 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted bu LFR FSW - We should see a part of the 16Hz peak perturbation (amplitude should be ~60% lower than unfiltered conf) due to the fact that TBAD does not cover all the perturbation (16Hz is emitted in the [1s, 1.5s] range): |  |  |  |  |
| - 5 | End |  |  |  |  |  |  |  |  |  |

| TEST CASE |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test procedure identifier |  |  |  |  |  |  | Date | Assessment |  |  |
| SVS-1204_Ed1 |  |  |  |  |  |  | DD/MM/YYYY | NT / NA / NOK / POK / OK |  |  |
| TS Tested requirements |  |  |  |  |  |  | RB Tested requirements |  |  |  |
| N/A |  |  |  |  |  |  | F1 part of REQ-LFR-SRS-6100_Ed1 |  |  |  |
| Component: |  |  |  |  |  |  | Version:V3++ |  |  |  |
| Involved subsystems |  |  |  |  |  |  |  |  |  |  |
| SW: |  |  |  |  | 3.2.0.24 |  |  |  |  |  |
| HW: |  |  |  |  | 1.1.91 + StarDundee |  |  |  |  |  |
| Start time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Operator: |  |  |  |
| End Time: |  |  |  |  | (DD/MM/YYYY) hh:mm |  | Test Duration: N/A |  |  |  |
| Purpose: |  | - Verify PAS filtering behavior and efficiency for F1 channel. |  |  |  |  |  |  |  |  |
| General remarks about the test |  |  |  |  |  |  |  |  |  |  |
| Requirement Title |  |  |  | LFR REAL TIME FILTERING OF SWA/PAS PERTURBATIONS |  |  |  |  |  |  |
| dependencies |  |  |  | SSS-CP-EQS-761 |  |  |  |  |  |  |
| restrictions |  |  |  |  |  |  |  |  |  |  |
| means |  |  |  |  |  |  |  |  |  |  |
| input data |  |  |  | 4s custom sweep signal repeated loop with 1s of 96Hz then 1s of 144Hz then 1s of 192Hz then 1s of 240Hz. PAS_perturbation_1s-96-144-192-240.csv @2048Hz sampling rate: |  |  |  |  |  |  |
| prerequisite |  |  |  | This test uses lfrcompliance Python notebooks framework and LFR SGSE. Also, we use specific synchronized start script to ensure that ASM production process starts on the first timecode of on MODULUS (we ensure that we go to NORMAL mode when current coarse time%MODULUS =0 ) AND sweep signal loop is triggered at the same time. |  |  |  |  |  |  |
| used test programs |  |  |  | lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualStarter.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/ManualLoadFilterPar.py lfrcompliance/test_engine_R3pp/ManualTestsToolkit/SynchroStartPASandTRIGGER.py |  |  |  |  |  |  |
| test script |  |  |  |  |  |  |  |  |  |  |
| output data |  |  |  | Validation is made directly by monitoring ASM products on LFR SGSE |  |  |  |  |  |  |
| notes |  |  | Wave form generation configuration made with Digilent WaveForms2015 GUI |  |  |  |  |  |  |  |
| Procedure steps |  |  |  |  |  |  |  |  |  |  |
| Step | Actions |  |  |  |  | Expected Results |  |  | Comments | Status |
| - 1 | - Boot LFR - ASM period is set to 4s - Switch to NORMAL mode on the beginning of a MODULUS + custom signal triggered at the same time. |  |  |  |  |  |  |  |  |  |
| - 2 | All LFR PAS filtering parameters are default (No TC_LOAD_FILTER_PAR is sent) e.g. PAS filtering is disabled |  |  |  |  | We should see the 4 peaks of injected frequencies : 96 Hz, 144Hz, 192Hz and 240Hz on the 4s ASM@F1 with approximately the same amplitude (1.2x10^6 counts): |  |  |  |  |
| - 3 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1.0 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.0 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted by LFR FSW -We should see the filtering of the freq injected during the first second e.g. 96Hz peak should be filtered : |  |  |  |  |
| - 4 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1.0 sy_lfr_pas_filter_offset = 1 sy_lfr_pas_filter_shift = 0.0 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted by LFR FSW -We should see the filtering of the freq injected during the 2nd second e.g. 144Hz peak should be filtered : |  |  |  |  |
| - 5 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1.0 sy_lfr_pas_filter_offset = 2 sy_lfr_pas_filter_shift = 0.0 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted by LFR FSW -We should see the filtering of the freq injected during the 3rd second e.g. 192Hz peak should be filtered : |  |  |  |  |
| - 6 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1.0 sy_lfr_pas_filter_offset = 3 sy_lfr_pas_filter_shift = 0.0 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted by LFR FSW -We should see the filtering of the freq injected during the 4th second e.g. 240Hz peak should be filtered : |  |  |  |  |
| - 7 | PAS filtering is enabled with default parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 1.0 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.5 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted by LFR FSW -We should see the filtering of half the freq injected during the 1st second and half the freq injected during the 2nd second e.g. amplitudes of 96Hz and 144Hz peaks should be lowered by ~50%: |  |  |  |  |
| - 8 | PAS filtering is enabled with following parameters loaded through TC_LOAD_FILTER_PAR: sy_lfr_pas_filter_enabled = 1 sy_lfr_pas_filter_modulus = 4 sy_lfr_pas_filter_tbad = 3.0 sy_lfr_pas_filter_offset = 0 sy_lfr_pas_filter_shift = 0.0 |  |  |  |  | - TC_LOAD_FILTER_PAR is accepted and acquitted : TM_LFR_TC_EXE_SUCCESS is emitted by LFR FSW -We should see the filtering of the freqs injected during the 3 first seconds : 96Hz, 144Hz and 192Hz: |  |  |  |  |
| - 9 | End |  |  |  |  |  |  |  |  |  |

# Validation test platform requirements

NB 1 : Since January 2015, validation test platform has been deeply upgraded : LPPMON has been replaced by SocExplorer. Also, RMAP Plugin has been split into LFRControlPlugin and SpwPlugin. Part of the following steps are impacted by those changes. Please report to SGSE user manual for up to date procedures ([RD07]).

NB 2 : A system clock simulator ([RD08]) has been developed to emulate the Central Time Reference behavior (a spacewire command packet coupled to a spacewire timecode sent to LFR analyzer). This clock simulator is used in all test cases related to time management.

This section describes the startup of the target.

The PC driving the SpW board is supposed to nominal state. Note lppmon soft is not launched.

The board powered on, and its switch is initially off. See figure below:

Led

The "SpaceWire-USB Brick" is disconnected from the PC.

Preliminary steps to run without going too fast:

- Ensure USB "SpaceWire-USB Brick" is connected to the Board via «Link 1”.

- Ensure RS232 link of the board is connected to the PC.

- Connect USB "SpaceWire-USB Brick" to the PC. Ensure (green or red) LEDs are lit (the S/N:32111332 brick may be used). Connect the brick to the board (Link 1)

- Switch on the board.

- Initialize by reset (possibly several times) the board. Note this reset unload any FSW out of the LEON.

- Use LppMon to open the bridge.

Link 1nk 1

Link 2

1nk 1

2nk 1

3nk 1

4nk 1

- Usb is not connected

- Usb connected; strart up in progress

- Start up completed.

- Bridge (link 1) opened; brick ready.

Dup

Dup

Dup

Dup

Dup

Dup

Dup

Dup

Dup

- Power off

- Begin of start up

- Start up in progress (10s)

- Start up completed

- (Reset pushed)

- Grmon launched

- fsw load in progress

- Soft loaded

- Run of loaded fsw (bridge opened)

See below to begin the exchanges with the target. There are two independent ways: by commands (load FSW, search for information about HW …), or by a dedicated HMI.

1 LppMon HM:

Launch LppMon with associated icon:

In lppmon/LPPMON Toolbar/Plug-in Manager, plug-in are loadable by drag/move:

Instantiate the root plugging “RMAPPlugin0” (in “LPPMON Toolbar/Plugin Manager/Loaded plugins”).

In “RMAP and SPW Communication/connection” tab, select Bridge selection=”Star Dundee”

In “LPPMON Toolbar/Plugin Manager”, instantiate the useful child plug-in: GenericRWplugin (Generic Driver for the Memory editor), MemControler (Memctrlr Driver for the Memory Check) …

Activate SpW exchanges with loaded plug-in: in RMAP and SPW Communication/connection/General parameters, click on the “Open selected bridge” button (Plug-ins are activated after the “Open->Close” selected bridge transition).

Note, in case of randomly non-synchronization, that box appears. Then, re-launch the brick until this problem is solved: Reset the brick, wait a few seconds, click “Retry”.

The final result is:

Ensure the LED on brick is green for the link used.

Ensure nominal logs are:

Instantiated objects are accessible by the LppMon Python console. List them with dir() command:

py> dir()

['BUTTON_rmapCloseCommunication', 'BUTTON_rmapOpenCommunication', 'BUTTON_selectGRESB', 'BUTTON_selectStarDundee', 'GRESB_Bridge', 'GenericRWplugin0', 'Pylppmon', 'Qt', 'QtCore', 'QtGui', 'QtNetwork', 'QtOpenGL', 'QtSql', 'QtSvg', 'QtUiTools', 'QtWebKit', 'QtXml', 'RMAPPlugin0', '__builtins__', '__doc__', '__name__', '__package__', 'lppmonEngine', 'private', 'proxy']

Notice to type multi-line commands, use <SHIFT>+<ENTER> keys (ensure the Python indentation).

2 LFR FSW

In case of randomly non-synchronization, initialize by reset (possibly several times) the board. Note this reset unload any FSW out of the LEON.

Type grmon-eval, or grmon-eval -u, (use the auto completion features) in a console.

Note the port /dev/ttyS0 is used for displays.

Moreover, if the serial port uses a USB port, type “grmon-eval -uart /dev/ttyUSB0 –u”

Then, these instructions may be used: load (only if serial link is connected), run, help, info sys, exit. Example for a console session:

[saule@pc-alison ~]$ grmon-eval

GRMON LEON debug monitor v1.1.56 evaluation version

Copyright (C) 2004-2011 Aeroflex Gaisler - all rights reserved.

For latest updates, go to http://www.gaisler.com/

Comments or bug-reports to support@gaisler.com

This evaluation version will expire on 21/9/2013

using port /dev/ttyS0 @ 115200 baud

Device ID: : 0x509

GRLIB build version: 4098

initialising ................

detected frequency:  30 MHz

Component                            Vendor

LEON3 SPARC V8 Processor             Gaisler Research

Unknown device                       Gaisler Research

AHB Debug UART                       Gaisler Research

SVGA Controller                      Gaisler Research

DDR2 Controller                      Gaisler Research

AHB/APB Bridge                       Gaisler Research

LEON3 Debug Support Unit             Gaisler Research

LEON2 Memory Controller              European Space Agency

System ACE I/F Controller            Gaisler Research

Generic APB UART                     Gaisler Research

Multi-processor Interrupt Ctrl       Gaisler Research

Modular Timer Unit                   Gaisler Research

PS/2 interface                       Gaisler Research

PS/2 interface                       Gaisler Research

General purpose I/O port             Gaisler Research

AHB status register                  Gaisler Research

Use command 'info sys' to print a detailed report of attached cores

grlib> info sys

00.01:003   Gaisler Research  LEON3 SPARC V8 Processor (ver 0x0)

ahb master 0

01.01:029   Gaisler Research  Unknown device (ver 0x0)

ahb master 1, irq 10

apb: 80000900 - 80000a00

02.01:007   Gaisler Research  AHB Debug UART (ver 0x0)

ahb master 2

apb: 80000700 - 80000800

baud rate 115200, ahb frequency 30.00

03.01:063   Gaisler Research  SVGA Controller (ver 0x0)

ahb master 3

apb: 80000600 - 80000700

DCOM communication error, retrying ...

00.01:02e   Gaisler Research  DDR2 Controller (ver 0x0)

ahb: 40000000 - 60000000

ahb: fff00100 - fff00200

64-bit DDR2 : 1 * 256 Mbyte @ 0x40000000, 4 internal banks

190 MHz, col 10, ref 7.8 us, trfc 131 ns

01.01:006   Gaisler Research  AHB/APB Bridge (ver 0x0)

ahb: 80000000 - 80100000

02.01:004   Gaisler Research  LEON3 Debug Support Unit (ver 0x1)

ahb: 90000000 - a0000000

AHB trace 128 lines, 32-bit bus, stack pointer 0x4ffffff0

CPU#0 win 8, hwbp 2, itrace 128, srmmu, lddel 1

icache 2 * 8 kbyte, 32 byte/line lru

dcache 1 * 8 kbyte, 16 byte/line

03.04:00f   European Space Agency  LEON2 Memory Controller (ver 0x1)

ahb: 00000000 - 20000000

ahb: 20000000 - 40000000

ahb: c0000000 - c2000000

apb: 80000000 - 80000100

16-bit prom @ 0x00000000

04.01:067   Gaisler Research  System ACE I/F Controller (ver 0x0)

irq 13

ahb: fff00200 - fff00300

01.01:00c   Gaisler Research  Generic APB UART (ver 0x1)

irq 2

apb: 80000100 - 80000200

DCOM communication error, retrying ...

baud rate 3750000

02.01:00d   Gaisler Research  Multi-processor Interrupt Ctrl (ver 0x3)

apb: 80000200 - 80000300

03.01:011   Gaisler Research  Modular Timer Unit (ver 0x0)

irq 8

apb: 80000300 - 80000400

8-bit scaler, 2 * 32-bit timers, divisor 30

04.01:060   Gaisler Research  PS/2 interface (ver 0x2)

irq 4

apb: 80000400 - 80000500

05.01:060   Gaisler Research  PS/2 interface (ver 0x2)

irq 5

apb: 80000500 - 80000600

08.01:01a   Gaisler Research  General purpose I/O port (ver 0x0)

apb: 80000800 - 80000900

0f.01:052   Gaisler Research  AHB status register (ver 0x0)

irq 7

apb: 80000f00 - 80001000

grlib>

Note the Component=”Unknown device” (Vendor=”Gaisler Research”) corresponds to the unit under test.

When the opened bridge is defined, the “run” command provides:

grlib> run

In configure_spw_link *** /dev/grspw0 opened and started successfully

In configure_spw_link *** /dev/grspw0 configured successfully

In SMIQ ***

In SPIQ *** Waiting for SPW_LINKERR_EVENT

In ACTN ***

In AVFO ***

In BPFO ***

In WFRM ***

In STAT ***

In RECV ***

Serial link lost

In this unfortunate case, typical message provided:

Cannot continue, processor not in debug mode

IU in error mode (tt = 0x00)

fff90688  05000000   sethi  %hi(0x0), %g2

Leave (<CTRL>+<C>) and restart the GRMON session.

3.Scripts

Drag the *.py script in the “py>” part of the lppmon console.

Note the explicit command exists:

execfile('/opt/PYTHON/lfrverif/LFR_SVS/<path>/action.py')

Moreover, tests generate several log files:

-Raw-TC

-Raw-TM

-Synthetic sequence of TC/TM

-The chronology of TC/TM with the detail of all the fields.

(a control mechanism is implemented to tag any problem "/!\").

Note it’s possible to generate the synthesis and detailed files from raw TC/TM:

py> test_launcher.test_monitor.loadRecord("2013_05_29-11:07:09")

py> test_launcher.test_monitor.buildSynthesis("2013_05_29-11:07:09")

# Software validation specification additional information

All scripts, results, logs and backtraces of each SVS-xxx test case presented in this document is versioned on a mercurial deposit hosted under a Rhodecode server here :

https://hephaistos.lpp.polytechnique.fr/rhodecode/HG_REPOSITORIES/LPP/INSTRUMENTATION/SOLO_LFR/VALIDATION_R3

Each test case has his own directory identified by his SVS number.